## 10.               APPLICATION A:

17/0046/OUT - Outline Application: Demolition of 2 x 3 storey office buildings (Units 3 and 4 Wolsey Business Park). Construction of replacement building to include office (Class B1) and residential use (274 apartments) with associated facilities and underground parking for 661 cars (all matters reserved)

## APPLICATION B:

17/0015/OUT - Outline Application: Demolition of 2 x 3 storey office buildings (Units 3 and 4 Wolsey Business Park). Construction of replacement buildings to include office (Class B1) and residential use (332 apartments) with associated facilities and underground parking for 851 cars (all matters reserved)

## APPLICATION C:

16/2735/OUT - Outline Application: Demolition of 2 x 3 storey office buildings (Units 3 and 4 Wolsey Business Park). Construction of replacement buildings to include office (Class B1) and residential use (403 apartments) with associated facilities and underground parking for 943 cars (all matters reserved)

## APPLICATION D:

16/2709/OUT - Outline Application: Demolition of 2 x 3 storey office buildings (Units 3 and 4 Wolsey Business Park). Construction of replacement buildings to include office (Class B1) and residential use (416 apartments) with associated facilities and underground parking for 943 cars (all matters reserved)

at SYMBIO POINT UNITS 3 & 4 WOLSEY BUSINESS PARK, TOLPITS LANE, WATFORD, RICKMANSWORTH, HERTFORDSHIRE, WD18 9BL, for RSquare Properties Ltd

(DCES)

| Parish: Non-Parished | Ward: Moor Park & Eastbury |
| --- | --- |
| Expiry Statutory Period: 19.04.2017 | Officer: Claire Westwood |
| Recommendations: ## APPLICATION A: That Planning Permission be refused. ## APPLICATION B: That Planning Permission be refused. ## APPLICATION C: That Planning Permission be refused. ## APPLICATION D: That Planning Permission be refused. |  |
| Reason for consideration by the Committee: All four applications called in by 3 members of the Planning Committee. |  |

Members are advised that this report deals with 4 separate outline planning applications A-D described above; each relating to the same planning application site, namely, Symbio Point, Units 3 & 4 Wolsey Business Park.  The applications, in each case, propose the demolition of the existing 2 x 3 storey office buildings on the site comprising Units 3 & 4, and the redevelopment of the site for B1 office use and residential apartments with parking. They differ in respect of the content of each application as regards B1 office use, the number of apartments and parking spaces.  Accordingly, while there are elements common to each of the four outline planning applications, members are advised that each application must be considered and determined separately on its individual planning merits having regard to the development plan and other material considerations.

1.	Relevant Planning History

Units 3 & 4, Wolsey Business Park

1.1	8/889/89 - Erection of six buildings totalling 20,342 sq.m (218,880 sq.ft) of office and warehousing space with car parking – Permitted 23.11.89.

1.2	15/1935/FUL - Demolition of 2 x 3 storey office buildings (Units 3 and 4 Wolsey Business Park) and erection of 1 x 8 storey mixed use building (plus 3 storey basement) to include 401 residential units, leisure and communal facilities, crèche, coffee shop, retail, roof garden, internal arboretum and parking. Erection of 2 x 4 storey office (Class B1) buildings.  Associated works.  Withdrawn.

1.3	16/2497/FUL - Demolition of 2 x 3 storey office buildings (Units 3 and 4 Wolsey Business Park) and erection of 1 x 8 storey mixed use building (plus 3 storey basement) to include 401 residential units, leisure and communal facilities (including gym, swimming pool, sauna, steam room, crèche, projector room and coffee shop), retail, roof garden, internal arboretum and parking. Erection of 2 x 4 storey office (Class B1) buildings. Associated works.  Refused 24 February 2017 for the following reasons;

R1	The site is within an allocated Employment Area and the proposals would result in a net loss of Class B1 (office) floor space.  The proposed development, by reason of its excessive scale, height, design and proximity to existing buildings would be an overbearing and overdominant form of development to the detriment of the amenity of neighbouring business occupiers.  The proposal would therefore be contrary to Policy SA2 of the Site Allocations LDD (adopted November 2014) and Policy CP6 of the Core Strategy (adopted October 2011).

R2	The site is remotely located and is not within an existing sustainable location and it has not been demonstrated that the location could be made sustainable.  There is also high dependence on the private car evidenced by the overprovision of car parking spaces.  The NPPF seeks to ensure that patterns of growth make the fullest possible use of public transport and that developments that generate significant movement are located where the use of sustainable transport modes can be maximised.  The proposal is contrary to these strategic aims which seek to achieve a sustainable pattern of development and would undermine the Spatial Strategy of the Core Strategy.  The development would fail to comply with Policies CP1, CP2 and CP12 of the Core Strategy (adopted October 2011) and the NPPF.

R3	The proposed development, by reason of its design, excessive height, width, scale, bulk, building-to-plot ratio which would not allow adequate space for landscaping, would be cramped, excessively dominant in the street scene and surrounding locality and would have an adverse impact on the landscape.  It would represent overdevelopment of the site, to the detriment of the visual amenities, appearance and character of the street scene and area as a whole.  This is contrary to Policies CP1, CP3 and CP12 of the Core Strategy (adopted October 2011), Policies DM1 and DM7 and Appendix 2 of the Development Management Policies (adopted July 2013) and the NPPF.

R4	The proposed development, by reason of its excessive scale, bulk, height, design and close proximity to existing buildings, would result in an overbearing and overdominant form of development and overlooking to the detriment of the visual and residential amenities of neighbouring occupiers at The Dell, Glendale and The Oaks, contrary to Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM1 and Appendix 2 of the Development Management Policies LDD (adopted July 2013).

R5	The proposed development, by reason of its excessive scale, height and design (including layout/arrangement of flats) incorporating extensive glazing and balconies and relationship with existing development, would result in unacceptable levels of overlooking of a number of the proposed residential units and would create a sense of overbearing and enclosure that would be experienced by a number of the residential units to the detriment of the residential amenities of future residential occupiers.  The proposed development would also fail to provide sufficient amenity space to the detriment of the residential amenities of future occupiers.  This is contrary to Policies CP1 and CP12 of the Core Strategy (adopted October 2011), Policies DM1 and DM11 and Appendix 2 of the Development Management Policies LDD (adopted July 2013) and the NPPF.

R6	The proposed development fails to meet the requirements of Policy CP4 of the Core Strategy (adopted October 2011) and SPD: Affordable Housing (approved June 2011) in that the scheme would not provide policy compliant affordable housing and it has not been demonstrated that it would not be viable to meet policy requirements for affordable provision; a S106 Agreement has not been agreed to secure provision; and the proposed housing mix would fail to meet the requirements of Policy CP3 of the Core Strategy (adopted October 2011) and no evidence or justification has been provided to support the proposed housing mix.

R7	The proposed development would result in a significant increase in demand for sustainable transport provision in the area.  There is currently a shortage of these facilities in the area. The proposed development would exacerbate this situation and in the absence of a signed agreement under the provisions of Section 106 of Town and Country Planning Act 1990 fails to recognise the impact of the development upon these services.  The proposal would also attract a requirement for fire hydrant provision. The application therefore fails to meet the requirements of Policies CP1, CP8 and CP10 of the Core Strategy (adopted October 2011).

Unit 3, Wolsey Business Park

1.4	07/0928/FUL - Single storey extension to existing standby generator building and change of use to cafe with covered and outdoor decked seating areas – Permitted 19.07.07.

1.5	13/0968/FUL - Temporary change of use from Offices (Class B1) to a non-residential institution (Class D1) for use as a secondary school for 3 years, and associated minor works - Permitted 02.10.13, implemented; the Reach Free School is in 2nd academic year of occupation of building/site.

1.6	13/1391/FUL - Erection of a roof mounted 3kW Vertical Axis Wind Turbine and associated infrastructure and other works – Permitted 25.09.13, implemented.

1.7	13/1392/FUL - Installation of a 5kW Vertical Axis Wind Turbine and associated infrastructure and other works – Permitted 25.09.13, implemented.

1.8	13/1622/ADV - Advertisement Consent: Erection of non-illuminated fascia sign to front elevation of building above entrance and erection of non illuminated freestanding directional sign adjacent to building – Permitted 28.10.13, implemented.

1.9	14/1041/PDR - Prior Notification: Change of use from Office (Class B1) to 33 Residential units (Class C3).  Permitted.

1.10	15/1281/PDR - Prior Notification: Change of use from Office (Class B1) to 71 Residential units (Class C3).  Permitted.

1.11	16/0997/FUL - Variation of condition 1 (time limit) of planning permission 13/0968/FUL (Temporary change of use from Offices (Class B1) to a non-residential institution (Class D1) for use as a secondary school for 3 years, and associated minor works) to allow extension to time. Permitted 13.06.16.

1.12	16/2240/PDR - Prior Notification: Change of use from Office (Class B1) to 120 Residential units (Class C3).  Withdrawn.

1.13	16/2369/DIS - Discharge of condition 4 (bus fares) pursuant to planning permission 16/0997/FUL.  Determined.

1.14	17/0119/PDR - Prior Notification: Change of use from Office (Class B1) to 120 Residential units (Class C3).  Refused.

Unit 4, Wolsey Business Park

1.15	15/1184/PDR - Prior Notification: Change of use from Office (Class B1) to 36 Residential units (Class C3).  Permitted.

1.16	16/2241/PDR - Prior Notification: Change of use from Office (Class B1) to 60 Residential units (Class C3). Withdrawn.

1.17	17/0112/PDR - Prior Notification: Change of use from Office (Class B1) to 60 Residential units (Class C3).  Refused.

2.	Site Description

2.1		The application site is located within Wolsey Business Park which is accessed from a roundabout on Tolpits Lane, between Rickmansworth and Watford.  The site is bordered to the east by Dwight Road, to the west by neighbouring buildings, to the south by Tolpits Lane (A4145) and to the north by the Business Park’s private access road.  Dwight Road is an unclassified local access road with a 30mph speed limit and Tolpits Lane is a principal A (main distributor) road with a 40mph speed limit.

2.2	Unit 3 to the east of the site is currently in use as a school and occupied by the Reach Free School.  This use was originally granted planning permission for a temporary period of three years (ref. 13/0968/FUL), however, this has subsequently been extended for an additional year (ref. 16/0777/FUL), after which period the unit would revert to its lawful use as Class B1 (office).

2.3	Unit 3 is three-storeys in height and of red brick construction in a contemporary design with a shallow pitched roof. There is a central recessed section which includes a single storey entrance and there is a service core which results in a higher roof to the central part of the building. There are solar panels to the east roofslope and a roof mounted wind turbine to the centre of the building. There is also a freestanding turbine to the east of the building. To the west of the building (between Units 3 and 4) is an enclosed games area associated with the current school use.

2.4	Unit 4 is located to the west of the site.  The lawful use of this building is Class B1 (office), and it is understood that the building is currently partly occupied.  The building is three-storeys in height and of red brick construction in a contemporary design with a shallow pitched roof. There is a central recessed section which includes a single storey entrance and there is a service core which results in a higher roof to the central part of the building.  The existing buildings (Units 3 and 4) are of like style and design.

2.5	The front elevations of both buildings face onto car parking.  The rear elevations run parallel to Tolpits Lane but are separated from the road by a landscaped verge.

2.6	Other buildings in the vicinity of the site within the business park are two and three storeys in height and are predominantly flat roofed in their design. The immediate context of the site is commercial; however, there are three residential properties opposite the roundabout on Tolpits Lane to the south east of the site.

2.7	The site is located within a designated employment area as identified in the Three Rivers Site Allocations Local Development Document (adopted November 2014).  In October 2016 the Council confirmed an Article 4 Direction to remove permitted development rights in relation to the conversion of office space (Use Class B1a) to residential (Use Class C3) on certain employment areas, including Tolpits Lane (including the application site).  This will come into effect on the 5 August 2017.

2.8	The Ebury Way is located to the north of the site and provides a pedestrian and cycle route between Rickmansworth and Watford.  Beyond the Ebury Way to the north is Croxley Common Moor, a Local Nature Reserve and Site of Special Scientific Interest.  The site is within the Central River Valley Landscape Area.  To the south of the site on the opposite side of Tolpits Lane is the River Colne and Hampermill Lake, sited within the Metropolitan Green Belt.  Land to the south of Tolpits Lane falls within a registered archaeological site.  Moor Park Conservation Area is approximately 0.9 km to the south-west.

3.	Description of Proposed Development

APPLICATION A (17/0046/OUT):

3.1	Outline planning permission (all matters reserved) is sought for the demolition of 2 x 3 storey office buildings comprising Units 3 & 4  and the construction of a replacement building to include office space (Class B1) and residential use (274 flats) with associated facilities and underground parking for 661 cars.

3.2	The submitted details indicate a single building with an elongated ‘M’ footprint, sited with its widest elevation to the south and running roughly parallel to Tolpits Lane.  The building would comprise a mixture of straight and angled elevations.  The details indicate a maximum width (east to west) of approximately 168 metres, with a width of approximately 154 metres adjacent to Tolpits Lane.  The depth (north to south) would be a maximum of 58 metres and 10 metres at its narrowest point.  Three main wings would project north from the building’s spine, each with a depth of approximately 41 metres at ground floor and with a separation distance of approximately 43 metres between them.  It is noted that from the second floor level upwards the projecting wings would have an additional depth of approximately 7 metres as the building would overhang an access road within the Business Park.  The overhang would have a height of approximately 6.5 metres above the access road.  Between the three main wings, two smaller projecting wings are proposed each with a maximum depth of approximately 20 metres.  The submitted details indicate a five storey building with a maximum height (above ground) of approximately 17 metres.  Extensive glazing is indicated to all elevations, with solar PV panels around the perimeter of the roof.

3.3	A two storey basement car park is proposed, the application form details that this would provide 661 car parking spaces (including 35 accessible spaces), plus 44 motorcycle spaces.  The basement would be accessed via a single vehicle ramp.  Cycle storage and refuse storage is shown at surface level between the projecting wings.  There would also be two areas of soft landscaping providing children’s play areas between the main projecting wings.

3.4	The submitted details indicate that the ground floor would comprise a series of office spaces (Class B1), in addition to retail, gymnasium, coffee shop, cinema and training spaces.  It is understood that leisure facilities would be for use by residents and employees.  The upper four floors would comprise residential use with the accommodation schedule referring to 26 x studio units; 134 x 1 bed units; 106 x 2 bed units; and 8 x 3 bed units.  The Design and Access Statement refers to 11 affordable units being provided.

3.5	A number of energy efficiency/saving measures are proposed as set out within the submitted Design and Access Statement.  This also details that the roof of the building would provide an external amenity area.

3.6	While the scale and layout of the development have been shown on the submitted plans, the application is for all matters reserved and as such, appearance, means of access, landscaping, layout and scale of the proposal are indicative only.

APPLICATION B (17/0015/OUT):

3.7	Outline planning permission (all matters reserved) is sought for the demolition of 2 x 3 storey office buildings and the construction of replacement buildings to include office space (Class B1) and residential use (332 flats) with associated facilities and underground parking for 851 cars.

3.8		The submitted details indicate a main building with an elongated ‘M’ footprint, sited with its widest elevation to the south and running roughly parallel to Tolpits Lane.  The building would comprise a mixture of straight and angled elevations.  The details indicate a maximum width (east to west) of approximately 168 metres, with a width of approximately 154 metres adjacent to Tolpits Lane.  The depth (north to south) would be a maximum of 58 metres and 10 metres at its narrowest point.  Three main wings would project north from the building’s spine, each with a depth of approximately 41 metres at ground floor and with a separation distance of approximately 43 metres between them.  It is noted that from the second floor level upwards the projecting wings would have an additional depth of approximately 7 metres as the building would overhang an access road within the Business Park.  The overhang would have a height of approximately 6.2 metres above the access road.  Between the three main wings, two smaller projecting wings are proposed each with a maximum depth of approximately 20 metres.  The submitted details indicate a five storey building with a maximum height (above ground) of approximately 17 metres.    Extensive glazing is indicated to all elevations, with solar PV panels around the perimeter of the roof.

3.9	Two smaller four storey buildings are indicated and would be sited between the projecting wings of the main building, adjacent to the internal access road within the Business Park.  The buildings would both have an octagonal footprint with a maximum width of approximately 23 metres, depth of approximately 17 metres (increasing to approximately 24 metres from the second floor) and height of 13.7 metres.

3.10		A two storey basement car park is proposed, the application form details that this would provide 851 car parking spaces (including 35 accessible spaces), plus 42 motorcycle spaces.  The basement would be accessed via a single vehicle ramp.  Cycle storage and refuse storage is shown at surface level between the projecting wings.

3.11	The two smaller buildings would not include residential, they would comprise of office space (Class B1), retail, gymnasium and coffee shop.  The ground floor of the main building would include cinema and training spaces in addition to residential.  It is understood that leisure facilities would be for use by residents and employees.  The upper four floors would be residential with the accommodation schedule referring to 50 x studio units; 155 x 1 bed units; 119 x 2 bed units; and 8 x 3 bed units.  The Design and Access Statement refers to 20 affordable units being provided.

3.12	A number of energy efficiency/saving measures are proposed as set out within the submitted Design and Access Statement.  This also details that the roof of the building would provide an external amenity area.

3.13	While the scale and layout of the development have been shown on the submitted plans, the application is for all matters reserved and as such, appearance means of access, landscaping, layout and scale of the proposal are indicative only.

APPLICATION C (16/2735/OUT):

3.14	Outline planning permission (all matters reserved) is sought for the demolition of 2 x 3 storey office buildings and the construction of replacement buildings to include office space (Class B1) and residential use (403 flats) with associated facilities and underground parking for 943 cars.

3.15		The submitted details indicate a main building with an elongated ‘M’ footprint, sited with its widest elevation to the south and running roughly parallel to Tolpits Lane.  The building would comprise a mixture of straight and angled elevations.  The details indicate a maximum width (east to west) of approximately 168 metres, with a width of approximately 154 metres adjacent to Tolpits Lane.  The depth (north to south) would be a maximum of 58 metres and 10 metres at its narrowest point.  Three main wings would project north from the building’s spine, each with a depth of approximately 41 metres at ground floor and with a separation distance of approximately 43 metres between them.  It is noted that from the second floor level upwards the projecting wings would have an additional depth of approximately 7 metres as the building would overhang an access road within the Business Park.  The overhang would have a height of approximately 6.3 metres above the access road.  Between the three main wings, two smaller projecting wings are proposed each with a maximum depth of approximately 20 metres.  The submitted details indicate a six storey building with a maximum height (above ground) of approximately 19.5 metres.  Extensive glazing is indicated to all elevations, with solar PV panels around the perimeter of the roof.

3.16	Two smaller four storey buildings are indicated and would be sited between the projecting wings of the main building, adjacent to the internal access road within the Business Park.  The buildings would both have an octagonal footprint with a maximum width of approximately 23 metres, depth of approximately 17 metres (increasing to approximately 24 metres from the second floor) and height of 13.7 metres.

3.17		A two storey basement car park is proposed, the application form details that this would provide 943 car parking spaces (including 35 accessible spaces), plus 44 motorcycle spaces.  The basement would be accessed via a single vehicle ramp.  Cycle storage and refuse storage is shown at surface level between the projecting wings.

3.18	The two smaller buildings would not include residential, they would comprise of office space (Class B1), retail, gymnasium and coffee shop.  The ground floor of the main building would include cinema and training spaces in addition to residential.  It is understood that leisure facilities would be for use by residents and employees.  The upper four floors would be residential with the accommodation schedule referring to 56 x studio units; 190 x 1 bed units; 149 x 2 bed units; and 8 x 3 bed units.  The Design and Access Statement refers to 41 affordable units being provided.

3.19	A number of energy efficiency/saving measures are proposed as set out within the submitted Design and Access Statement.  This also details that the roof of the building would provide an external amenity area.

3.20	While the scale and layout of the development have been shown on the submitted plans, the application is for all matters reserved and as such, appearance means of access, landscaping, layout and scale of the proposal are indicative only.

APPLICATION D (16/2709/OUT):

3.21	Outline planning permission (all matters reserved) is sought for the demolition of 2 x 3 storey office buildings and the construction of a replacement building to include office space (Class B1) and residential use (416 flats) with associated facilities and underground parking for 943 cars.

3.22	The submitted details indicate a single building with an elongated ‘M’ footprint, sited with its widest elevation to the south and running roughly parallel to Tolpits Lane.  The building would comprise a mixture of straight and angled elevations.  The details indicate a maximum width (east to west) of approximately 168 metres, with a width of approximately 154 metres adjacent to Tolpits Lane.  The depth (north to south) would be a maximum of 58 metres and 10 metres at its narrowest point.  Three main wings would project north from the building’s spine, each with a depth of approximately 41 metres at ground floor and with a separation distance of approximately 43 metres between them.  It is noted that from the second floor level upwards the projecting wings would have an additional depth of approximately 7 metres as the building would overhang an access road within the Business Park.  The overhang would have a height of approximately 6.6 metres above the access road.  Between the three main wings, two smaller projecting wings are proposed each with a maximum depth of approximately 20 metres.  The submitted details indicate a seven storey building with a maximum height (above ground) of approximately 22.8 metres.  Extensive glazing is indicated to all elevations, with solar PV panels around the perimeter of the roof.

3.23	A two storey basement car park is proposed, the application form details that this would provide 943 car parking spaces (including 35 accessible spaces), plus 44 motorcycle spaces.  The basement would be accessed via a single vehicle ramp.  Cycle storage and refuse storage is shown at surface level between the projecting wings.  There would also be two areas of soft landscaping providing children’s play areas between the main projecting wings.

3.24	The submitted details indicate that the ground floor would comprise a series of office spaces (Class B1), in addition to retail, gymnasium, coffee shop, cinema and training spaces.  It is understood that leisure facilities would be for use by residents and employees.  The upper four floors would comprise residential use with the accommodation schedule referring to 38 x studio units; 204 x 1 bed units; 166 x 2 bed units; and 8 x 3 bed units.  The Design and Access Statement refers to 66 affordable units being provided.

3.25	A number of energy efficiency/saving measures are proposed as set out within the submitted Design and Access Statement.  This also details that the roof of the building would provide an external amenity area.

3.26	While the scale and layout of the development have been shown on the submitted plans, the application is for all matters reserved and as such, appearance means of access, landscaping, layout and scale of the proposal are indicative only.

Summary:

3.27	The table below provides an overview of the 4 applications.

|  | Application A | Application B | Application C | Application D |
| --- | --- | --- | --- | --- |
| No. of Buildings | 1 | 3 | 3 | 1 |
| No. of Residential Units | 274 | 332 | 403 | 416 |
| No. of Affordable Housing Units | 11 | 20 | 41 | 66 |
| Storeys (max) | 5 | 5 | 6 | 7 |
| Height (max) | 17m | 16.6m | 19.5m | 22.8m |
| Parking | 661 | 851 | 943 | 943 |

3.28	In addition to the submitted plans (listed at Appendix 1), all applications are accompanied by:

- Cover letter

- Application form

- CIL Additional Information form

- Design and Access Statement and Appendices

  - Environmental Report

  - Consultation Brochure

  - Social Housing Layout

  - Amenity Areas

  - Reach School Transport and Travel Plan

- External Renders

- Internal Renders

- Atrium Review

- Landscape Concept

- Contamination Report

- Energy Sustainability Report

- Primary Ecology Report

- Secondary Ecology Report

- Technical Note Ecology

- Bio Diversity Checklist

- Flood Risk Report

- Drainage Strategy Survey

- Lighting Assessment

- LVIA (Landscape Visual Impact Assessment)

- Transport Assessment

- Viability Report

- Affordable Housing Statement

- Draft Section 106 Agreement

- Copy of Pre Application Advice Note

3.29	The applicant has also submitted a number of written responses in relation to consultee and public comments received during the course of the applications.

4.	Consultation

4.1.	Statutory Consultation

A summary of responses for all applications is provided in the table below, with full comments included at Appendix 2.

|  | Application A 17/0046/OUT | Application B 17/0015/OUT | Application C 16/2735/OUT | Application D 16/2709/OUT |
| --- | --- | --- | --- | --- |
| Watford Borough Council | No comments received. | No comments received. | No comments received. | No comments received. |
| LDF Transport | No comments received. | No comments received. | No comments received. | No comments received. |
| Biodiversity Projects Officer | Advisory comments | Advisory comments | Advisory comments | Advisory comments |
| Hertfordshire Highways | No objection, conditions requested. | Objection | Objection | No objection, conditions requested. |
| Housing Development Officer | Request proposal reconsidered in view of policy requirements. | Request proposal reconsidered in view of policy requirements. | Request proposal reconsidered in view of policy requirements. | Request proposal reconsidered in view of policy requirements. |
| Housing Manager | No comments received. | No comments received. | No comments received. | No comments received. |
| Herts Archaeology | No objection | No objection | No objection | No objection |
| Herts Property Services | No comments to make | No comments to make | No comments to make | No comments to make |
| HCC Flood Risk Management Team | No objection | No objection | No objection | No objection |
| HCC Footpath | No comments received. | No comments received. | No comments received. | No comments received. |
| HCC Waste & Minerals | No comments received. | No comments received. | No comments received. | No comments received. |
| Herts Ecology | No objection | No objection | No objection | No objection |
| Herts & Middlesex Wildlife Trust | No comments received. | No comments received. | No comments received. | No comments received. |
| Fire Protection Department | Advisory comments | Advisory comments | Advisory comments | Advisory comments |
| Herts Constabulary | Advisory comments | Advisory comments | Advisory comments | Advisory comments |
| Environment Agency | No comments received. | No comments received. | No comments received. | No comments received. |
| Environmental Protection | No comments received. | No comments received. | No comments received. | No comments received. |
| Integrated Accommodation Commission | No comments received. | No comments received. | No comments received. | No comments received. |
| NHS Herts Valleys | No comments received. | No comments received. | No comments received. | No comments received. |
| Landscape Officer | Objection | Objection | Objection | Objection |
| Herts Public Health | No comments received. | No comments received. | No comments received. | No comments received. |
| Highways Agency | No objection | No objection | No objection | No objection |
| Natural England | No comments received. | No comments received. | No comments received. | No comments received. |
| Sustainable Projects Officer | No comments received. | No comments received. | No comments received. | No comments received. |
| Leisure Development Officer | Advisory comments | Advisory comments | Advisory comments | Advisory comments |
| Local Plans | Objection | Objection | Objection | Objection |
| NHS England | No comments received. | No comments received. | No comments received. | No comments received. |
| Primary Care Trust | No comments received. | No comments received. | No comments received. | No comments received. |
| Health & Community Services | No comments received. | No comments received. | No comments received. | No comments received. |
| Thames Water | No comments received. | Advisory comments | Advisory comments | Advisory comments |
| Affinity Water | Advisory comments | Advisory comments | Advisory comments | No comments received. |
| National Grid | Advisory comments | Advisory comments | Advisory comments | Advisory comments |
| Moor Park 1958 Ltd | Objection | Objection | Objection | Objection |

4.2	Public Consultation

4.2.1	Neighbour consultation:

|  | Application A 17/0046/OUT | Application B 17/0015/OUT | Application C 16/2735/OUT | Application D 16/2709/OUT |
| --- | --- | --- | --- | --- |
| No. households consulted | 431 | 431 | 431 | 431 |
| No. responses received | 11 | 10 | 12 | 9 |

4.2.2	Site Notice(s): Expired 14/02/2017 for all applications

4.2.3	Press notice(s): Expired 17/02/2017 for all applications

4.2.4	Summary of Responses

N.B. The full responses can be viewed via Planning Online.

APPLICATION A:

Not in compliance with local and national policy; Application has not been subject of EIA screening; Loss of office floorspace; Does not safeguard employment area; Not an allocated housing site; Fails to comply with housing policies; Unsustainable and inaccessible location; Significant infrastructure improvements would be required; Additional traffic and congestion; No public transport or pedestrian access; Impact on Croxley Common Moor SSSI and LNR; Light pollution; Noise disturbance to adjacent buildings; Significant overdevelopment; Out of character; Scale and design unacceptable; Excessive density; Impact on Green Belt; Overshadowing; Reiterate objections made to application 16/2497/FUL; No outdoor amenity space for occupiers; Reasons for refusal of 16/2497/FUL remain.

APPLICATION B:

Not in compliance with local and national policy; Application has not been subject of EIA screening; Loss of office floorspace; Loss of jobs; Does not safeguard employment area; Impact on existing businesses; Set precedent for loss of offices; Not an allocated housing site; Fails to comply with housing policies; Unsustainable and inaccessible location;  Additional traffic; Congestion; Significant infrastructure improvements would be required; Schools, doctors could not cope; Negative impact on local environment; Impact on Croxley Common Moor SSSI and LNR; Light pollution; Noise disturbance to adjacent buildings; Significant overdevelopment; Out of character; Scale and design unacceptable; Not in keeping with low density development; Impact on skyline; Impact on Green Belt; Overshadowing; Reiterate objections made to application 16/2497/FUL; Reasons for refusal of 16/2497/FUL remain.

APPLICATION C:

Not in compliance with local and national policy; Application has not been subject of EIA screening; Loss of office floorspace; Does not safeguard employment area; Loss of jobs; Negative impact for existing businesses; Not an allocated housing site; Fails to comply with housing policies; Lack of affordable housing; Unsustainable and inaccessible location; Significant infrastructure improvements would be required; Schools, doctors could not cope; No public transport links; No pedestrian access; Additional traffic and congestion; Rail bridge restricts road access on Tolpits Lane; Impact on Croxley Common Moor SSSI and LNR; Negative impact on local environment; Light pollution; Noise disturbance to adjacent buildings; Significant overdevelopment; Out of character; Excessive density; Urban sprawl; Impact on street scene; Scale and design not appropriate for area; Impact on skyline; Poor level of amenity for future occupiers; Fall-back position does not hold material weight. No suitable outdoor space; Reasons for refusal of 16/2497/FUL remain.

Would support the application if it meant improvements to public transport to the industrial estates on Tolpits Lane; The inclusion of retail units as well as ample parking are other reasons to support the application.

APPLICATION D:

Not in compliance with local and national policy; Application has not been subject of EIA screening; Loss of office floorspace; Does not safeguard employment area; Set precedent for loss of offices; No evidence that offices not required; Not an allocated housing site; Fails to comply with housing policies; Lack of affordable housing; Unsustainable and inaccessible location; Significant infrastructure improvements would be required; Additional traffic; No public transport or pedestrian access; Heavy reliance on cars; Impact on Croxley Common Moor SSSI and LNR; Area known for natural beauty; Light pollution; Noise disturbance to adjacent buildings; Significant overdevelopment; Out of character; Scale and design not appropriate for area; Excessive density; Impact on street scene; Impact on skyline; Overshadowing; Poor level of amenity for future occupiers; Fall-back position does not hold material weight; Reasons for refusal of 16/2497/FUL remain.

5.	Reason for Delay

5.1	No delay.

6.	Planning Policy, Guidance and Legislation relevant to all 4 applications.

6.1		The Three Rivers Local Plan

The Core Strategy was adopted on the 17 October 2011 having been through a full public participation process and Examination in Public. Relevant policies include Policies CP1, CP2, CP3, CP4, CP6, CP7, CP8, CP9, CP10 and CP12.

The Development Management Policies Local Development Document (LDD) was adopted on 26 July 2013 after the Inspector concluded that it was sound following Examination in Public which took place in March 2013. Relevant policies include DM1, DM2, DM3, DM4, DM5, DM6, DM7, DM8, DM9, DM10, DM11, DM12, DM13 and Appendices 2 and 5.

The Site Allocations Local Development Document (SALDD) was adopted on 25 November 2014 having been through a full public participation process and Examination in Public. Policy SA2 is relevant.

The Community Infrastructure Levy (CIL) Charging Schedule (adopted February 2015).

6.2	National Planning Policy Framework (NPPF)

On 27 March 2012, the framework of government guidance in the form of Planning Policy Statements and Planning Policy Guidance Notes was replaced by the National Planning Policy Framework (NPPF). The application has been considered against the policies of the Core Strategy (adopted October 2011), the Development Management Policies Local Development Document (adopted July 2013) and the Site Allocations Local Development Document (adopted November 2014) as well as government guidance. The policies of Three Rivers District Council reflect the content of the NPPF.

6.3	Other planning policies

Supplementary Planning Document 'Affordable Housing' (approved June 2011 following a full public consultation).

The Hertfordshire County Council Waste Core Strategy and Development Management Policies Development Plan Document (2012).

Hertfordshire Minerals Local Plan Review 2002-2016 (adopted March 2007).

The Localism Act received Royal Assent on 15 November 2011. The growth and Infrastructure Act achieved Royal Assent on 25 April 2013.

The Wildlife and Countryside Act 1981 (as amended), the Conservation of Habitats and Species Regulations 2010, the Natural Environment and Rural Communities Act 2006 and the Habitat Regulations 1994 may also be relevant.

- Flood and Water Management Act 2010.

Open Space, Amenity and Children's Playspace Supplementary Planning Document - Adopted December 2007.

7.	Planning Analysis

7.1	Outline Nature of Applications

7.1.1	The applications have been submitted in outline and consequently, only those matters which are not reserved may be given significant weight.  The applications are for all matters reserved, and therefore if planning permission were  to be  granted (for any of the applications),  the  reserved matters  (access,  appearance,  landscaping,  layout  and  scale) would  all need to be the subject of another application or applications.

7.1.2	An outline application must include information about the proposed use and amount of each use, and must also indicate the area or areas where access points to the development will be situated, even if access has been reserved.  Where layout is a reserved matter, the application shall state the approximate location of buildings, routes and open spaces.  Where scale is a reserved matter, the application shall state the upper and lower limit for the height, width and length of each building included in the development.

7.1.3	The four applications each seek approval for the demolition of two existing office buildings and the erection of between 1 – 3 new buildings to provide residential led development with office (Class B1) space and leisure facilities.  As described in section 3 above the number of residential units and number of parking spaces varies between the 4 applications.  Similarly, the number of buildings and building storeys also varies.

7.1.4	The assessment of the outline applications must have regard to the design, massing, height and siting of the development proposed in each application and shown on the application plans, together with landscaping and parking to the extent that these details are shown.

7.1.5	All 4 applications are each accompanied by indicative plans and elevations showing how the application site could potentially be developed.

7.2	Policy/Principle of Development relevant to the 4 applications

7.2.1	Paragraph 19 of the NPPF advises that the Government is committed to ensuring that the planning system supports economic growth.

7.2.2	Paragraph 22 of the NPPF advises that;

“Planning policies should avoid the long term protection of sites allocated for employment use where there is no reasonable prospect of the site being used for that purpose…”

7.2.3	The application site forms part of the Tolpits Lane Employment Site Allocation allocated for employment use by the Site Allocations Local Development Document which was adopted in November 2014.  Policy SA2 of the Site Allocations LDD (adopted November 2014) states that;

“Allocated employment sites will be safeguarded for business, industrial and storage or distribution uses.

Sites allocated as having potential for mixed use development may provide for mixed use development including, but not limited to business, industrial and storage or distribution; residential or community uses”.

7.2.4	Policy SA4 of the Site Allocations LDD (adopted November 2014) relates to ‘Retail Allocations’.  The application site is not within one of the identified shopping centres.  Policy SA4 advises that;

“Applications for new retail development outside the identified centres will only be considered if the applicant has established that there is a need for the development and that there is no suitable and viable site likely to become available within or on the edge of the existing centres that could satisfactorily accommodate the development”.

7.2.5	Policy CP6 (Employment and Economic Development) of the Core Strategy (adopted October 2011) seeks to support the sustainable growth of the Three Rivers District economy by (for example);

“(j) Continuing to focus employment use within the key employment areas within the District”.

7.2.6	Tolpits Lane is listed within Policy CP6 as one of the key employment areas that the policy seeks to safeguard.

7.2.7	In addition to the above, and in recognition of the need to protect employment sites, the Council made a non-immediate Article 4 Direction on 5 August 2016 which removes permitted development rights for changes of use from office or light industrial to residential use within the Tolpits Lane employment area.  The direction was confirmed on 13 October 2016 and will come into effect on 5 August 2017 (in the case of office to residential) and the 30 September 2017 (in the case of light industrial to residential).  The Council considers the Article 4(1) Direction to be necessary to protect this important location for employment within the District and to secure the viability of businesses within the employment area.

7.2.8	Policy CP6 of the Core Strategy (adopted October 2011) sets out that the sustainable growth of the Three Rivers economy will be supported by;

“(n) Releasing office space from employment use where this is expected to be surplus to employment needs across the plan period as indicated by an up to date Employment Land Study”.

7.2.9	The South West Herts Economic Study (published in February 2016) indicates that there will be an estimated total jobs growth in the District of 8,400 during the period 2013-2036.  Of these, 43% (or 3,600 jobs) are expected to be required within ‘B’ Classes including office (Class B1).  This equates to approximately 7.1 hectares of additional land being required for office space or approximately 60,000 square metres of office space during this period.

7.2.10	The existing and proposed B1 use floor areas for the 4 individual applications are set out in the table below.  In all cases the existing figure for Units 3 and 4 (5,838m²) is taken from the submitted CIL Additional Information Forms.  The proposed figures are taken from the accommodation schedules submitted with each application.

|  | Existing B1 space (m²) | Proposed B1 space (m²) | Difference (m²) |
| --- | --- | --- | --- |
| Application A | 5,838m² | 2,231.24m² | 3,606.76m² |
| Application B | 5,838m² | 2,345.38m² | 3,492.62m² |
| Application C | 5,838m² | 2,345.38m² | 3,492.62m² |
| Application D | 5,838m² | 2,231.24m² | 3,606.76m² |

7.2.11	The table above demonstrates that in the case of all 4 applications there would be a reduction in Class B1 (office) floor space of approximately 3,606.76m² - 3,492.62m² and it is noted that the residential element forms the main component in each of the proposed schemes.

7.2.12	Whilst the content of letters from Braser Freeth Chartered Surveyors and Knight Frank LLP (included within each of the 4 submitted Design and Access Statements) detailing the letting history of the site and submitted in support of the applications are noted, these are not considered sufficient to demonstrate that the existing B1 office space is surplus to employment needs across the plan period.  As noted above, these contradict the findings of the South West Herts Economic Study (published in February 2016) which identifies a need for additional office space in the District.

7.2.13	Whilst the letters state that the site is not attractive to the market, it is noted that the South West Herts Economic Study (published in February 2016) states that Tolpits Lane is;

“Overall good quality and intensively developed business park catering for a range of employment activities and sectors. There is little evident development potential. The Council should seek to support employment uses at the site”.

7.2.14	Contrary to the requirements of the NPPF it is concluded that it has not been demonstrated by any substantial evidence produced in support of any of the 4 applications that there is no reasonable prospect of the site being used for employment use.

7.2.15	Accordingly, it is concluded that, in each case, the proposals would result in a net loss of Class B1 (office) floor space contrary to Policy SA2 of the Site Allocations LDD (adopted November 2014) and Policy CP6 of the Core Strategy (adopted October 2011).

7.2.16	Paragraph 49 of the NPPF advises that;

“Housing applications should be considered in the context of the presumption in favour of sustainable development.  Relevant policies for the supply of housing should not be considered up-to-date if the local planning authority cannot demonstrate a five-year supply of deliverable housing sites”.

7.2.17	Since the application site is not identified or allocated as part of the District’s housing supply were it to be developed for housing it would constitute a windfall site.  Policy CP2 (Housing Supply) of the Core Strategy (adopted October 2011) states that applications for windfall sites will be considered on a case by case basis having regard to;

- the location of the proposed development, taking into account the Spatial Strategy;

- the sustainability of the development and its contribution to meeting local housing needs;

- infrastructure requirements and the impact on the delivery of allocated housing sites;

- monitoring information relating to housing supply and the Three Rivers housing target.

7.2.18	It is noted that the Housing Land Supply Update and Annual Monitoring Report published in December 2016 indicate 9.8 years supply of housing against the target set out within the Core Strategy.

7.2.19	The Core Strategy was adopted in October 2011 and sets out a housing target of 180 dwellings per year 2001-2026 and it is against this target that there is a 9.8 year supply of housing.

7.2.20	It is noted that in January 2016, the South West Hertfordshire Strategic Housing Market Assessment (SHMA) was published. The SHMA considers housing need over the period 2013-2036 and indicates a need for 514 dwellings per year to be provided in the Three Rivers area. Having regard to NPPF section 6, the updated housing need information is a material planning consideration which must be taken into account in the assessment of each of the four proposed developments and may be given weight in the determination of applications. However, the SHMA does not set the housing target for the District or provide a substitute target for the existing housing target within the Core Strategy. It may nevertheless be an important factor in the setting of future housing targets that would be taken into account in informing the review of the Local Plan.

7.2.21	The Spatial Strategy of the Core Strategy states that new development will be directed towards previously developed land and appropriate infilling opportunities within the urban areas of the Principal Town (Rickmansworth) and Key Centres (South Oxhey, Croxley Green, Abbots Langley, Chorleywood, Leavesden and Garston and Mill End) which have been identified as the most sustainable locations within the District.  More limited levels of new development will be directed towards Secondary Centres (Kings Langley, Carpenders Park, Eastbury, Oxhey Hall, Maple Cross and Moor Park).

7.2.22	Place Shaping Policies PSP1, PSP2 and PSP3 of the Core Strategy (adopted October 2011) set out the proportion of development that will be directed towards the Principal Town (15%), Key Centres (60%) and Secondary Centres (24%).

7.2.23	The application site is not within or close to the boundaries of the Key Town, Key Centres or Secondary Centres identified by the Core Strategy.  Tolpits Lane is a busy road between the settlements of Rickmansworth and Watford and the application site is within an ‘out of town’ employment area in a relatively isolated location.  The nearest station is Croxley Green which is approximately 1km from the site when taking a direct line; however, accessing the station would involve a longer walk (approximately 3km), including along Tolpits Lane which has no footpaths or street lighting for part of the way.  There is an alternative route (approximately 1.1km) across the Business Park and Croxley Common Moor (SSSI and Local Nature Reserve).  Whilst there is a right of way across Croxley Common Moor, there is no formal footpath or lighting provided in this sensitive location.

7.2.24	Moor Park Station is approximately 1.8km from the site (direct line).  The submitted details indicate access via Tolpits Lane, which as noted above has limited lighting and footways.   There is alternative access via Sandy Lodge Road (approximately 2.9km), however, it is noted that this is a private barrier controlled road within the Moor Park Estate.

7.2.25	The proposed Vicarage Road Station (due to be functional in 2020) would be approximately 1.4km from the site off Hagden Lane.  The proposed Cassiobridge Station (due to be functional in 2020) would be approximately 1.5km by foot/bicycle.  The most direct route to both proposed stations would include Tolpits Lane.  Whilst these stations are due to be operational in 2020, there remains some uncertainty regarding their delivery at this time.

7.2.26	It is acknowledged at paragraph 3.14 of the Core Strategy that major development outside of the Principal Town and Key Centres is not necessarily precluded, but its planning merits need to be carefully assessed because of lower accessibility.  It continues that development may be appropriate in these areas, particularly where it offers opportunities to rectify specific deficiencies in vital services and facilities or where it provides development necessary to sustain centres as providers of local employment, shopping and other services.  None of the 4 proposed developments would meet any such identified need and as set out above, the application site is not serviced by public bus routes, Tolpits Lane lacks footpaths and there is no specific provision for cyclists on the highway.  The application site is isolated from key services and does not have easy access to existing facilities.

7.2.27	The Design and Access Statements and Transport Statements submitted in support of each of the applications make reference to a subsidised bus route and state that the applicant and Arriva are ‘keen to work together’ and have sought views from HCC regarding their preferred route.  The Design and Access Statements refer to two routes, route 1 connecting the site with Rickmansworth Town and route 2 connecting the site to Watford.  The details reflect those submitted with application 16/2497/FUL.  At the time of application 16/2497/FUL, HCC (Sustainable Transport and Development Officer) confirmed that their preference would be for route 2 (Watford) and that a S106 Agreement between the applicant and Arrivia could ensure a certain level of service for a certain length of time.  While improvements to public transport would be supported, no S106 Agreement has been agreed at this time to secure the necessary provisions.  Similarly, it is likely that the service would be subsidised for a temporary period and the Local Planning Authority would therefore have no mechanism to secure the long term provision of a bus route.  In any case, it is not considered that the provision of a bus route in this location would in itself overcome the above identified concerns regarding the relatively inaccessible location of the site for residential development (highways and accessibility are discussed in more detail below).

7.2.28	It is acknowledged that each application proposes community and retail facilities to serve the residential development.  However such provision would not overcome the concerns regarding the inappropriateness of the location of the site for residential development as regards its relative inaccessibility in that none of the applications would deliver the range of such services that could reasonably be expected to be provided.

7.2.29	Paragraph 17 of the NPPF sets out the Core Planning Principles which include;

“Actively manage patterns of growth to make the fullest possible use of public transport, walking and cycling, and focus significant development in locations which are or can be made sustainable”.

7.2.30	The application site is not located within an existing sustainable location and it has not been demonstrated in materials supporting any of the applications that the location could be made sustainable for the housing development proposed.  Each application, in so far as it includes housing development, is accordingly considered to be contrary to the principles set out in the Spatial Strategy and contrary to the objectives set out in the Place Shaping Policies (PSP1, PSP2 and PSP3), and Policy CP1 (Overarching Policy on Sustainable Development) of the Core Strategy (adopted October 2011) and the NPPF.  Furthermore, it is within an allocated employment site safeguarded for employment uses.

7.2.31	As such, development of the site as proposed in each application would undermine the Spatial Strategy of the adopted Core Strategy and would be contrary to the requirement as set out at Core Strategy Policy CP6 and Policy SA2 of the Site Allocations document to ensure sufficient employment space is available to maintain and support the economic growth of the District. This policy conflict substantially outweighs any benefits of providing additional housing on the site, as proposed in each of the applications, which is not required to meet current targets in the adopted Core Strategy.

7.2.32	In summary, the site is an allocated employment site and the proposed developments would result in a net loss of Class B1 (office) floor space, contrary to Policy SA2 of the Site Allocations LDD (adopted November 2014) and would be an unsustainable location for housing contrary to Policies CP1 and CP6 of the Core Strategy (adopted October 2011). The scale and location of the developments in each case would undermine the Spatial Strategy for the district.  The 4 proposed developments would each therefore also be contrary to Policy CP2 of the Core Strategy (adopted October 2011).

7.3	Permitted Development Rights

7.3.1	The following analysis is applicable to each one of the applications.

7.3.2	The planning history for Units 3 and 4 Wolsey Business Park is set out at section 1 above and includes prior approval applications determined in relation to Class O (Development consisting of a change of use of a building and any land within its curtilage to a use falling within Class C3 (dwellinghouses) of the Schedule to the Use Classes Order from a use falling within Class B1(a) (offices) of that Schedule), of Schedule 2, Part 3 of the Town and Country (General Permitted Development) Order 2015.  These applications were permitted and relate to up to 71 flats at Unit 3 and 36 flats at Unit 4.

7.3.3	Whilst not implemented to date, the applicant has indicated that the prior approval applications would be implemented in the absence of planning permission being granted and therefore contends that in respect of each application the existence of permitted development rights for housing attaching to the applications site to be a material consideration of significant weight in the determination of the applications..

7.3.4	Whilst the applicant’s position is noted, the Local Planning Authority does not consider the existence of these Prior Approvals is a material consideration of sufficient weight to overcome the conflict with adopted planning policy caused by each of the 4 outline applications.

7.4	Affordable Housing / Housing Mix

Housing Mix:

7.4.1	Policy CP3 (Housing Mix and Density) of the Core Strategy (adopted October 2011) states that the Council will require housing proposals to take into account the range of housing needs, in terms of size and type of dwellings as identified by the Strategic Housing Market Assessment and subsequent updates, although the supporting text acknowledges that the identified proposed housing may need to be adjusted for specific schemes to take account of market information, housing needs and preferences and specific site factors.  The most recent SHMA was published in February 2016 and has identified the indicative targets for market sector dwelling sizes within Three Rivers District as: 1 bedroom 7.7%; 2 bedrooms 27.8%; 3 bedroom 41.5%; and 4+ bedrooms 23%.

7.4.4	The details submitted with the 4 applications indicate the following breakdown of unit sizes:

|  | Studio / 1 bed | 2 bed | 3 bed | Total |
| --- | --- | --- | --- | --- |
| Application A | 160 (58%) | 106 (39%) | 8 (3%) | 274 |
| Application B | 205 (61%) | 119 (36%) | 8 (3%) | 332 |
| Application C | 246 (61%) | 149 (37%) | 8 (2%) | 403 |
| Application D | 242 (58%) | 166 (40%) | 8 (2%) | 416 |

7.4.5	In the case of each application, the proposed housing mix would fail to meet the requirements of Policy CP3 of the Core Strategy (adopted October 2011) and no satisfactory evidence or justification has been provided to support any of the proposed housing mixes.

Affordable Housing:

7.4.6	Paragraph 50 of the NPPF requires Local Planning Authorities to set policies to meet identified affordable housing need. The NPPF does not prescribe minimum levels of affordable housing but does require authorities to take account of viability when setting local planning policy (Paragraph 173). The background research undertaken for the Core Strategy found that the requirement for affordable housing in and around Three Rivers is exceptionally high and that in order to fully satisfy the need for affordable housing all housing delivered between 2011 and 2021 would need to be affordable (either social rented or intermediate).  However, it acknowledged that delivery of affordable housing at this level would not be economically viable and therefore to understand what level of affordable housing provision would be viable the Council undertook a Development Economics Study which concluded that on average the provision of 45% affordable housing from future housing development would be economically viable.

7.4.7	As such, Policy CP4 (Affordable Housing) of the Core Strategy (adopted 2011) and supported by the Affordable Housing SPD, requires 45% of all new housing to be provided as affordable housing, unless it can be clearly demonstrated with financial evidence that this is not viable.  As a guide the tenure split should be 70% social rented and 30% intermediate.

7.4.6	The details submitted with the applications indicate the following affordable housing provision:

|  | Total No. Units | No. Affordable Units | Percentage of Affordable Units |
| --- | --- | --- | --- |
| Application A | 274 | 11 | 4% |
| Application B | 332 | 20 | 6% |
| Application C | 403 | 41 | 10% |
| Application D | 416 | 66 | 16% |

7.4.7	The applications have each been accompanied by a viability appraisal seeking to justify the affordable housing provision in each case.

7.4.8	The viability reports submitted with each of the 4 applications have been reviewed and the conclusions drawn in respect of these reports by the Council are set out below:

7.4.9	APPLICATION A:

The appraisal we have carried out is on a policy compliant scheme with 45% housing. This has been carried out to establish the residual land value with the policy compliant amount of affordable housing so that it can be compared to a sensible benchmark land value.

The appraisal carried out (appendix 1) shows a Residual Land Value of £12,139,500.

This demonstrates that a policy compliant scheme with 45% affordable housing is viable and generates a land value in excess of our “benchmark” of £9,775,000.

This land value is at a level that means that all of the policies of the Council can be fulfilled.

There are a number of variables that will affect the viability of the scheme and result in a different residual land value:

1. Much more detail is required regarding the build cost

2. The actual return from a registered provider could be lower or higher than we have assumed which again would decrease / increase the RLV

3. The property market is also recovering and sales values are increasing. Whilst this cannot be taken into account (as values can also decrease) this may have a positive effect on the viability of the scheme.

4. We have not made an allowance for the car parking spaces (income or expenditure)

5. The inputs for the non-residential element into the HCA Toolkit seem incorrect based on the submitted scheme and again more detail is required.

It is our opinion that the scheme can provide the 45% affordable housing that the Council require.

7.4.10	APPLICATION B:

The appraisal we have carried out is on a policy compliant scheme with 45% housing. This has been carried out to establish the residual land value with the policy compliant amount of affordable housing so that it can be compared to a sensible benchmark land value.

The appraisal carried out (appendix 1) shows a Residual Land Value of £13,642,500.

This demonstrates that a policy compliant scheme with 45% affordable housing is viable and generates a land value well in excess of the “benchmark” value suggested by Bespoke Property Services and our own “benchmark” of £9,775,000.

This land value is at a level that means that all of the policies of the Council can be fulfilled.

There are a number of variables that will affect the viability of the scheme and

result in a different residual land value:

1. Much more detail is required regarding the build cost

2. The actual return from a registered provider could be lower or higher than we have assumed which again would decrease / increase the RLV

3. The property market is also recovering and sales values are increasing. Whilst this cannot be taken into account (as values can also decrease) this may have a positive effect on the viability of the scheme.

4. We have not made an allowance for the car parking spaces (income or expenditure)

5. The inputs for the non-residential element into the HCA Toolkit seem incorrect based on the submitted scheme and again more detail is required.

It is our opinion that the scheme can provide the 45% affordable housing that the Council require.

7.4.11	APPLICATION C:

The appraisal we have carried out is on a policy compliant scheme with 45% housing. This has been carried out to establish the residual land value with the policy compliant amount of affordable housing so that it can be compared to a sensible benchmark land value.

The appraisal carried out (appendix 1) shows a Residual Land Value of £17,107,000.

This demonstrates that a policy compliant scheme with 45% affordable housing s viable and generates a land value well in excess of the “benchmark” value suggested by Bespoke Property Services and our own “benchmark” of £9,775,000.

This land value is at a level that means that all of the policies of the Council can

be fulfilled.

There are a number of variables that will affect the viability of the scheme and result in a different residual land value:

1. Much more detail is required regarding the build cost

2. The actual return from a registered provider could be lower or higher than we have assumed which again would decrease / increase the RLV

3. The property market is also recovering and sales values are increasing. Whilst this cannot be taken into account (as values can also decrease) this may have a positive effect on the viability of the scheme.

4. We have not made an allowance for the car parking spaces (income or expenditure)

5. The inputs for the non-residential element into the HCA Toolkit seem incorrect based on the submitted scheme and again more detail is required.

It is our opinion that the scheme can provide the 45% affordable housing that the Council require.

7.4.12	APPLICATION D:

The appraisal we have carried out is on a policy compliant scheme with 45% housing. This has been carried out to establish the residual land value with the

policy compliant amount of affordable housing so that it can be compared to a

sensible benchmark land value.

The appraisal carried out (appendix 1) shows a Residual Land Value of £18,970,000.

This demonstrates that a policy compliant scheme with 45% affordable housing is viable and generates a land value well in excess of the “benchmark” value suggested by Bespoke Property Services and our own “benchmark” of £9,775,000.

This land value is at a level that means that all of the policies of the Council can

be fulfilled.

There are a number of variables that will affect the viability of the scheme and

result in a different residual land value:

1. Much more detail is required regarding the build cost

2. The actual return from a registered provider could be lower or higher than we have assumed which again would decrease / increase the RLV

3. The property market is also recovering and sales values are increasing.

Whilst this cannot be taken into account (as values can also decrease) this may have a positive effect on the viability of the scheme.

4. We have not made an allowance for the car parking spaces (income or expenditure)

5. The inputs for the non-residential element into the HCA Toolkit seem incorrect based on the submitted scheme and again more detail is required.

It is our opinion that the scheme can provide the 45% affordable housing that the Council require.

7.4.13	Agreement regarding the level of affordable housing provision has not been reached at this time and none of the applications have demonstrated that it would not be viable to meet the Council’s affordable housing  policy requirement.

7.4.14	Accordingly each of the proposed mixed developments in the 4 applications conflicts with the requirements of Policy CP4 of the Core Strategy (adopted October 2011) and SPD: Affordable Housing (approved June 2011).

7.5	Scale, Design, Street Scene, Character & Appearance

7.5.1	The NPPF (paragraph 56) advises that;

“The Government attaches great importance to the design of the built environment.  Good design is a key aspect of sustainable development, is indivisible from good planning, and should contribute positively to making places better for people”.

7.5.2	The NPPF continues at paragraph 60;

“Planning policies and decisions should not attempt to impose architectural styles or particular tastes and they should not stifle innovation, originality or initiative through unsubstantiated requirements to conform to certain development forms or styles.  It is, however, proper to seek to promote or reinforce local distinctiveness”.

7.5.3	Paragraph 64 of the NPPF states that:

“Permission should be refused for development of poor design that fails to take the opportunities available for improving the character and quality of an area and the way it functions”.

7.5.4	Policy CP3 of the Core Strategy (adopted October 2011) advises that;

“The Council will promote high quality residential development that respects the character of the District and caters for a range of housing needs.  Development will make the most efficient use of land, without compromising the quality of the environment and existing residential areas”.

7.5.4	Policy CP12 (Design of Development) of the Core Strategy (adopted October 2011) advises that in seeking a high standard of design, the Council will expect all development proposals to:

a) Have regard to the local context and conserve or enhance the character, amenities and quality of an area

d) Make efficient use of land whilst respecting the distinctiveness of the surrounding area in terms of density, character, layout and spacing, amenity, scale, height, massing and use of materials

k) Use high standards of building materials, finishes and landscaping…

7.5.5	Policy DM1 (Residential Design and Layout) of the Development Management Policies LDD (adopted July 2013) requires all applications for residential development to satisfy the design criteria set out in Appendix 2 of the Development Management Policies LDD (adopted July 2013) to ensure that development does not lead to a gradual deterioration in the quality of the built environment, and that landscaping, the need for privacy and amenity space and the creation of identity in housing layouts are taken into account.

7.5.6	Appendix 2 (Design Criteria) of the Development Management Policies LDD (adopted July 2013) advises that new development should take into consideration impacts on neighbouring properties, both within and surrounding the development, and visual impact generally.  Oversized, unattractive and poorly sited development can result in loss of light and outlook for neighbours and detract from the character and appearance of the street scene.

7.5.7	Policy DM7 (Landscape Character) of the Development Management Policies LDD (adopted July 2013) advises that in all landscape regions, the Council will require proposals to make a positive contribution to the surrounding landscape.

7.5.8	For the purposes of considering the scale, design and bulk of the each of the 4 proposed developments and their likely individual impact on the character and appearance of the street scene and general area, a summary of the content of each proposal is provided in the table below and is based on the details submitted with each application, though it is acknowledged that scale, design and layout are reserved matters.  The table also includes details of refused application 16/2497/FUL for comparison purposes:

|  | Application A | Application B | Application C | Application D | 16/2497/FUL |
| --- | --- | --- | --- | --- | --- |
| No. of Buildings | 1 | 3 | 3 | 1 | 3 |
| No. of Units | 274 | 332 | 403 | 416 | 401 |
| Storeys (max above ground) | 5 | 5 | 6 | 7 | 8 |
| Height (max) | 17m | 17m | 19.5m | 22.8m | 30m |
| Width (max) | 168m | 168m | 168m | 168m | 180m |
| Depth (max) | 58m | 58m | 58m | 58m | 63m |
| Density | 201 dph | 244 dph | 296 dph | 306 dph | 295 dph |

*dph = dwellings per hectare

APPLICATION A

7.5.9	Application A for ‘construction of replacement building to include office (Class B1) and residential use (274 apartments) with associated facilities and underground parking for 661 cars (all matters reserved)’, the smallest of the proposals, is considered first.

7.5.10	The application form details the site area as 1.36 hectares which would indicate a residential density of approximately 201 dwellings per hectare.  Even on a sustainable site in a town centre location, this would be considered a high residential density and this does not take into account the non-residential aspects of the proposal.  Additionally, and as previously noted, whilst the development would contribute to the supply of housing within the district, Three Rivers District Council currently has an identified 9.8 year supply of housing sites that meet the definition of deliverability set out in the NPPF and as such the proposed residential development is not required to meet the District’s housing targets.

7.5.10	As existing, Unit 3 has a maximum width of approximately 77 metres, depth of approximately 25 metres and height of approximately 13.9 metres.  Unit 4 has a maximum width of approximately 45 metres, depth of approximately 18 metres and height of approximately 13.9 metres.  The maximum widths and depths of both Units 3 and 4 do not extend across the full extent of the buildings due to existing stepped footprints.  There is spacing of approximately 22 metres between Units 3 and 4 currently.  The existing buildings are 3 storeys, although it is acknowledged that due to their original commercial use, there are higher floor to ceiling levels than would be expected on a residential building.  Other buildings in the Business Park are generally a mix of 2 and 3 storeys.

7.5.11	The proposed development (APPLICATION A) would be up to 5 storeys in height above ground with a maximum height of approximately 17 metres and occupying a significantly greater footprint than the existing buildings due to it infilling the existing spacing between Units 3 and 4 and its design incorporating wings projecting into the existing car park area and overhanging the access road within the Business Park.  It is acknowledged that there would be some space between the main projecting wings, which the submitted details indicate would provide external play areas, however, these are not of significant size and due to the design of the building they would be largely screened from public view points (with the exception of views from the north) and would not make a significant contribution in terms of creating a sense of space around the building.

7.5.12	The submitted details indicate that the building would have a maximum width (east to west) of approximately 168 metres and maximum depth (north to south) of approximately 58 metres.  Whilst it is acknowledged that indicative details submitted illustrate that the building would be broken up to some degree on its northern side as a result of its design incorporating angled elements and projecting wings, the spacing would not be significant and would be viewed against the backdrop of the main east-west spine behind.  The southern elevation of the main building would run adjacent to Tolpits Lane and with a limited set back from the road, with a total width of approximately 168 metres.  This 5 storey unbroken elevation would be significantly and overbearingly prominent within the street scene.

7.5.13	It is considered that the scale, mass and bulk of the proposed development (APPLICATION A) illustrated within the submitted details, would be out of keeping with both the immediate surroundings and the wider area and that the building would be unduly dominant when approaching the site from both directions along Tolpits Lane.  The excessive scale of the proposed development is demonstrated in the existing and proposed street scenes submitted with the application.

7.5.14	Whilst the objective to make efficient use of land is noted, this must be balanced against other considerations such as the requirement for development to have regard to local context.  The proposed layout is considered cramped and indicative that the proposals represent an overdevelopment of the site.  This is evidenced further by the failure of this application to comply with other policies, for example, failure to provide sufficient amenity space (discussed later in analysis) and layouts that would result in overshadowing and overlooking between units.

7.5.15	As noted above, it is acknowledged that the NPPF (paragraph 60) advises that planning policies and decisions should not impose architectural styles or particular tastes; however it is proper to seek to promote local distinctiveness. The illustrative design is contemporary, which within this area may not appear out of keeping, however, there are significant concerns regarding the overall scale.

7.5.16	The proposed development, by reason of its excessive height, width, scale, bulk and building-to-plot ratio, would be cramped and excessively dominant in the street scene and surrounding locality and would represent overdevelopment of the site, to the detriment of the visual amenities and character of the area as a whole.  This is contrary to Policies CP1, CP3 and CP12 of the Core Strategy (adopted October 2011) and Policy DM1 and Appendix 2 of the Development Management Policies (adopted July 2013).

APPLICATIONS B, C & D

7.5.17	When comparing APPLICATION A with APPLICATIONS B, C & D, the submitted details indicate a proposal of comparable height, width and depth (APPLICATION B) and proposals of comparable width and depth but greater height (APPLICATIONS C & D).  It is also noted that APPLICATIONS B and C include additional buildings between the main projecting wings which would further reduce spacing around the building.

7.5.18	The above analysis in paragraphs 7.5.10 – 7.5.16 is therefore considered equally applicable to APPLICATIONS B, C and D.  The proposed development in each of these applications, by reason of its excessive height, width, scale, bulk and building-to-plot ratio, would be cramped and excessively dominant and overbearing in the street scene and surrounding locality and would result, in each case, in overdevelopment of the application site, to the detriment of the visual amenities and character of the area as a whole.  This is contrary to Policies CP1, CP3 and CP12 of the Core Strategy (adopted October 2011) and Policy DM1 and Appendix 2 of the Development Management Policies (adopted July 2013).

Landscape & Visual Impact Assessment:

7.5.19	The 4 applications have each been accompanied by Landscape and Visual Impact Assessments (LVIA) prepared by Lockhart Garratt.  The submitted documents have been amended to reflect the proposals of each application.

7.5.20	Landscape and visual impact assessments were submitted with applications 15/1935/FUL and 16/2497/FUL, Land Use Consultants (LUC) were commissioned by TRDC to provide a review of the two Landscape and Visual Impact Assessments (LVIAs) submitted with 15/1935/FUL. Their report provided a technical review of the LVIAs, considering the scope, methodology, baseline, assessment and mitigation, with reference to the Guidelines for Landscape and Visual Impact Assessment, 3rd edition (GLVIA3).  In addition, it considered the accuracy and compliance of the included visualisations in relation to the Landscape Institute’s guidance ‘Photography and Photomontages in Landscape and Visual Impact Assessment’ and provided a professional opinion on the robustness of the judgements made in the LVIAs.

7.5.21	The review by LUC raised a number of concerns in relation to the methodology used, assumptions made, and judgements reached within the two LVIAs submitted in support of application 15/1935/FUL.  For example, LUC questioned whether appropriate landscape and visual receptors had been used and with regards to the assessment itself, questioned whether the “assessment allows a sufficiently clear understanding of the landscape effects”.  They also commented that as a consequence of the development occupying most of the site, there was little opportunity for landscaping.  No further review has been undertaken at this time of the LVIAs submitted with the current applications.

7.6	Amenity

7.6.1	One of the core planning principles listed in the NPPF (paragraph 17) is that planning should;

“Always seek to secure high quality design and a good standard of amenity for all existing and future occupiers of land and buildings”.

7.6.2	Policy CP12 of the Core Strategy (adopted October 2011) states that the Council will expect development proposals to;

a) Have regard to the local context and conserve or enhance the character, amenities and quality of an area.

c) Protect residential amenities by taking into account the need for adequate levels and disposition of privacy, prospect, amenity and garden space.

7.6.3	Policy CP12 is supported by Policy DM1 and Appendix 2 of the Development Management Policies LDD (adopted July 2013).  Appendix 2 includes design criteria against which new development should be assessed in order to ensure that they would not result in demonstrable harm to the residential amenity of neighbouring occupiers.

7.6.4	It is necessary to consider the impact of each of the 4 proposed developments on the residential amenities of existing neighbouring occupiers and future occupiers of the proposed developments.  In this case the immediate neighbours are largely existing offices; however, the Local Planning Authority has a duty to consider the impact of development on all neighbours.

Impact on Neighbours:

7.6.5	The application site is in an area predominantly consisting of employment uses; however, there are three residential neighbours (The Dell, Glendale and The Oaks) in a terrace to the south east of the application site.  The Dell is the closest and is located approximately 50 metres from the proposed development at the closest point (all APPLICATIONS).  Due to the relative siting with the three dwellings located to the south east of the application site, notwithstanding the indicated mass and bulk of the buildings proposed in each of the 4 applications (ranging from 5 to 7 storeys) it is not considered that any of the proposed developments would result in demonstrable harm to the residential amenities of occupiers of The Dell, Glendale and The Oaks by virtue of overshadowing or loss of light.  However, due to the scale, height, mass and bulk of the buildings proposed in each of the applications and their indicative designs incorporating high levels of glazing, balconies and green roofs for amenity use, the proposed developments would represent an un-neighbourly and overbearing form of development to the detriment of the residential amenity of occupiers of The Dell, Glendale and The Oaks and would result in both actual and perceived overlooking.  The 4 applications would therefore each fail to comply with Policy CP12 of the Core Strategy (adopted October 2011) and Policy DM1 and Appendix 2 of the Development Management Policies LDD (adopted July 2013).

7.6.6	As previously noted, the majority of neighbouring units to the application site are in employment use.  Units 5 and 6 Wolsey Business Park are two-storey units located to the north of the application site.  These buildings are set back approximately 22 metres from the central access road which runs through the business park, with car parking between the buildings and access road.  The indicative design of the proposed buildings in each of the 4 applications includes a built element from second floor level upwards that would overhang the central access road.  As such, the proposed buildings would be sited approximately 22 metres from Units 5 and 6 Wolsey Business Park.

7.6.7	The maximum building heights indicated range from 16.6 metres (APPLICATION B) to 22.8 metres (APPLICATION D) and all the buildings proposed in the applications would therefore significantly exceed the height of the existing two-storey units to the north.  The overall bulk of the development indicated in APPLICATIONS B and C would be exacerbated by the inclusion of additional 4 storey buildings between the main projecting wings of the main building in these schemes.  Whilst it is noted that the design of Units 5 and 6 includes limited glazing to their southern elevations facing the application site and that therefore unacceptable overlooking is unlikely; given the height, mass, bulk and limited separation distances proposed and  the siting of Units 5 and 6 due north of the application site, it is considered that each of the 4 proposed developments would represent un-neighbourly and overbearing forms of development, contrary to Policy CP12 of the Core Strategy (adopted October 2011).  There are similar concerns with regards to the impact on existing neighbouring Units within Orbital Business Park which are located even closer at approximately 6 metres from each of the proposed developments at the closest point and in relation to Unit 2 Wolsey Business Park sited approximately 17 metres from each of the 4 proposed developments at the closest point.  In this regard, each of the 4 applications would also fail to comply with Policy CP6 (employment and economic development) of the Core Strategy (adopted October 2011) which advises that the Council will support development that sustains parts of the District as attractive areas for business location.

Amenity of Future Occupiers:

ALL APPLICATIONS

7.6.8	The residential units proposed in the 4 applications would vary in size from 1 – 3 bedrooms.  In the case of all of the applications, those units facing south would have outlook across Tolpits Lane towards Hampermill Lake and beyond and would not be overlooked by any neighbouring buildings.  Similarly, due to the unbroken nature of the southern elevations, there would be limited opportunity for overlooking between flats on this southern elevation.

7.6.9	Units in the outside western flank elevation proposed in each of the 4 applications would face towards the neighbouring Orbital Business Park to the west of the application site.  These are two storey buildings with limited glazing in their rear elevations such that overlooking of the proposed units is unlikely.  However, the western flank of the proposed buildings in each of the 4 applications would be sited close to the boundary (approximately 2 metres in each case) resulting in a limited separation distance of approximately 6 metres between the flank elevation of the proposed buildings and rear elevation of buildings in the neighbouring Orbital Business Park.  Whilst this would not affect units from the second floor upwards, it is considered that the amenity of occupiers of units on the ground and first floors would be adversely affected by reason of the poor outlook that would be afforded to them.

7.6.10	Unit 2 Wolsey Business Park is located to the east of the application site and is a three-storey office building with glazing to all elevations.  This neighbouring Unit is separated from the application site by the roundabout and access road and is set back such that its southern most elevation is roughly level with the existing rear elevation of Unit 3.  The proposed buildings would be of a significantly increased depth and height in all cases, resulting in flank wings of approximately 41 metres and increasing to approximately 48 metres from 2nd floor level at a distance of approximately 17 metres from Unit 2 at the closest point.  Whilst the separation distance and slightly angled siting of the buildings is such that it is not considered that occupiers of proposed flats in the eastern elevation would be adversely affected through overshadowing or loss of light, it is considered that opportunities for overlooking of the proposed flats (specifically floors 1 – 3) would exist and that this would be to the detriment of the residential amenities of future occupiers.

7.6.11	There are significant concerns about the relationship between proposed units on the three central wings and inside elevations of the outer wings in terms of both overlooking and overshadowing.  In relation to overlooking, guidance within Appendix 2 of the Development Management Policies LDD (adopted July 2013) indicates that distances between buildings should be sufficient so as to prevent overlooking, particularly from upper floors.  As an indicative figure, 28 metres should be achieved between the faces of single or two storey buildings backing onto each other, although this should be greater between buildings in excess of two-storeys especially dwellings/flats with elevations which directly face one another.  Whilst it is acknowledged that the indicative 28 metre figure refers to ‘back to back’ distances and ‘distances between buildings’, it is still considered relevant due to the layouts proposed.  Appendix 2 also advises that development should not incorporate balconies which would overlook neighbouring properties to any degree.

7.6.12	The submitted details (all APPLICATIONS) indicate a separation distance of approximately 43 metres between the central projecting wing and outer wings, however, two smaller projecting wings would be located between the main wings and would have angled elevations such that glazing and balconies would be angled towards the deeper main wings resulting in opportunities for overlooking as discussed below.

7.6.13	For example and with reference to APPLICATION A, the balcony to Unit 12 would be angled towards and sited only 6.5 metres from the balcony at Unit 16.  A bedroom window to Unit 12 would be sited only 5 metres from the window of Unit 15.  Similar relationships are found in each of the applications and would result in unacceptable levels of actual and perceived overlooking for a number of future occupiers in each of the 4 proposed schemes.

7.6.14	The limited separation distances and overall heights of the proposed buildings as described would also result in proposed residential units experiencing a significant sense of enclosure and overshadowing.  This would be most notable for those at lower levels on the smaller projections and on the central elements of the main projections closest to the core of the proposed buildings in each of the 4 applications and would be exacerbated due to the orientation of the proposed developments with the projections facing north.

APPLICATIONS B & C

7.6.15	APPLICATIONS B and C include additional 4 storey buildings, indicated as sited between the main projecting wings of the proposed buildings in each application.  These would be sited approximately 10 metres from the flank elevations of the main projecting wings.  The submitted details indicate that these buildings would be mixed use, including office, retail, café and gym uses.  The floor plans submitted also indicate green roofs intended to provide amenity space.  Whilst residential units within the fifth floor (APPLICATION C) would not be adversely affected, central units on lower floors (all floors in the case of APPLICATION B) would be adversely affected both in terms of overlooking from these smaller 4 storey buildings (including from their green roofs) and as a result of the sense of enclosure created by the siting of these smaller buildings which would significantly restrict their outlook in the case of a number of units.

ALL APPLICATIONS

7.6.16	The indicative design of the development in each of the 4 applications includes an overhang element to the three main projecting wings.  The overhang at second floor level would have a depth of approximately 7 metres and it is considered that this would impact adversely on the residential units below, resulting in an overwhelming sense of enclosure exacerbated by the north facing orientation of these units meaning these units would receive little daylight or direct sunlight making them gloomy and providing poor living conditions and outlook.

7.6.17	For the reasons set out above in relation to overlooking and the overbearing nature of the proposed developments as indicated on the submitted details in each of the 4 applications, it is considered that each of the 4 proposed developments would result in demonstrable harm to the visual and residential amenities of future occupiers and each would therefore fail to comply with Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM1 and Appendix 2 of the Development Management Policies LDD (adopted July 2013).

Amenity Space:

7.6.18	Standards for amenity space provision are set out in Appendix 2 of the Development Management Policies Local Development Document (adopted July 2013).  The requirements are 21 square metres for 1 bedroom flats, 31 square metres for 2 bedroom flats and 41 square metres for 3 bedroom flats.

7.6.19	Appendix 2 comments that depending on the character of the development, the space may be provided in the form of private gardens or in part, may contribute to formal spaces/settings for groups of buildings.  Communal space for flats should be well screened from highways and casual passers-by.

7.6.20	The requirement and provision for all of the applications (based on details of unit numbers / sizes as set out within the submitted accommodation schedules and plans) is set out in the table below.  The ‘proposed’ figures are also taken from the submitted accommodation schedules.

|  | Required | Proposed | Difference |
| --- | --- | --- | --- |
| Application A | 160 x 21 106 x 31 8 x 41 = 6,974m² | 1,060.61m² (balconies) 5,880.06m² (roof garden) = 6,940.67m² | -33.33m² |
| Application B | 205 x 21 119 x 31 8 x 41 = 8,322m² | 1,042.61m² (balconies) 6,745.24m² (roof garden) = 7,787.85m² | -534.15m² |
| Application C | 246 x 21 149 x 31 8 x 41 = 10,113m² | 1,325.52m² (balconies) 6745.24m² (roof garden) = 8,070.76m² | -2,042.24m² |
| Application D | 242 x 21 166 x 31 8 x 41 = 10,556m² | 1,590.43m² (balconies) 5,880.06m² (roof garden) = 7,470.49m² | -3,085.51m² |

7.6.21	The table above indicates that in the case of each of the 4 applications the proposed amenity space to be provided would fail to comply with adopted standards.  In the case of APPLICATION A, the shortfall would not be significant, however, in the case of APPLICATION B there would be a shortfall of approximately 500 square metres, increasing to approximately 3,000 square metres in the case of APPLICATION D.

7.6.22	In the case of all 4 applications, amenity space would be provided on private balconies and external communal roof terraces.  APPLICATIONS A and D would also include some external communal space at ground floor level, indicated on the submitted details as a 677.99m² childrens’ play area.

7.6.23	In addition to the significant deficiencies in the amount of amenity space to be provided in applications B, C and D, it is necessary to consider the usability of the space provided and also the proximity of the site to public open space which may to some degree offset the requirement for full provision on site.  In terms of usability of the green roofs proposed, whilst these would provide a single space where residents could sit, there are concerns regarding the safety and practicality of use of the green roof, for example by children for ball games.  It is also noted that the green roof would be exposed due to its height above ground.  The submitted details in relation to all 4 applications indicate that most residential units (those above ground floor) would benefit from a small private balcony, however, these would not provide significant amounts of space for each unit due to their size.  Additionally, the buildings in all 4 applications are orientated with their main unbroken elevations facing south and as such balconies and terraces to the north and on the central flanks of the projecting wings in each application would experience limited sunlight due to both the indicated orientation and scale of the buildings.  This would be likely to affect adversely the usability of these balconies.

7.6.24	Croxley Common Moor is an allocated Open Space in the Site Allocations Local Development Document (adopted November 2014) and is located to the north of the application site beyond the Ebury Way.  Croxley Common Moor is within short walking distance of the site and there are footpaths where residents could walk, however, the designation of the Open Space as both a Local Nature Reserve and Site of Special Scientific would restrict the use of the Open Space to some degree.

7.6.25	It is acknowledged that, in addition to the green roof and private balconies, the applications propose a number of other facilities for residents including an internal arboretum, community room, training room and projector room.  Whilst these would provide amenities for residents, this is not the amenity or garden space that standards in Appendix 2 refer to and is not considered to overcome the shortfalls of outdoor amenity space.

7.6.26	In summary, the proposed developments (APPLICATIONS B, C & D) would fail to provide sufficient amenity space.  Whilst some reduction in standards against policy may be acceptable, the shortfalls would be significant.  Furthermore, in addition to the quantitative shortfall, the usability of the amenity space within all 4 applications would be limited and would be to the detriment of the residential amenities of future occupiers and contrary to the adopted standards set out in Appendix 2 of the Development Management Policies LDD (adopted July 2013).

7.7	Highways Matters

7.7.1	The NPPF recognises that transport policies have an important role to play in facilitating sustainable development.  Paragraph 32 of the NPPF comments that all developments that generate significant amounts of movement should be supported by a Transport Statement or Transport Assessment.  Plans and decisions should take account of whether:

- the opportunities for sustainable transport modes have been taken up depending on the nature and location of the site, to reduce the need for major transport infrastructure;

- safe and suitable access to the site can be achieved for all people; and

improvements can be undertaken within the transport network that cost effectively limit the significant impacts of the development.

7.7.2	Policy CP1 of the Core Strategy (adopted October 2011) advises that in ensuring all development contributes to the sustainability of the District, it is necessary to take into account the need to reduce the need to travel by locating development in accessible locations and promoting a range of sustainable transport modes.

7.7.3	Policy CP10 (Transport and Travel) of the Core Strategy (adopted October 2011) advises that all development should be designed and located to minimise the impacts of travel by motor vehicle on the District.  In particular, major development will be expected to be located in areas highly accessible by the most sustainable modes of transport, and to all people of abilities in a socially inclusive and safe manner in accordance with the following user hierarchy:

- Pedestrians, particularly people with restricted mobility

- Cyclists and where appropriate horse riders

- Public transport

- All forms of motor vehicle

7.7.4	Policy CP10 of the Core Strategy (adopted October 2011) advises that   Development will need to demonstrate that:

i) It provides a safe and adequate means of access

j) It is appropriate in scale to the existing infrastructure…

k) It is integrated with the wider network of transport routes…

l) It makes adequate provision for all users…

m) It includes where appropriate, provision for public transport either within the scheme or through contributions

n) The impact of the proposal on transport has been fully assessed…

o) The proposal is accompanied by a draft Green Travel Plan

7.7.5	The 4 applications have all been accompanied by Transport Assessments (TA).  The Highways Authority have reviewed the submitted details and raise no objections to APPLICATIONS A and D subject to conditions/S106 Agreements, however, they have raised objections to APPLICATIONS B and C due to the overprovision of car parking.  Whilst there is an overprovision in all applications, it is greatest in these two schemes.  In the case of APPLICATIONS B and C, the Highways Authority do not consider that the applicant has demonstrated that the proposals are not contrary to policies which seek to promote sustainable transport use.

7.7.6	In the case of all 4 applications, further information is required in relation to swept path assessments, service and delivery plans and provision of pedestrian access, though this could be secured by conditions on any consents.

7.7.7	The submitted details for each application include Trip Generation information.  The total proposed trip generation profile was compared to the existing trip generation profile to establish the net impact at the site for each proposal.  In addition to the vehicular trip generation profile, the applicant has provided multimodal trip generation profile for method of travel to work based on 2011 Census data.  This shows a demand for public transport and footway access, as such, contributions would be sought to improve connections to/from the site.

7.7.8	The applicant has provided junction assessments for the access road with Dwight Road and the Tolpits Lane / Dwight Road roundabout.  The results applying to all 4 applications indicate that the junctions will operate within desired thresholds and with minimal queuing and delays for public highway links.  In summary, the junction capacity results indicate that the additional traffic added to the network as a consequence of the proposed developments is not likely have a detrimental impact on the operation of the highway network in the vicinity of the proposed developments.

7.7.9	The TA includes detailed collision data for the surrounding road network.  A review of the 5 years of collision data from 1 April 2011 to 31 March 2016 revealed that there were 26 collisions in the vicinity of the site; all were designated as slight collisions.  The Highway Authority have commented that the review of the collision data does not present any safety issues in respect of any of the 4 applications regarding  highway impacts and all collisions appear to have human judgement errors as contributing factors.  As such, it is not considered likely that any of the proposed developments would create or exacerbate any existing highway issues.

7.7.10	In relation to access, it is noted that each of the 4 proposed developments would utilise the existing site access from Dwight Road, with a new access from the site road providing access to basement car parks.  As part of any Reserved Matters applications, it would be necessary to provide dimensions and swept path assessments demonstrating that vehicles can safely enter and depart from the car parking spaces.  Provision for safe pedestrian access to the site would also be required.

7.7.11	Servicing areas for servicing, delivery and collection are referred to in the submitted TAs.  A Refuse, Servicing and Delivery Plan would be required to ensure that suitable vehicles would be able to access the appropriate bays.  The TAs refer to appropriate swept path assessments being submitted at Reserved Matters stage and the Highways Authority raise no objection to this approach.

7.7.12	The Highway Authority notes that the proposed parking provision for APPLICATIONS B and C would significantly exceed maximum standards set out in the Three Rivers Local Plan.  Whilst acknowledging that the District is the parking authority, the Highway Authority considers that the provision of extra parking will encourage the use of a personal vehicle as a mode of transport.  As such, they do not consider the overprovision of parking to be appropriate in this location and they raise an objection to APPLICATIONS B and C in this regard.

7.7.13	The application site is not currently served by any bus services.  The submitted details refer to a subsidised bus route and state that the applicant and Arrivia are ‘keen to work together’ and have sought views from HCC regarding their preferred route.  The Design and Access Statements refer to two routes, route 1 connecting the site with Rickmansworth Town and route 2 connecting the site to Watford.  It is noted that this is as per that proposed pursuant to application 16/2497/FUL.  In relation to application 16/2497/FUL, HCC (Sustainable Transport and Development Officer) confirmed that their preference would be for route 2 (Watford) and that a S106 Agreement between the applicant and Arrivia could ensure a certain level of service for a certain length of time.  While improvements to public transport would be supported, no S106 Agreement in relation to any of the 4 applications has been agreed at this time to secure the necessary provisions.  In the absence of such agreements, the Local Planning Authority would have no mechanism by which to secure the long term provision of a bus route serving the site.

7.7.14	The Business Park is flat and considered walkable by the applicant, however, the Highways Authority notes that whilst there are footways on Dwight Road, there is no footway provision on Tolpits Lane.

7.7.15	Travel Plans in respect of each of the 4 applications would be required to encourage sustainable transport modes and to limit the reliance on private vehicles.  Separate Travel Plans for the residential and commercial elements would be required.

7.7.16	Construction Management Plans (CMP) would need to be secured by condition in relation to each of the 4 applications to ensure that construction vehicles will not have a detrimental impact on highway safety and usage in the vicinity of the site. The CMP would need to provide adequate parking for construction vehicles on-site to prevent on-street conflict and adverse impacts to the highway safety.

7.7.17	Section 106 Agreements would also be required in respect of each of the 4 applications to secure for example, Construction Traffic Management Plan Planning Obligations.  The Highways Authority has requested a contribution towards the provision of a foot and cycle way along the northern verge of Tolpits Lane linking the site with the Moor Park estate and the underground station within it.

7.7.18	In summary, whilst Hertfordshire County Council as Highway Authority raise no objections to APPLICATIONS A and D subject to conditions/S106 Agreements; they raise objections to APPLICATIONS B and C due to overprovision of car parking.  Concerns regarding swept path assessments, servicing and delivery plans and provision of pedestrian access in respect of each of the 4 applications could be secured via conditions and Section 106 Agreements, however, in the absence of a Section 106 Agreement to secure such measures the developments would fail to comply with Policies CP1, CP8 and CP10 of the Core Strategy (adopted October 2011).

7.8	Parking

7.8.1	The NPPF (paragraph 39) advises that, in setting local parking standards for residential and non-residential development, local planning authorities should take into account:

- The accessibility of the development;

- The type, mix and use of development;

- The availability of and opportunities for public transport;

- Local car ownership levels; and

- An overall need to reduce the use of high-emission vehicles.

7.8.2	Policy DM13 of the Development Management Policies LDD (adopted July 2013) advises that development should make provision for parking in accordance with the parking standards set out in Appendix 5.

7.8.3	The adopted standards set out in Appendix 5 of the Development Management Policies LDD (adopted July 2013) vary depending on the nature/use proposed.  Appendix 5 also advises that the standards for car parking (except C3 Residential) may be adjusted according to which zone the proposed development is located in.  The application site is located in zone 4 (least accessible) where provision of 75-100% may be applied.

7.8.4	The level of parking proposed for each application is set out in the table below:

|  | Car Spaces | Cycle Spaces | Motor Bike Space |
| --- | --- | --- | --- |
| Application A | 661 | 468 | 44 |
| Application B | 851 | 516 | 42 |
| Application C | 943 | 516 | 44 |
| Application D | 943 | 468 | 44 |

7.8.5	In the case of each application 35 of the car spaces provided would be accessible spaces.  Each application would also include a 20 car electric car sharing scheme and 20 bike electric bike sharing scheme.

7.8.6	As noted above, parking standards vary depending on the use proposed.  The tables below sets out the proposed parking requirements for each application based on the details submitted within the accommodation schedules provided for each application.

7.8.7	APPLICATION A

| Use | Standard | Requirement |
| --- | --- | --- |
| Residential (C3) | 1 bed – 1.75 spaces 2 bed – 2 spaces 3 bed – 2.25 spaces | 160 x 1.75 = 280 106 x 2 = 212 8 x 2.25 = 18 = 510 |
| Supermarket (A1) | 1 space per 30sqm gross floor area | 218.52m² / 30 = 7 |
| Office (B1) | 1 space per 30sqm gross floor area | 2,231.24m² / 30 = 74 |
| Café/coffee shop (A3) | 1 space per 5sqm of floorspace of dining area plus 3 space per 4 employees | 204.96m² / 5 = 41 NB. Does not include reference to employees as details unknown. |
| Gymnasium/ Aerobics/Yoga Fitness Area (D2) | 1 space per 22sqm | 218.51m² / 22 = 10 |
|  |  | Total: 642 spaces |

NB. The requirement for the non-residential aspects totals 132 spaces.  Applying a zonal reduction (75-100% for Zone 4) would result in a requirement for 99-132 spaces to serve non-residential uses and a total requirement for 609-642 spaces.

7.8.8	APPLICATION B

| Use | Standard | Requirement |
| --- | --- | --- |
| Residential (C3) | 1 bed – 1.75 spaces 2 bed – 2 spaces 3 bed – 2.25 spaces | 205 x 1.75 = 358.75 119 x 2 = 238 8 x 2.25 = 18 = 615 |
| Supermarket (A1) | 1 space per 30sqm gross floor area | Supermarket 1 (127.27m²) Supermarket 2 (108.81m²) 236.08m² / 30 = 8 |
| Office (B1) | 1 space per 30sqm gross floor area | 2,345.38m² / 30 = 78 |
| Café/coffee shop (A3) | 1 space per 5sqm of floorspace of dining area plus 3 space per 4 employees | 127.27m² / 5 = 25 NB. Does not include reference to employees as details unknown. |
| Gymnasium/ Aerobics/Yoga Fitness Area (D2) | 1 space per 22sqm | 201.46m² / 22 = 9 |
|  |  | Total: 735 spaces |

NB. The requirement for the non-residential aspects totals 120 spaces.  Applying a zonal reduction (75-100% for Zone 4) would result in a requirement for 90-120 spaces to serve non-residential uses and a total requirement for 705-735 spaces.

7.8.9	APPLICATION C

| Use | Standard | Requirement |
| --- | --- | --- |
| Residential (C3) | 1 bed – 1.75 spaces 2 bed – 2 spaces 3 bed – 2.25 spaces | 246 x 1.75 = 430.5 149 x 2 = 298 8 x 2.25 = 18 = 747 |
| Supermarket (A1) | 1 space per 30sqm gross floor area | 236.08m² / 30 = 8 |
| Office (B1) | 1 space per 30sqm gross floor area | 2,345.38m² / 30 = 78 |
| Café/coffee shop (A3) | 1 space per 5sqm of floorspace of dining area plus 3 space per 4 employees | 127.27m² / 5 = 25 NB. Does not include reference to employees as details unknown. |
| Gymnasium/ Aerobics/Yoga Fitness Area (D2) | 1 space per 22sqm | 310.27m² / 22 = 14 |
|  |  | Total: 872 spaces |

NB. The requirement for the non-residential aspects totals 125 spaces.  Applying a zonal reduction (75-100% for Zone 4) would result in a requirement for 94-125 spaces to serve non-residential uses and a total requirement for 841-872 spaces.

7.8.10	APPLICATION D

| Use | Standard | Requirement |
| --- | --- | --- |
| Residential (C3) | 1 bed – 1.75 spaces 2 bed – 2 spaces 3 bed – 2.25 spaces | 242 x 1.75 = 423.5 166 x 2 = 332 8 x 2.25 = 18 = 774 |
| Supermarket (A1) | 1 space per 30sqm gross floor area | 218.52m² / 30 = 7 |
| Office (B1) | 1 space per 30sqm gross floor area | 2,231.24m² / 30 = 74 |
| Café/coffee shop (A3) | 1 space per 5sqm of floorspace of dining area plus 3 space per 4 employees | 204.96m² / 5 = 41 NB. Does not include reference to employees as details unknown. |
| Gymnasium/ Aerobics/Yoga Fitness Area (D2) | 1 space per 22sqm | 218.51m² / 22 = 10 |
|  |  | Total: 906 spaces |

NB. The requirement for the non-residential aspects totals 132 spaces.  Applying a zonal reduction (75-100% for Zone 4) would result in a requirement for 99-132 spaces to serve non-residential uses and a total requirement for 873-906 spaces.

7.8.11	It is noted that in the case of each application, the development is indicated to include other uses such as a cinema and training rooms that have not been included within the tables above.  It is understood that these would be for the sole use of residents and would not therefore generate their own parking requirement.  A condition could be included on any grant of consent to restrict the use of these elements to residents.

7.8.12	A summary of the car parking requirement and proposed provision for each application is set out in the table below:

|  | Max. Requirement | Proposed | Difference |
| --- | --- | --- | --- |
| Application A | 642 | 661 | +19 |
| Application B | 735 | 851 | +116 |
| Application C | 872 | 943 | +71 |
| Application D | 906 | 943 | +37 |

7.8.13	In the case of each application, the level of car parking indicated would exceed the maximum policy requirement.  In the case of APPLICATION A, this would by 19 spaces, increasing to 116 spaces in the case of APPLICATION B.  The Highway Authority has contended that there would be overprovision of car parking, specifically in relation to APPLICATIONS B and C.  It is noted that in each case, the provision does not take account of provision for employees for the A3 use.  However, with a maximum A3 floor area of 204m² provided in any one of the 4 applications it is not considered that there would be a significant number of employees.

7.8.14	Whilst sufficient parking provision would be provided to meet standards, details of the allocation have not been provided and would be required to be secured by condition

7.8.15	Requirements for parking for disabled motorists are also set out in Appendix 5 of the Development Management Policies LDD (adopted July 2013).  For residential development, the requirement is for 1 accessible space for every dwelling built to mobility standards such as Lifetime Homes.  Each of the 4 applications includes provision of 35 accessible car parking spaces and any additional requirement could be taken into consideration at Reserved Matters stage when further details, including the number of dwellings being built to mobility standards, were known.

7.8.16	Section 4 of the NPPF relates to ‘promoting sustainable transport’ and paragraph 30 advises that encouragement should be given to solutions which support reductions in greenhouse gas emissions and reduce congestion.  Measures such as an electric car and bike share scheme would therefore be supported, subject to the appropriate level of detail regarding the operation and maintenance of such schemes.

7.8.17	It is noted that the submitted Design Access Statements with the 4 applications indicate that existing neighbouring occupiers Camelot and Nissan have a shortage of parking and as such capacity has been built in to accommodate parking from neighbouring buildings who could rent spaces..  However no evidence has been provided at this time to demonstrate that there is demand from neighbouring businesses for additional parking and similarly no details of how the arrangement would be managed have been provided.

7.8.18	Appendix 5 also sets out standards for cycle parking provision which should be incorporated into new developments.  These standards vary depending on the use proposed.  The tables below set out the proposed cycle parking requirements for each application based on the details submitted within the accommodation schedules provided for each application.

7.8.19	APPLICATION A

| Use | Standard | Requirement |
| --- | --- | --- |
| Residential (C3) | Flats 1 space per 2 units | 274 / 2 = 137 |
| Supermarket (A1) | 1 short term space per 150sqm gross floor area plus 1 long term space per 10 maximum staff on site at any one time | 218.52m² / 150 = 1.5 NB. Does not include reference to staff as details unknown. |
| Office (B1) | 1 short term space per 500sqm gross floor area plus 1 space per 10 full time staff | 2,231.24m² / 500 = 4 NB. Does not include reference to staff as details unknown. |
| Café/coffee shop (A3) | 1 short term space per 100sqm gross floor area plus 1 long term space per 10 maximum staff on site at any one time | 204.96m² / 100 = 2 NB. Does not include reference to staff as details unknown. |
| Gymnasium/ Aerobics/Yoga Fitness Area (D2) | 1 short term space per 25sqm | 218.51m² / 25sqm = 9 |
|  |  | Total: 154 |

7.8.20	APPLICATION B

| Use | Standard | Requirement |
| --- | --- | --- |
| Residential (C3) | Flats 1 space per 2 units | 332 / 2 = 166 |
| Supermarket (A1) | 1 short term space per 150sqm gross floor area plus 1 long term space per 10 maximum staff on site at any one time | 236.08m² / 150 = 1.5 NB. Does not include reference to staff as details unknown. |
| Office (B1) | 1 short term space per 500sqm gross floor area plus 1 space per 10 full time staff | 2,345.38m² / 500 = 5 NB. Does not include reference to staff as details unknown. |
| Café/coffee shop (A3) | 1 short term space per 100sqm gross floor area plus 1 long term space per 10 maximum staff on site at any one time | 127.27m² / 100 = 1 NB. Does not include reference to staff as details unknown. |
| Gymnasium/ Aerobics/Yoga Fitness Area (D2) | 1 short term space per 25sqm | 201.46m² / 25sqm = 8 |
|  |  | Total: 182 |

7.8.21	APPLICATION C

| Use | Standard | Requirement |
| --- | --- | --- |
| Residential (C3) | Flats 1 space per 2 units | 403 / 2 = 201.5 |
| Supermarket (A1) | 1 short term space per 150sqm gross floor area plus 1 long term space per 10 maximum staff on site at any one time | 236.08m² / 150 = 1.5 NB. Does not include reference to staff as details unknown. |
| Office (B1) | 1 short term space per 500sqm gross floor area plus 1 space per 10 full time staff | 2,345.38 m² / 500 = 5 NB. Does not include reference to staff as details unknown. |
| Café/coffee shop (A3) | 1 short term space per 100sqm gross floor area plus 1 long term space per 10 maximum staff on site at any one time | 127.27m² / 100 = 1 NB. Does not include reference to staff as details unknown. |
| Gymnasium/ Aerobics/Yoga Fitness Area (D2) | 1 short term space per 25sqm | 310.27m² / 25sqm = 12 |
|  |  | Total: 221 |

7.8.22	APPLICATION D

| Use | Standard | Requirement |
| --- | --- | --- |
| Residential (C3) | Flats 1 space per 2 units | 416 / 2 = 208 |
| Supermarket (A1) | 1 short term space per 150sqm gross floor area plus 1 long term space per 10 maximum staff on site at any one time | 218.52m² / 150 = 1.5 NB. Does not include reference to staff as details unknown. |
| Office (B1) | 1 short term space per 500sqm gross floor area plus 1 space per 10 full time staff | 2,231.24m² / 500 = 4 NB. Does not include reference to staff as details unknown. |
| Café/coffee shop (A3) | 1 short term space per 100sqm gross floor area plus 1 long term space per 10 maximum staff on site at any one time | 204.96m² / 100 = 2 NB. Does not include reference to staff as details unknown. |
| Gymnasium/ Aerobics/Yoga Fitness Area (D2) | 1 short term space per 25sqm | 218.51m² / 25sqm = 9 |
|  |  | Total: 225 |

7.8.23	A summary of the cycle parking requirement and proposed provision for each application is set out in the table below:

|  | Requirement | Proposed | Difference |
| --- | --- | --- | --- |
| Application A | 154 | 468 | +314 |
| Application B | 182 | 516 | +334 |
| Application C | 221 | 516 | +295 |
| Application D | 225 | 468 | +243 |

7.8.24	All 4 applications are indicated as providing cycle parking that would well exceed standards.

7.8.25	It is concluded that the proposed developments in each case would provide sufficient parking (car and bicycle) to meet the standards set out in Appendix 5 of the Development Management Policies LDD (adopted July 2013).  However, as discussed elsewhere, the overprovision of car parking adds to sustainability concerns, particularly in the case of APPLICATIONS B and C.

7.9	Refuse/Re-cycling/Waste

7.9.1	Policy DM10 (Waste Management) of the Development Management Policies LDD (adopted July 2013) advises that the Council will ensure that there is adequate provision for the storage and recycling of waste and that these facilities are fully integrated into design proposals.  New developments will only be supported where:

i) The siting or design of waste/recycling areas would not result in any adverse impact to residential or work place amenity

ii) Waste/recycling areas can be easily accessed (and moved) by occupiers and by local authority/private waste providers

iii) There would be no obstruction of pedestrian, cyclists or driver site lines.

7.9.2	The plans in respect of each of the 4 applications illustrate that consideration has been given to the provision of refuse/recycling storage.  Further details regarding the amount of provision and the design and external appearance of any external facilities could be secured by condition.

7.9.3	It is noted that the indicative buildings (all 4 applications) include a second floor overhang element which would overhang the access road at a minimum height of 6.3 metres above ground level.  The Environmental Protection Services Manager has previously advised that refuse collection vehicles have a maximum height of 3.8 metres and as such the proposed overhangs would provide sufficient clearance.

7.9.4	HCC Minerals and Waste Policy Team have not commented on the current applications, however, in relation to previous application 16/2497/FUL they noted that the site is located within Employment Land Area of Search (ELAS) 212 as designated within the Waste Site Allocations Document (adopted July 2014).  Uses such as general industrial (B2) and storage and distribution (B8) are considered compatible waste management uses.  As such, HCC commented that mixed use including Classes A1, A3, B1, C3, D1 and D2 would not be compatible with the purpose of designating the site in Waste Site Allocations Document.  However, whilst HCC did not wish to see the loss of identified ELAS for non-waste use, they did not raise an objection due to sufficient land remaining within the area.

7.9.5	Government policy seeks to ensure that all planning authorities take responsibility for waste management.  This is reflected in the county council’s adopted waste planning documents.  In particular, the waste planning documents seek to promote the sustainable management of waste in the county and encourage districts and boroughs to have regard to the potential for minimising waste generated by development.  This includes encouraging re-use of unavoidable waste where possible and the use of recycled materials where appropriate to the construction.

7.9.6	Waste Policy 12 (Sustainable Design, Construction and Demolition) of the Waste Site Allocations Document (adopted July 2014) requires all relevant construction projects to be supported by a Site Waste Management Plan (SWMP) which should aim to reduce the amount of waste produced on site and should contain information including types of waste removed from the site and where that waste is being taken to.

7.9.7	The requirement for appropriate SWMP would be a conditional requirement of any grants of consent.

7.10	Wildlife & Biodiversity

7.10.1	Section 40 of the Natural Environment and Rural Communities Act 2006 requires Local Planning Authorities to have regard to the purpose of conserving biodiversity.  This is further emphasised by regulation 3(4) of the Habitat Regulations 1994 which state that Councils must have regard to the strict protection for certain species required by the EC Habitats Directive.  The Habitats Directive places a legal duty on all public bodies to have regard to the habitats directive when carrying out their functions.

7.10.2	The NPPF (paragraph 109) advises that the planning system should contribute to and enhance the natural and local environment by:

“Minimising impacts on biodiversity and providing net gains in biodiversity where possible, contributing to the Government’s commitment to halt the overall decline in biodiversity, including by establishing coherent ecological networks that are more resilient to current and future pressures”.

7.10.3	When determining planning applications, the NPPF (paragraph 118) advises that Local Planning Authorities should aim to conserve and enhance biodiversity by applying principles which include:

- If significant harm resulting from a development cannot be avoided, adequately mitigated, or, as a last resort, compensated for, then planning permission should be refused.

- Opportunities to incorporate biodiversity in and around developments should be encouraged.

7.10.4	National Planning Policy requires Local Authorities to ensure that a protected species survey is undertaken for applications where biodiversity may be affected prior to the determination of a planning application.  This is in line with Policy CP9 (Green Infrastructure) of the Core Strategy (adopted October 2011) which sets out the Council’s priorities for green infrastructure, which includes conserving and enhancing key biodiversity habitats and species.

7.10.5	Policy DM6 of the Development Management Policies LDD (adopted July 2013) requires that development conserves, enhances, and where appropriate, restores biodiversity.

7.10.6	The Biodiversity Checklists submitted with the applications identify the need for further surveys and as such the applications have all been accompanied by a Primary Ecology Report, Secondary Ecology Report and Technical Note.

7.10.7	The applications (A – D) are considered to raise the same considerations in relation to wildlife and biodiversity.

7.10.8	Croxley Common Moor, which is a Site of Special Scientific Interest (SSSI), Local Natural Reserve (LNR), and Common Land, lies just over 100 metres to the north of the site.  Croxley Common Moor is the only known location in the UK for the Slate Sober moth.  There is also a non-statutory Local Wildlife Site 40 metres to the south - Hamper Mill Lakes (reference: 89/004).  These sites have been recognised for their habitats and species interest.

7.10.9	Natural England has not commented in relation to the current applications, however, it is noted that they raised no objection to previous application 16/2497/FUL subject to conditions to ensure that there were no impacts upon Croxley Common Moor SSSI or its non-notified species of significant concern, the Slate Sober moth.

7.10.10	It is concluded against this background that in order to ensure an appropriate lighting design that is appropriate to the location and that does not impact negatively on biodiversity; a condition should be included on any grant of consent for any of the 4 applications requiring a detailed lighting plan to be submitted.  This would also address the concerns of Hertfordshire Ecology who have made similar comments regarding the need for further lighting details.

7.10.11	It is also concluded, given the proximity of Croxley Common Moor SSSI to the application site, that intrusive site investigations would be required prior to construction of any of the 4 proposed developments  to enable  a conceptual understanding of ground and surface water interactions and hydro-ecological requirements for the designated features at Croxley Common Moor SSSI and associated ground and surface water catchments; provision of information on ground and surface water levels pre construction; details of dewatering during construction including timings and volumes of water to be abstracted and modelling information to show potential lowering of groundwater during and post dewatering at Croxley Common Moor SSSI. These are matters that can be addressed by planning condition

7.10.12	Each of the 4 applications may provide opportunities to incorporate features into the design which are beneficial to wildlife, such as the incorporation of roosting opportunities for bats or the installation of bird nest boxes as set out in the submitted Ecology reports.  The Local Planning Authority would seek to secure such necessary measures by condition on any grant of consent.

7.10.13	Whilst the concerns of the Biodiversity Projects Officer are noted, subject to appropriate conditions it is not considered that any of the 4 proposed developments would result in demonstrable harm to the SSSI, LNR or protected species and the proposals would therefore be acceptable in this regard in accordance with Policy CP9 of the Core Strategy (adopted October 2011) and Policy DM6 of the Development Management Policies LDD (adopted July 2013).

7.11	Amenity & Children’s Play Space Provision in New Residential Development

7.11.1	Policy DM11 of the Development Management Policies LDD (adopted July 2013) relates to ‘Open Space, Sport and Recreation Facilities and Children’s Play Space’.  The policy advises that in order to ensure that new residential developments do not exacerbate deficiencies in open space and children’s play space, new residential development will be expected to provide for amenity and children’s play space:

“Developments of 25 or more dwellings or 0.6ha (whichever is greater) should make provision on site for open space and play space.  10% of the site area should be set aside as open space, and where the development is likely to be occupied by families with children 2% of the site area should provide formal equipped play facilities”.

7.11.2	The application forms refer to a site area of 1.36 hectares.  Provision of 10% of the site would therefore result in a requirement for 1,360 square metres as open space and 272 square metres as formal equipped play space.  This requirement is additional to the requirement for private amenity space considered at 7.6 above.

7.11.3	As set out in the amenity section above, the submitted details indicate that due to the footprint of the built form there would be limited opportunity for open space to be provided at ground level around the buildings, although APPLICATIONS A and D would provide some opportunity for external play space at ground floor level (approximately 677.99 square metres in each case).

7.11.4	The plans indicate that the majority of external amenity space would be provided on the green roofs proposed as part of each application.  Whilst these would exceed the 10% site area figure in terms of open space, as discussed previously in this analysis, in the case of APPLICATIONS B, C and D there would be a significant shortfall of amenity space against standards, and in the case of APPLICATION A provision for open space in addition to private amenity space would result in a significant shortfall of 1,393,33 square metres.

7.12	Trees & Landscape

7.12.1	Paragraph 109 of the NPPF advises that the planning system should contribute to and enhance the natural and local environment by protecting and enhancing valued landscapes.

7.12.2	In ensuring that all development contributes to the sustainability of the District, Policy CP12 of the Core Strategy (adopted October 2011) advises that development proposals should:

i) Ensure that development is adequately landscaped and is designed to retain, enhance or improve important existing natural features; landscaping should reflect the surrounding landscape of the area and where appropriate integrate with adjoining networks of green open spaces.

7.12.3	Policy DM6 (Biodiversity, Trees, Woodlands, Watercourses and Landscaping) of the Development Management Policies LDD (adopted July 2013) advises that development proposals for new development should be submitted with landscaping proposals which seek to retain trees and other landscape and nature conservation features.  Landscaping proposals should also include new trees to enhance the landscape of the site and its surroundings as appropriate.

7.12.4	It is noted that landscaping is a reserved matter in the case of each application, however, illustrative details have been provided.  The Council’s Landscape Officer has viewed the submitted details and notes that there are no arboricultural constraints for the site.  He therefore raises no objection subject to conditions.  It is noted that there are some early mature trees (including London Plan and Pine Trees) which are part of the original planting scheme for the Business Park that have established well within the vicinity.  It would be beneficial as regards the maintenance of trees and landscaping for these existing amenity features to be incorporated into the proposed development in each of the 4 proposed schemes as they would make an instant impact and would contribute to any future landscaping.

7.12.5	With regards to landscaping, the Landscape Officer notes that the roof gardens would be exposed sites and therefore they assume that appropriate plants will be used and measures implemented to limit exposure.  Further details would be required by condition.

7.12.6	The Landscape Officer has requested a number of conditions to be included on any grant of consent in respect of any of the 4 applications including landscaping details; landscape management plan details; no felling or lopping and tree protection details.

7.13	Flood Risk / Drainage / Contamination / Pollution

7.13.1	The NPPF at paragraph 94 states:

“Local Planning Authorities should adopt proactive strategies to mitigate and adapt to climate change, taking full account of flood risk, costal change and water supply and demand considerations”.

7.13.2	Paragraph 100 states:

“Inappropriate development in areas at risk of flooding should be avoided by directing development away from areas at highest risk, but where development is necessary, making it safe without increasing flood risk elsewhere…”

7.13.3	Paragraph 103 continues:

“When determining planning applications, local planning authorities should ensure flood risk is not increased elsewhere and only consider development appropriate in areas at risk of flooding where, informed by a site specific flood risk assessment following the sequential test, and if required the exception test, it can be demonstrated that;

- within the site, the most vulnerable development is located in areas of lowest flood risk unless there are overriding reasons to prefer a different location; and

- development is appropriately flood resilient and resistant, including safe access and escape routes where required, and that any residual risk can be safely managed, including by emergency planning; and its gives priority to the use of sustainable drainage systems”.

7.13.4	Paragraph 109 of the NPPF advises that the planning system should contribute to and enhance the natural and local environment by remediating and mitigating despoiled, degraded, derelict, contaminated and unstable land, where appropriate.

7.13.5	Policy CP1 of the Core Strategy (adopted October 2011) recognises that taking into account the need to (b) avoid development in areas at risk of flooding will contribute towards the sustainability of the District.  In certain circumstances, and provided effective mitigation measures are in place, development may be acceptable in such areas.  Policy CP1 of the Core Strategy (adopted October 2011) also advises that in order to contribute towards the sustainability of the District, development proposals should manage and reduce risk of and from pollution in relation to quality of land, air and water dealing with land contamination.

7.13.6	Policy CP12 of the Core Strategy (adopted October 2011) also acknowledges that the Council will expect development proposals to build resilience into a site’s design taking into account climate change, for example flood resistant design.

7.13.7	Policy DM8 (Flood Risk and Water Resources) of the Development Management Policies LDD (adopted July 2013) advises that development will only be permitted where it would not be subject to unacceptable risk of flooding and would not unacceptably exacerbate the risks of flooding elsewhere and that the Council will support development where the quantity and quality of surface and groundwater are protected and where there is adequate and sustainable means of water supply.

7.13.8	Policy DM9 (Contamination and Pollution Control) of the Development Management Policies LDD (adopted July 2013) requires development to include Sustainable Drainage Systems (SuDs).  Policy DM9 also advises that the Council will refuse planning permission for development which would or could give rise to polluting emissions to land, air and/or water.

7.13.9	The 4 applications have all been accompanied by a Contamination Report, Flood Risk Report and a Drainage Strategy Survey.

7.13.10	No comments have been received from the Environment Agency in relation to the current applications, however, it is acknowledged that they raised no objection subject to conditions to previous application 15/1935/FUL.  A Preliminary Risk Assessment (PRA) was considered necessary in respect of that application and the same should be provided in respect of each of the 4 applications identifying all previous uses of the site; potential contaminants associated with those uses; a conceptual model of the site indicating sources, pathways and receptors; and potentially unacceptable risks arising from contamination at the site.  This is necessary as the site is located within Source Protection Zone (SPZ) 1 for public water supplies.  A site investigation scheme based on the PRA, an options appraisal and remediation strategy and verification plan would also be required in respect of each of the 4 applications

7.13.11	A Verification Report demonstrating completion of the works set out in the approved remediation strategy and the effectiveness of the remediation was also considered necessary in respect of each of the 4 applications and should include results of sampling and monitoring carried out in accordance with the approved verification plan to demonstrate that the site remediation criteria have been met. It should also include a plan for longer-term monitoring of pollutant linkages, maintenance and arrangements for contingency action, as identified in the verification plan.

7.13.12	Further details would be required by condition for any piling or other foundation designs using penetrative measures with respect to the proposed development in any of the 4 applications.  This would include details of any ground source heating/cooling and geo-thermal systems using penetrative methods.  This would be necessary as some piling techniques can facilitate pathways for contaminants to migrate to ground water.

7.13.13	A long-term monitoring and maintenance plan in respect of contamination including a timetable of monitoring and submission of reports to the Local Planning Authority would also be required by condition on any planning consent.

Sustainable Urban Drainage Systems (SuDS):

7.13.14	Herts County Council as Lead Local Flood Authority (HCC LLFA) are responsible for assessing surface water flood risk.  Having reviewed the submitted details relating to the 4 applications they raise no objection on flood risk grounds and consider that the proposed developments can be adequately drained and mitigated if carried out in accordance with the overall drainage strategy for each application.

7.3.15	The proposed drainage strategies are based upon attenuation and discharge into Thames Water sewer restricted to 10l/s with the required storage volumes 720m3 to be provided on site. HCC acknowledge that Thames Water have been contacted and have no objection in principle to the schemes.

7.3.16	HCC note that the final detail has not been provided at this stage and would therefore require a detailed surface water drainage scheme to be submitted in respect of each application, this would be secured via condition on any grant of consent.  Subject to appropriate conditions, no objection is raised to the proposed sustainable urban drainage system (SuDS) and the 4 proposals as discrete applications would be acceptable in this regard in accordance with Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM8 of the Development Management Policies LDD (adopted July 2013).

7.14	Sustainability

7.14.1	The National Planning Policy Framework, at Paragraph 6. states that “The purpose of the planning system is to contribute to the achievement of sustainable development. The policies in paragraphs 18 to 219, taken as a whole, constitute the Government’s view of what sustainable development in England means in practice for the planning system.”

7.14.2	Paragraph 7 continues…”There are three dimensions to sustainable development: economic, social and environmental. These dimensions give rise to the need for the planning system to perform a number of roles:

- an economic role – contributing to building a strong, responsive and competitive economy, by ensuring that sufficient land of the right type is available in the right places and at the right time to support growth and innovation; and by identifying and coordinating development requirements, including the provision of infrastructure;

- a social role – supporting strong, vibrant and healthy communities, by providing the supply of housing required to meet the needs of present and future generations; and by creating a high quality built environment, with accessible local services that reflect the community’s needs and support its health, social and cultural well-being; and

- an environmental role – contributing to protecting and enhancing our natural, built and historic environment; and, as part of this, helping to improve biodiversity, use natural resources prudently, minimise waste and pollution, and mitigate and adapt to climate change including moving to a low carbon economy.

7.14.3	It is necessary to consider whether the application proposals constitute sustainable development, in the context of the NPPF’s advice.

Economic Role:

7.14.4	The application site forms part of the Tolpits Lane Employment Site Allocation as allocated by the Site Allocations Local Development Document which was adopted in November 2014.  Policy SA2 of the Site Allocations LDD (adopted November 2014) states that;

“Allocated employment sites will be safeguarded for business, industrial and storage or distribution uses.

Sites allocated as having potential for mixed use development may provide for mixed use development including, but not limited to business, industrial and storage or distribution; residential or community uses”.

The site is not allocated for mixed use development.

7.14.5	Policy CP6 (Employment and Economic Development) of the Core Strategy (adopted October 2011) seeks to support the sustainable growth of the Three Rivers economy by (for example);

“(j) Continuing to focus employment use within the key employment areas within the District”.

7.14.6	Tolpits Lane is listed within Policy CP6 as one of the key employment areas that the policy seeks to safeguard.  In addition, and in recognition of the need to protect employment sites, the Council made a non-immediate Article 4 Direction on 5 August 2016 which removes permitted development rights for changes of use from office or light industrial to residential use within the Tolpits Lane employment area.  The direction was confirmed on 13 October 2016 and will come into effect on 5 August 2017 (in the case of B1 office to C3 residential) and the 30 September 2017 (in the case of B1 light industrial to C3 residential).  The Council considers the Article 4(1) Direction to be necessary to protect the use of this important location for employment within the District and to secure the viability of businesses within the employment area.

7.14.7	The details submitted for all 4 applications indicate that the proposed developments would all result in a net loss of Class B1 (office) floor space (refer to table at 7.2.10 above) and each is therefore contrary to Policy SA2 of the Site Allocations LDD (adopted November 2014) and Policy CP6 of the Core Strategy (adopted October 2011).

7.14.8	As set out at section 7.6 (amenity) above, concerns are raised regarding the impact of the proposed developments on neighbouring business and commercial units and in this regard each of the 4 developments would fail to comply with Policy CP6 (employment and economic development) of the Core Strategy (adopted October 2011) which advises that the Council will support development that sustains parts of the District as attractive areas for businesses to locate.

7.14.9	Whilst it is acknowledged that there may be some economic benefits generated by each of the 4 applications, for example, as a result of employment during construction and the inclusion of employment generating components to the schemes (e.g. office, retail, gym, café etc), and the provision of additional housing stock, these are not so significant as to overcome the identified objections to the proposed developments outlined above to justify approval of any of the 4 applications.

Social Role:

7.14.10	Each of the 4 proposed developments would conflict with the  requirements of Policy CP4 of the Core Strategy (adopted October 2011) and SPD: Affordable Housing (approved June 2011), together with the appropriate housing mix in terms of unit sizes.

7.14.11	All applications fail to provide sufficient amenity space for future residents contrary to policy and by reason of the excessive scale, height and indicative design incorporating extensive glazing and balconies and relationship with surrounding development, each of the 4 proposed developments would result in unacceptable levels of overlooking of a number of the residential units and would create a sense of unacceptable overbearing and enclosure that would be experienced by a number of the residential units to the detriment of the residential amenities of future residential occupiers of any of the 4 schemes.

7.14.12	Concerns are also raised in respect of each of the 4 applications regarding the inaccessibility of the application site in terms of proximity and accessibility to existing public transport links, shops and services.

7.14.13	As such, each of the 4 proposed developments is not considered to be socially sustainable.

Environmental Role:

7.14.14	Paragraph 93 of the NPPF states that:

“Planning plays a key role in helping to shape places to secure radical reductions in greenhouse gas emissions, minimising vulnerability and providing resilience to the impacts of climate change, and supporting the delivery of renewable and low carbon energy and associated infrastructure”.

7.14.15	Policy CP1 of the Core Strategy (adopted October 2011) stipulates that all applications for new residential development of one unit and above must be submitted with a ‘CPLAN Energy and Sustainability Statement’ demonstrating the extent to which sustainability principles have been incorporated into the location, design, construction and future use of proposals, and the carbon emissions.  Following the withdrawal of ‘CPLAN’, from the 21 March 2016 the requirement has been to submit an Energy Statement to demonstrate that development will achieve a 5% saving over the 2013 Building Regulations in accordance with the attached guidance instead of CPLAN.

7.14.16	Policy DM4 (Carbon Dioxide Emissions and On-Site Renewable Energy) of the Development Management Policies LDD (adopted July 2013) stipulates that from 2013, applicants will be required to demonstrate that development will produce 5% less carbon emissions than Building regulations Part L requirements (2013) having regard to feasibility and viability.  This may be achieved through a combination of energy efficiency measures, incorporation of on-site low carbon and renewable technologies, connection to a local, decentralised, renewable or low carbon energy supply.  The policy states that from 2016, applicants will be required to demonstrate that new residential development will be zero carbon. However, the Government has announced that it is not pursuing zero carbon and the standard remains that development should produce 5% less carbon dioxide emissions than Building Regulations Part L (2013) requirements having regard to feasibility and viability.

7.14.17	The submitted details in respect of each of the 4 applications demonstrate that the development proposals would exceed these requirements and therefore meet the requirements of Policy DM4 of the Development Management Policies LDD (adopted July 2013) in this regard.

7.14.18	The submitted Design and Access Statements with each of the 4 applications provide details of the measures proposed to ensure that the buildings would be an EPC A+ building (total energy neutral).

7.14.19	The credentials of the proposed developments in terms of energy efficiency and renewable energy are commendable, however, the NPPF is clear that there are three dimensions to sustainable development, and as such the advanced energy efficiency credentials in themselves may not amount to a sustainable development in the context of the NPPF’s advice.

7.14.20	Paragraph 17 of the NPPF sets out the Core Planning Principles which include;

“Actively manage patterns of growth to make the fullest possible use of public transport, walking and cycling, and focus significant development in locations which are or can be made sustainable”.

7.14.21	The proposed application site is not located within an existing sustainable location for residential development and it has not been demonstrated that the location could be made sustainable for such use.  The proposed location for housing common to all 4 applications is considered to be contrary to the principles set out in the Spatial Strategy and contrary to the objectives set out in the Spatial Strategy’s Place Shaping Policies (PSP1, PSP2 and PSP3) and Policy CP1 (Overarching Policy on Sustainable Development) of the Core Strategy (adopted October 2011) and the NPPF.

7.14.22	The applications also propose an electric car and bike share scheme.  Whilst it is acknowledged that these proposed measures would contribute towards reducing greenhouse gas emissions, they would account for only a small element of the proposed parking provision within any of the 4 schemes.  It is also noted that there would be an overprovision of car parking in all schemes, albeit greatest within APPLICATIONS B and C.  This is indicative of the likelihood that the residential use of the site in any of the 4 schemes would be heavily reliant on use of the private car.

7.23.24	None of the 4 applications have therefore adequately demonstrated that they are environmentally sustainable as required by the National Planning Policy Framework and would be contrary to Policies CP1, CP9 and CP12 of the Core Strategy (adopted October 2011) and Policies DM6 and DM8 of the Development Management Policies LDD (adopted July 2013).

Summary:

7.23.25	There is increasing pressure on Local Authorities to ensure that development is sustainable.  The 4 applications each propose a number of renewable and energy efficiency measures which are viewed positively including the advanced energy efficiency credentials of the proposed built developments.

7.23.26	However the common application site is not located within an existing sustainable location and it has not been demonstrated that the location could be made sustainable as a location for the housing development proposed

7.23.27	Paragraphs 6-9 of the NPPF are clear that ‘sustainability’ should not be interpreted narrowly. Sustainable development also includes ‘seeking positive improvements in the quality of the built environment as well as in people’s quality of life’. On this wider basis, as set out above, it is not considered that any of the 4 proposed developments would amount to sustainable development for the purposes of the NPPF.

7.15	Planning Obligations & CIL

7.15.1	Paragraph 203 of the NPPF states that:

“Local Planning Authorities should consider whether otherwise unacceptable development could be made acceptable through the use of conditions or planning obligations.  Planning Obligations should only be used where it is not possible to address unacceptable impacts through a planning condition”.

7.15.2	Policy CP8 (Infrastructure and Planning Obligations) of the Core Strategy (adopted October 2011) advises that:

“Development proposals will provide, or make adequate contributions towards, infrastructure and services to:

a) Make a positive contributions to safeguarding or creating sustainable, linked communities

b) Offset the loss of any infrastructure through compensatory provision

c) Meet ongoing maintenance costs where appropriate”.

7.15.3	The Community Infrastructure Levy (CIL) is a planning charge, introduced by the Planning Act 2008 as a tool for local authorities in England and Wales to help deliver infrastructure to support the development of their area. It came into effect In Three Rivers on 1 April 2015 following the adoption by the Council of the CIL Charging Schedule in February 2015.  However, the application site is located within Area C where the CIL charge is nil.  As such no CIL contribution would be required in relation to the proposed development.

7.15.4	If required, fire hydrant provision could be secured via condition if the applications were not subject to a Section 106 Agreement for other contributions/provisions.

7.15.5	A S106 Agreement would be required in the case of each of the 4 applications to secure a Construction and Logistics Plan Planning Obligation and to provide for a Travel Plan.  The S106 agreements should include a contribution towards the provision of a foot and cycle way along the northern verge of Tolpits Lane linking the site with the Moor Park estate and the underground station within it.  Mitigation measures for local junctions and the provision of a subsidised bus service would also be required to be secured in respect of any of the 4 applications under a section 106 Agreement prior to the grant of planning permission.

7.15.6	A S106 Agreement would also be required to secure the maintenance of open space and play space to be provided in respect of each of the 4 schemes.

7.15.7	No S106 Agreement in respect of any of the 4 applications has been agreed at this time.  In the absence of a S106 Agreement to secure these provisions, all or any of the proposed developments would fail to comply with Policies CP1, CP8 and CP10 of the Core Strategy (adopted October 2011) and Policy DM11 of the Development Management Policies LDD (adopted July 2013).

7.15.8	As noted at section 7.4 above, each of the proposed developments fails to meet the requirements of Policy CP4 of the Core Strategy (adopted October 2011) and SPD: Affordable Housing (approved June 2011).

7.16	Other Matters

7.16.1	Screening is the process of deciding which projects will require an Environmental Impact Assessment (EIA).  It is noted that no request for a Screening Opinion has been made at this time.

7.17	Conclusion

7.17.1	The applications have been considered on their individual merits, although as set out in the analysis above there are a number of common issues.

8.	Recommendation(s)

8.1	That PLANNING PERMISSION 17/0046/OUT (APPLICATION A) BE REFUSED for the following reasons:

Reasons for Refusal:–

R1	The application site is within an allocated Employment Area and the proposal would result in a net loss of Class B1 (office) floor space.  The proposed development, by reason of its excessive scale, height and proximity to existing buildings would be an overbearing and overdominant form of development to the detriment of the amenity of neighbouring business occupiers.  The proposal would therefore be contrary to Policy SA2 of the Site Allocations LDD (adopted November 2014) and Policy CP6 of the Core Strategy (adopted October 2011).

R2	The site is remotely located with regard to services and access and is not within an existing sustainable location and it has not been demonstrated that the location could be made sustainable.  The proposed development would encourage high dependence on use of the private car.  The NPPF seeks to ensure that patterns of growth make the fullest possible use of public transport and that developments that generate significant movement are located where the use of sustainable transport modes can be maximised.  The proposal is contrary to these strategic aims which seek to achieve a sustainable pattern of development and would undermine the Spatial Strategy of the Core Strategy.  The development would accordingly fail to comply with Policies CP1, CP2, CP10 and CP12 of the Core Strategy (adopted October 2011) and the NPPF.

R3	The proposed development, by reason of its excessive height, width, scale, bulk and building-to-plot ratio which would not allow adequate space for landscaping, would be cramped, excessively dominant in the street scene and surrounding locality and would have an adverse impact on the landscape.  It would represent overdevelopment of the site on these grounds, to the detriment of the visual amenities, appearance and character of the street scene and the area as a whole.  This is contrary to Policies CP1, CP3 and CP12 of the Core Strategy (adopted October 2011), Policies DM1 and DM7 and Appendix 2 of the Development Management Policies (adopted July 2013) and the NPPF.

R4	The proposed development, by reason of its excessive scale, bulk, height and close proximity to existing buildings, would result in an overbearing and overdominant form of development and overlooking to the detriment of the visual and residential amenities of neighbouring occupiers at The Dell, Glendale and The Oaks, contrary to Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM1 and Appendix 2 of the Development Management Policies LDD (adopted July 2013).

R5	The proposed development, by reason of its excessive scale, height and indicative design (including layout/arrangement of flats) incorporating extensive glazing and balconies and relationship with existing development, would result in unacceptable levels of overlooking of a number of the proposed residential units and would create a sense of overbearing and enclosure that would be experienced by a number of the residential units to the detriment of the residential amenities of future residential occupiers.  The proposed development would also fail to provide sufficient and usable amenity space.  This is contrary to Policies CP1 and CP12 of the Core Strategy (adopted October 2011), Policies DM1 and DM11 and Appendix 2 of the Development Management Policies LDD (adopted July 2013) and the NPPF.

R6	The proposed development fails to meet the requirements of Policy CP4 of the Core Strategy (adopted October 2011) and SPD: Affordable Housing (approved June 2011) in that the scheme would not provide policy compliant affordable housing and it has not been demonstrated that it would not be viable to meet policy requirements for affordable provision; a S106 Agreement has not been agreed to secure provision; and the proposed housing mix would fail to meet the requirements of Policy CP3 of the Core Strategy (adopted October 2011) and no evidence or justification has been provided to support the proposed housing mix.

R7	The proposed development would result in a significant increase in demand for sustainable transport provision in the area.  There is currently a shortage of these facilities in the area. The proposed development would exacerbate this situation and in the absence of a signed agreement to provide public transport services to serve the site under the provisions of Section 106 of Town and Country Planning Act 1990.The proposal would also  requirement  fire hydrant provision. The application therefore fails to meet the requirements of Policies CP1, CP8 and CP10 of the Core Strategy (adopted October 2011).

Informatives

I1	In line with the requirements of the National Planning Policy Framework and in accordance with the Town and Country Planning (Development Management Procedure) (England) Order 2015, the Local Planning Authority has considered, in a positive and proactive manner, whether the planning objections to this proposal could be satisfactorily resolved within the statutory period for determining the application. Whilst the applicant and the Local Planning Authority have engaged in pre-application discussions regarding the development of the site and further information has been submitted through the course of the application, the proposed development fails to comply with the requirements of the Development Plan and does not maintain/improve the economic, social and environmental conditions of the District.

8.2	That PLANNING PERMISSION 17/0015/OUT (APPLICATION B) BE REFUSED for the following reasons:

Reasons for Refusal:–

R1	The application site is within an allocated Employment Area and the proposal would result in a net loss of Class B1 (office) floor space.  The proposed development, by reason of its excessive scale, height and proximity to existing buildings would be an overbearing and overdominant form of development to the detriment of the amenity of neighbouring business occupiers.  The proposal would therefore be contrary to Policy SA2 of the Site Allocations LDD (adopted November 2014) and Policy CP6 of the Core Strategy (adopted October 2011).

R2	The site is remotely located with regard to services and access and is not within an existing sustainable location and it has not been demonstrated that the location could be made sustainable.  There is also high dependence on the private car evidenced by the overprovision of car parking spaces.   The NPPF seeks to ensure that patterns of growth make the fullest possible use of public transport and that developments that generate significant movement are located where the use of sustainable transport modes can be maximised.  The proposal is contrary to these strategic aims which seek to achieve a sustainable pattern of development and would undermine the Spatial Strategy of the Core Strategy.  The development would accordingly fail to comply with Policies CP1, CP2, CP10 and CP12 of the Core Strategy (adopted October 2011) and the NPPF.

R3	The proposed development, by reason of its excessive height, width, scale, bulk and building-to-plot ratio which would not allow adequate space for landscaping, would be cramped, excessively dominant in the street scene and surrounding locality and would have an adverse impact on the landscape.  It would represent overdevelopment of the site on these grounds, to the detriment of the visual amenities, appearance and character of the street scene and the area as a whole.  This is contrary to Policies CP1, CP3 and CP12 of the Core Strategy (adopted October 2011), Policies DM1 and DM7 and Appendix 2 of the Development Management Policies (adopted July 2013) and the NPPF.

R4	The proposed development, by reason of its excessive scale, bulk, height and close proximity to existing buildings, would result in an overbearing and overdominant form of development and overlooking to the detriment of the visual and residential amenities of neighbouring occupiers at The Dell, Glendale and The Oaks, contrary to Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM1 and Appendix 2 of the Development Management Policies LDD (adopted July 2013).

R5	The proposed development, by reason of its excessive scale, height and indicative design (including layout/arrangement of flats) incorporating extensive glazing and balconies and relationship with existing development, would result in unacceptable levels of overlooking of a number of the proposed residential units and would create a sense of overbearing and enclosure that would be experienced by a number of the residential units to the detriment of the residential amenities of future residential occupiers. The proposed development would also fail to provide sufficient and usable amenity space to the detriment of the residential amenities of future occupiers.  This is contrary to Policies CP1 and CP12 of the Core Strategy (adopted October 2011), Policies DM1 and DM11 and Appendix 2 of the Development Management Policies LDD (adopted July 2013) and the NPPF.

R6	The proposed development fails to meet the requirements of Policy CP4 of the Core Strategy (adopted October 2011) and SPD: Affordable Housing (approved June 2011) in that the scheme would not provide policy compliant affordable housing and it has not been demonstrated that it would not be viable to meet policy requirements for affordable provision; a S106 Agreement has not been agreed to secure provision; and the proposed housing mix would fail to meet the requirements of Policy CP3 of the Core Strategy (adopted October 2011) and no evidence or justification has been provided to support the proposed housing mix.

R7	The proposed development would result in a significant increase in demand for sustainable transport provision in the area.  There is currently a shortage of these facilities in the area. The proposed development would exacerbate this situation and in the absence of a signed agreement to provide public transport services to serve the site under the provisions of Section 106 of Town and Country Planning Act 1990.The proposal would also  requirement  fire hydrant provision. The application therefore fails to meet the requirements of Policies CP1, CP8 and CP10 of the Core Strategy (adopted October 2011).

Informatives

I1	In line with the requirements of the National Planning Policy Framework and in accordance with the Town and Country Planning (Development Management Procedure) (England) Order 2015, the Local Planning Authority has considered, in a positive and proactive manner, whether the planning objections to this proposal could be satisfactorily resolved within the statutory period for determining the application. Whilst the applicant and the Local Planning Authority have engaged in pre-application discussions regarding the development of the site and further information has been submitted through the course of the application, the proposed development fails to comply with the requirements of the Development Plan and does not maintain/improve the economic, social and environmental conditions of the District.

8.3	That PLANNING PERMISSION 16/2735/OUT (APPLICATION C) BE REFUSED for the following reasons:

Reasons for Refusal:–

R1	The application site is within an allocated Employment Area and the proposal would result in a net loss of Class B1 (office) floor space.  The proposed development, by reason of its excessive scale, height and proximity to existing buildings would be an overbearing and overdominant form of development to the detriment of the amenity of neighbouring business occupiers.  The proposal would therefore be contrary to Policy SA2 of the Site Allocations LDD (adopted November 2014) and Policy CP6 of the Core Strategy (adopted October 2011).

R2	The site is remotely located with regard to services and access and is not within an existing sustainable location and it has not been demonstrated that the location could be made sustainable.  There is also high dependence on the private car evidenced by the overprovision of car parking spaces.   The NPPF seeks to ensure that patterns of growth make the fullest possible use of public transport and that developments that generate significant movement are located where the use of sustainable transport modes can be maximised.  The proposal is contrary to these strategic aims which seek to achieve a sustainable pattern of development and would undermine the Spatial Strategy of the Core Strategy.  The development would accordingly fail to comply with Policies CP1, CP2, CP10 and CP12 of the Core Strategy (adopted October 2011) and the NPPF.

R3	The proposed development, by reason of its excessive height, width, scale, bulk and building-to-plot ratio which would not allow adequate space for landscaping, would be cramped, excessively dominant in the street scene and surrounding locality and would have an adverse impact on the landscape.  It would represent overdevelopment of the site on these grounds, to the detriment of the visual amenities, appearance and character of the street scene and the area as a whole.  This is contrary to Policies CP1, CP3 and CP12 of the Core Strategy (adopted October 2011), Policies DM1 and DM7 and Appendix 2 of the Development Management Policies (adopted July 2013) and the NPPF.

R4	The proposed development, by reason of its excessive scale, bulk, height and close proximity to existing buildings, would result in an overbearing and overdominant form of development and overlooking to the detriment of the visual and residential amenities of neighbouring occupiers at The Dell, Glendale and The Oaks, contrary to Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM1 and Appendix 2 of the Development Management Policies LDD (adopted July 2013).

R5	The proposed development, by reason of its excessive scale, height and indicative design (including layout/arrangement of flats) incorporating extensive glazing and balconies and relationship with existing development, would result in unacceptable levels of overlooking of a number of the proposed residential units and would create a sense of overbearing and enclosure that would be experienced by a number of the residential units to the detriment of the residential amenities of future residential occupiers. The proposed development would also fail to provide sufficient and usable amenity space to the detriment of the residential amenities of future occupiers.  This is contrary to Policies CP1 and CP12 of the Core Strategy (adopted October 2011), Policies DM1 and DM11 and Appendix 2 of the Development Management Policies LDD (adopted July 2013) and the NPPF.

R6	The proposed development fails to meet the requirements of Policy CP4 of the Core Strategy (adopted October 2011) and SPD: Affordable Housing (approved June 2011) in that the scheme would not provide policy compliant affordable housing and it has not been demonstrated that it would not be viable to meet policy requirements for affordable provision; a S106 Agreement has not been agreed to secure provision; and the proposed housing mix would fail to meet the requirements of Policy CP3 of the Core Strategy (adopted October 2011) and no evidence or justification has been provided to support the proposed housing mix.

R7	The proposed development would result in a significant increase in demand for sustainable transport provision in the area.  There is currently a shortage of these facilities in the area. The proposed development would exacerbate this situation and in the absence of a signed agreement to provide public transport services to serve the site under the provisions of Section 106 of Town and Country Planning Act 1990.The proposal would also  requirement  fire hydrant provision. The application therefore fails to meet the requirements of Policies CP1, CP8 and CP10 of the Core Strategy (adopted October 2011).

Informatives

I1	In line with the requirements of the National Planning Policy Framework and in accordance with the Town and Country Planning (Development Management Procedure) (England) Order 2015, the Local Planning Authority has considered, in a positive and proactive manner, whether the planning objections to this proposal could be satisfactorily resolved within the statutory period for determining the application. Whilst the applicant and the Local Planning Authority have engaged in pre-application discussions regarding the development of the site and further information has been submitted through the course of the application, the proposed development fails to comply with the requirements of the Development Plan and does not maintain/improve the economic, social and environmental conditions of the District.

8.4	That PLANNING PERMISSION 16/2709/OUT (APPLICATION D) BE REFUSED for the following reasons:

Reasons for Refusal:–

R1	The application site is within an allocated Employment Area and the proposal would result in a net loss of Class B1 (office) floor space.  The proposed development, by reason of its excessive scale, height and proximity to existing buildings would be an overbearing and overdominant form of development to the detriment of the amenity of neighbouring business occupiers.  The proposal would therefore be contrary to Policy SA2 of the Site Allocations LDD (adopted November 2014) and Policy CP6 of the Core Strategy (adopted October 2011).

R2	The site is remotely located with regard to services and access and is not within an existing sustainable location and it has not been demonstrated that the location could be made sustainable.  There is also high dependence on the private car. The NPPF seeks to ensure that patterns of growth make the fullest possible use of public transport and that developments that generate significant movement are located where the use of sustainable transport modes can be maximised.  The proposal is contrary to these strategic aims which seek to achieve a sustainable pattern of development and would undermine the Spatial Strategy of the Core Strategy.  The development would accordingly fail to comply with Policies CP1, CP2, CP10 and CP12 of the Core Strategy (adopted October 2011) and the NPPF.

R3	The proposed development, by reason of its excessive height, width, scale, bulk and building-to-plot ratio which would not allow adequate space for landscaping, would be cramped, excessively dominant in the street scene and surrounding locality and would have an adverse impact on the landscape.  It would represent overdevelopment of the site on these grounds, to the detriment of the visual amenities, appearance and character of the street scene and the area as a whole.  This is contrary to Policies CP1, CP3 and CP12 of the Core Strategy (adopted October 2011), Policies DM1 and DM7 and Appendix 2 of the Development Management Policies (adopted July 2013) and the NPPF.

R4	The proposed development, by reason of its excessive scale, bulk, height and close proximity to existing buildings, would result in an overbearing and overdominant form of development and overlooking to the detriment of the visual and residential amenities of neighbouring occupiers at The Dell, Glendale and The Oaks, contrary to Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM1 and Appendix 2 of the Development Management Policies LDD (adopted July 2013).

R5	The proposed development, by reason of its excessive scale, height and indicative design (including layout/arrangement of flats) incorporating extensive glazing and balconies and relationship with existing development, would result in unacceptable levels of overlooking of a number of the proposed residential units and would create a sense of overbearing and enclosure that would be experienced by a number of the residential units to the detriment of the residential amenities of future residential occupiers. The proposed development would also fail to provide sufficient and usable amenity space to the detriment of the residential amenities of future occupiers.  This is contrary to Policies CP1 and CP12 of the Core Strategy (adopted October 2011), Policies DM1 and DM11 and Appendix 2 of the Development Management Policies LDD (adopted July 2013) and the NPPF.

R6	The proposed development fails to meet the requirements of Policy CP4 of the Core Strategy (adopted October 2011) and SPD: Affordable Housing (approved June 2011) in that the scheme would not provide policy compliant affordable housing and it has not been demonstrated that it would not be viable to meet policy requirements for affordable provision; a S106 Agreement has not been agreed to secure provision; and the proposed housing mix would fail to meet the requirements of Policy CP3 of the Core Strategy (adopted October 2011) and no evidence or justification has been provided to support the proposed housing mix.

R7	The proposed development would result in a significant increase in demand for sustainable transport provision in the area.  There is currently a shortage of these facilities in the area. The proposed development would exacerbate this situation and in the absence of a signed agreement to provide public transport services to serve the site under the provisions of Section 106 of Town and Country Planning Act 1990.The proposal would also  requirement  fire hydrant provision. The application therefore fails to meet the requirements of Policies CP1, CP8 and CP10 of the Core Strategy (adopted October 2011).

Informatives

I1	In line with the requirements of the National Planning Policy Framework and in accordance with the Town and Country Planning (Development Management Procedure) (England) Order 2015, the Local Planning Authority has considered, in a positive and proactive manner, whether the planning objections to this proposal could be satisfactorily resolved within the statutory period for determining the application. Whilst the applicant and the Local Planning Authority have engaged in pre-application discussions regarding the development of the site and further information has been submitted through the course of the application, the proposed development fails to comply with the requirements of the Development Plan and does not maintain/improve the economic, social and environmental conditions of the District.

APPENDIX 1 – PLAN NUMBERS

APPLICATION A - 17/0046/OUT

MOOR_V2.0_4002 V2.0		Topographical Survey Plan

MOOR_V2.0_4003 V2.0		Existing Site Plan

MOOR_V2.0_4004 V2.0		Proposed Site Plan

MOOR_V2.0_4005 V2.0		Block Plan

MOOR_V2.0_4006 			Access Plan

MOOR_V2.0_4007 A V2.0		Existing Unit 3 Floor Plans

MOOR_V2.0_4007 B V2.0		Existing Unit 3 Elevations

MOOR_V2.0_4008 V2.0		Existing Unit 4 Floor Plans and Elevations

MOOR_V2.0_4009 A V2.0		Proposed Basement Level -2 Plan (Part –A)

MOOR_V2.0_4009 B V2.0		Proposed Basement Level -2 Plan (Part –B)

MOOR_V2.0_4010 A V2.0		Proposed Basement Level -1 Plan (Part –A)

MOOR_V2.0_4010 B V2.0		Proposed Basement Level -1 Plan (Part –B)

MOOR_V2.0_4011 A V2.0		Proposed Ground Floor Plan (Part –A)

MOOR_V2.0_4011 B V2.0		Proposed Ground Floor Plan (Part –B)

MOOR_V2.0_4012 A V2.0		Proposed First Floor Plan (Part –A)

MOOR_V2.0_4012 B V2.0		Proposed First Floor Plan (Part –B)

MOOR_V2.0_4013 A V2.0		Proposed 2nd – 3rd Floor Plan (Part –A)

MOOR_V2.0_4013 A V2.0		Proposed 2nd – 3rd Floor Plan (Part –A)

MOOR_V2.0_4013 B V2.0		Proposed 2nd – 3rd Floor Plan (Part –B)

MOOR_V2.0_4014 A V2.0		Proposed 4th Floor Plan (Part –A)

MOOR_V2.0_4014 B V2.0		Proposed 4th Floor Plan (Part –B)

MOOR_V2.0_4015 A V2.0		Proposed Roof Plan (Part –A)

MOOR_V2.0_4015 B V2.0		Proposed Roof Plan (Part –B)

MOOR_V2.0_4016 A V2.0		Proposed West Elevation

MOOR_V2.0_4016 B V2.0		Proposed South Elevation

MOOR_V2.0_4017 A V2.0		Proposed East Elevation

MOOR_V2.0_4017 B V2.0		Proposed North Elevation

MOOR_V2.0_4018 A V2.0		Proposed Section CC

MOOR_V2.0_4018 B V2.0		Proposed Section AA

MOOR_V2.0_4019 V2.0		Existing & Proposed Street Scene Elevations

MOOR_V2.0_4020 A V2.0		Proposed CCTV Locations Plan (Part –A)

MOOR_V2.0_4020 B V2.0		Proposed CCTV Locations Plan (Part –B)

APPLICATION B - 17/0015/OUT

MOOR_V2.0_5001 V2.0		Location Plan

MOOR_V2.0_5002 V2.0		Topographical Survey Plan

MOOR_V2.0_5003 V2.0		Existing Site Plan

MOOR_V2.0_5004 V2.0		Proposed Site Plan

MOOR_V2.0	_5005 V2.0		Block Plan

MOOR_V2.0_5006 			Access Plan

MOOR_V2.0_5007 A V2.0		Existing Unit 3 Floor Plans

MOOR_V2.0_5007 B V2.0		Existing Unit 3 Elevations

MOOR_V2.0_5008 V2.0		Existing Unit 4 Floor Plans & Elevations

MOOR_V2.0	_5009 A V2.0		Proposed Basement Level -2 Plan (Part –A)

MOOR_V2.0	_5009 B V2.0		Proposed Basement Level -2 Plan (Part –B)

MOOR_V2.0_5010 A V2.0		Proposed Basement Level -1 Plan (Part –A)

MOOR_V2.0_5010 B V2.0		Proposed Basement Level -1 Plan (Part –B)

MOOR_V2.0_5011 A V2.0		Proposed Ground Floor Plan (Part –A)

MOOR_V2.0_5011 B V2.0		Proposed Ground Floor Plan (Part –B)

MOOR_V2.0_5012 A V2.0		Proposed First Floor Plan (Part –A)

MOOR_V2.0_5012 B V2.0		Proposed First Floor Plan (Part –B)

MOOR_V2.0_5013 A V2.0		Proposed 2nd – 3rd Floor Plan (Part –A)

MOOR_V2.0_5013 B V2.0		Proposed 2nd – 3rd Floor Plan (Part –B)

MOOR_V2.0_5014 A V2.0		Proposed 4th Floor Plan (Part –A)

MOOR_V2.0_5014 B V2.0		Proposed 4th Floor Plan (Part –B)

MOOR_V2.0_5015 A V2.0		Proposed Roof Plan (Part –A)

MOOR_V2.0_5015 B V2.0		Proposed Roof Plan (Part –B)

MOOR_V2.0_5016 A V2.0		Proposed West Elevation

MOOR_V2.0_5016 B V2.0		Proposed South Elevation

MOOR_V2.0_5017 A V2.0		Proposed East Elevation

MOOR_V2.0_5017 B V2.0		Proposed North Elevation

MOOR_V2.0_5018 A V2.0		Proposed Section C-C

MOOR_V2.0_5018 B V2.0		Proposed Section A-A

MOOR_V2.0_5019 V2.0		Existing & Proposed Street Scene Elevations

APPLICATION C - 16/2735/OUT

MOOR_V2.0	_6001 V2.0		Location Plan

MOOR_V2.0_6002	V2.0		Topographical Survey Plan

MOOR_V2.0_6003 V2.0		Existing Site Plan

MOOR_V2.0_6004 V2.0		Proposed Site Plan

MOOR_V2.0_6005 V2.0		Block Plan

MOOR_V2.0_6006			Access Plan

MOOR_V2.0_6007 A V2.0		Existing Unit 3 Floor Plans

MOOR_V2.0_6007 B V2.0		Existing Unit 3 Elevations

MOOR_V2.0_6008 V2.0		Existing Unit 4 Floor Plans & Elevations

MOOR_V2.0_6009 A Rev A	Proposed Basement Level -2 Plan (Part –A)

MOOR_V2.0_6009 B Rev A	Proposed Basement Level -2 Plan (Part –B)

MOOR_V2.0_6010 A V2.0		Proposed Basement Level -1 Plan (Part –A)

MOOR_V2.0_6010 B V2.0		Proposed Basement Level -1 Plan (Part –B)

MOOR_V2.0_6011 A V2.0		Proposed Ground Floor Plan (Part –A)

MOOR_V2.0_6011 B V2.0		Proposed Ground Floor Plan (Part –B)

MOOR_V2.0_6012 A V2.0		Proposed First Floor Plan (Part –A)

MOOR_V2.0_6012 B V2.0		Proposed First Floor Plan (Part –B)

MOOR_V2.0_6013 A V2.0		Proposed 2nd – 3rd Floor Plan (Part –A)

MOOR_V2.0_6013 B V2.0		Proposed 2nd – 3rd Floor Plan (Part –B)

MOOR_V2.0_6014 A V2.0		Proposed 4th Floor Plan (Part –A)

MOOR_V2.0_6014 B V2.0		Proposed 4th Floor Plan (Part –B)

MOOR_V2.0_6015 A V2.0		Proposed 5th Floor Plan (Part –A)

MOOR_V2.0_6015 B V2.0		Proposed 5th Floor Plan (Part –B)

MOOR_V2.0_6016 A V2.0		Proposed Roof Plan (Part –A)

MOOR_V2.0_6016 B V2.0		Proposed Roof Plan (Part –B)

MOOR_V2.0_6017 A V2.0		Proposed East Elevation

MOOR_V2.0_6017 B V2.0		Proposed North Elevation

MOOR_V2.0_6018 A V2.0		Proposed West Elevation

MOOR_V2.0_6018 B V2.0		Proposed South Elevation

MOOR_V2.0_6019 A V2.0		Proposed Section CC

MOOR_V2.0_6019 B V2.0		Proposed Section AA

MOOR_V2.0_6020 V2.0		Existing & Proposed Street Scene Elevations

APPLICATION D - 16/2709/OUT

MOOR_V2.0_7001 V2.0		Location Plan

MOOR_V2.0	_7002 V2.0		Topographical Survey Plan

MOOR_V2.0_7003 V2.0		Existing Site Plan

MOOR_V2.0_7004 V2.0		Proposed Site Plan

MOOR_V2.0_7005 V2.0		Block Plan

MOOR_V2.0_7006 			Access Plan

MOOR_V2.0_7007 A V2.0		Existing Unit 3 Floor Plans

MOOR_V2.0_7007 B V2.0		Existing Unit 3 Elevations

MOOR_V2.0_7008 V2.0		Existing Unit 4 Floor Plans & Elevations

MOOR_V2.0_7009 A V2.0		Proposed Basement Level -2 Plan (Part –A)

MOOR_V2.0_7009 B V2.0		Proposed Basement Level -2 Plan (Part –B)

MOOR_V2.0_7010 A V2.0		Proposed Basement Level -1 Plan (Part –A)

MOOR_V2.0_7010 B V2.0		Proposed Basement Level -1 Plan (Part –B)

MOOR_V2.0_7011 A V2.0		Proposed Ground Floor Plan (Part –A)

MOOR_V2.0_7011 B V2.0		Proposed Ground Floor Plan (Part –B)

MOOR_V2.0_7012 A V2.0		Proposed First Floor Plan (Part –A)

MOOR_V2.0_7012 B V2.0		Proposed First Floor Plan (Part –B)

MOOR_V2.0_7-13 A V2.0		Proposed 2nd – 5th Floor Plan (Part –A)

MOOR_V2.0_7-13 B V2.0		Proposed 2nd – 5th Floor Plan (Part –B)

MOOR_V2.0_7014 A V2.0		Proposed 6th Floor Plan (Part –A)

MOOR_V2.0_7014 B V2.0		Proposed 6th Floor Plan (Part –B)

MOOR_V2.0_7015 A V2.0		Proposed Roof Plan (Part –A)

MOOR_V2.0_7015 B V2.0		Proposed Roof Plan (Part –B)

MOOR_V2.0_7016 A V2.0		Proposed West Elevation

MOOR_V2.0_7016 B V2.0		Proposed South Elevation

MOOR_V2.0_7017 A V2.0		Proposed East Elevation

MOOR_V2.0_7017 B V2.0		Proposed North Elevation

MOOR_V2.0_7018 A V2.0		Proposed Section CC

MOOR_V2.0_7018 B V2.0		Proposed Section AA

MOOR_V2.0_7019 V2.0		Existing & Proposed Street Scene Elevations

MOOR_V2.0_7020 A V2.0		Proposed CCTV Locations Plan (Part –A)

MOOR_V2.0_7020 B V2.0		Proposed CCTV Locations Plan (Part –B)

APPENDIX 2 – CONSULTEE COMMENTS

Watford Borough Council: No comments received.

LDF Transport: No comments received.

Biodiversity Projects Officer: [Advisory comments for all applications]

The Development Plan and the Development Management Policies Local Development Document for Three Rivers District Council contain a number of policies which are relevant to wildlife and biodiversity and its conservation and enhancement and are, therefore, relevant to this application.  In general the policies aim to minimise the impacts of development on the environment whilst striving to conserve and enhance the species and habitats present and having regard to the local context of the area - see the appendices for more policy detail.

Croxley Common Moor has been designated as Site of Special Scientific Interest (SSSI) and a Local Nature Reserve (LNR) which reflects the high value it has to both wildlife and people.  SSSIs are designated by Natural England and represent the country’s very best wildlife whilst the LNR designation is made by the Local Authority due to the value of the site to wildlife, education and public enjoyment.

The northern boundary of the proposed development is only around 79m away from the southern boundary of Croxley Common Moor SSSI and LNR.

The Lighting Report refers to a lit route across Croxley Common Moor SSSI and LNR whilst other submitted documents state that the proposed lighting across Croxley Common Moor SSSI and LNR no longer forms part of the proposed development.  For the avoidance of doubt it is not acceptable to install lighting across the site or promote the route for access to Croxley Tube Station and beyond.

All footpaths are informal, with no formal surfacing.  The paths are not level and the surface is simply that of Croxley Common Moor SSSI and LNR itself – principally grass which has been worn away over the years.  As such this route would not be accessible to all, especially those with pushchairs or restricted mobility.

Due to the nature of Croxley Common Moor SSSI and LNR the area is open and relatively secluded.  This would not be a safe route to direct people after dark.

Furthermore the byelaws state that bicycles are not permitted to be ridden on Croxley Common Moor SSSI and LNR.  As such this would limit access only to those on foot.

The Design and Access Statement states that occupants of the proposed development would not use Croxley Common Moor SSSI and LNR as an access route to a tube station showing that they expect people to walk/cycle, for example, 1.8 miles to Croxley Tube Station.  However, the route to Croxley Tube Station directly across Croxley Common Moor SSSI and LNR is less than 1 mile.  It is not realistic to say that people will not use this route.  Furthermore Croxley Tube Station, is by foot, the closest station as the Design and Access Statement gives the following walking/cycling distances to tube stations:

Watford Station:		1.7 miles

Cassiobridge Station:		1.2 miles

Moor Park Station:		3.1 miles

Vicarage Road Station:	1.6 miles

Rickmansworth Station:	2.5 miles

As such it is not unreasonable to assume that people will choose to use Croxley Common Moor SSSI and LNR and the shortest route to the station.

There is no lighting design for the proposed development.  Lighting is of concern due to the wildlife value of Croxley Common Moor SSSI and LNR.  In particular there is potential for these buildings, especially the residential building, which includes a lit roof top garden, to be emitting light for many hours after dark.  The development faces Croxley Common Moor SSSI and LNR and although it is separated from the site by a building and Dwight Road light has the potential to reach Croxley Common Moor which is only around 79m away from the northern boundary of the proposed development.  This could be detrimental to a number of nocturnal species such as moths, bats, owls and badgers which will be active within the vicinity of the proposed development.  Of particular concern is the potential impact on the Slate Sober Moth - Croxley Common Moor SSSI and LNR is the ONLY location for the Slate Sober Moth in the UK.  The Slate Sober Moth is a Section 41 Species of Principal Importance as designated under Section 40 of the Natural Environment Rural Communities (NERC) Act 2006.  Section 40 of the NERC Act 2006 states that every public authority must, in executing its functions, have regard, as far as is consistent with the proper exercise of those functions, to the purpose of conserving biodiversity.  This one remaining UK population of the Slate Sober Moth on Croxley Common Moor SSSI and LNR is now perilously close to extinction and any form of lighting would be highly detrimental to the conservation of this night flying insect.  The moths Scythris potentillella and Coleophora genistae can also be found on Croxley Common Moor SSSI and LNR and are both rare species.  The former is classed as Nationally Rare as it occurs in less than 15 hectads (10km squares) in the UK.  The latter species is a Nationally Scarce species that occurs in 15 – 100 hectads in the UK, with Croxley Common Moor SSSI and LNR being the only site in Hertfordshire that it has been found.  As such any light spill would be harmful to these rare nocturnal invertebrates.

Although light does already reach Croxley Common Moor from existing buildings the additional height of the proposed development, which would be taller than any of the existing buildings and include a roof garden, would mean that light spill from the proposed development could further impact on Croxley Common Moor SSSI and LNR.  Also the fenced area on Croxley Common Moor is where the Slate Sober Moth is currently known to exist.  It is quite possible that the moth is in other areas of Croxley Common Moor which have yet to be fully surveyed.

I appreciate that this is an outline planning application but without a lighting design it is not possible to comment further on this aspect.

The Preliminary Ecological Appraisal states that two of the potential ecological impacts are:

- Increased recreational pressure on Croxley Common Moor SSSI/LNR

- Small scale losses of common and widespread habitats typical of the amenity setting

Neither of these potential impacts would be satisfactory.

The Landscape and Visual Impact Statement acknowledges that the Landscape Sensitivities associated with Croxley Common Moor are either Medium or High, for example:

“Level river valley between 0.6 and 1km wide: This land includes Registered Common Land of notable ecological value therefore its value is judged to be Medium.”

“Areas of remnant and restored pasture: This land includes Registered Common Land and culturally valued historic semi-natural pastoral fields, therefore its value is judged to be High.”

“Semi-natural habitats: Many of these habitats occur on Registered Common Land and they are recognised local characteristics, therefore their value is judged to be Medium.”

“Recreational opportunities associated with waterways, rights of way and wetlands: This land includes Registered Common Land and historic semi-natural pastoral fields whose access is valued by users, therefore this characteristic is judged to be of High value.”

Overall, with respect to Croxley Common Moor SSSI and LNR, the Landscape and Visual Impact Assessment concludes that it is “appropriate to attribute High sensitivity” to change of this receptor site with respect to the proposed development.  The Visual Assessment of the Landscape and Visual Impact Assessment also states that the overall impact upon Croxley Common Moor is judged to be an adverse impact of Major/Moderate significance.

In general the addition of 274 residential units, office buildings and the associated facilities in such close proximity to Croxley Common Moor SSSI and LNR will be increased use of the site, whether this be for commuting use or recreational purposes.  Croxley Common Moor SSSI and LNR is already a popular site for a number of recreational uses such as dog walking, walking, jogging and family outings.  There are no surfaced footpaths and if the proposed development were to go ahead then the consequences to this sensitive site would be an increase in the trampling of vegetation - it is the rare assemblages of plant species that can be found at the site which are the very reason as to why the Moor was designated as a SSSI.

Hertfordshire Highways:

APPLICATION A: [No objection, conditions requested]

Notice is given under article 18 of the Town and Country Planning (Development Management Procedure) (England) Order 2015 that the Hertfordshire County Council as Highway Authority does not wish to restrict the grant of permission subject to the following conditions:

Decision

Hertfordshire County Council as Highway Authority does not wish to raise an objection to the proposed development, subject to suitable conditions.

Conditions

Condition 1: Cinema and Training and Education Land Uses

The land uses ‘Leisure – Cinema’ and ‘Training and Education’ are to be for the sole use of residential and office users, as stated in submitted Transport Assessment application document by Nichols Consulting. Should the Cinema and Training and Education land uses be open to the public, a Transport Assessment will be required to demonstrate the impact of the added traffic on the local highway network.

Reason: In order to protect the highway safety and amenity of other users of the public highway and rights of way.

Condition 2: Construction Traffic Management Plan

Construction of the development hereby approved shall not commence until a Construction Traffic Management Plan has been submitted to and approved in writing by the Local Planning Authority.  Thereafter, the construction of the development shall only be carried out in accordance with the approved Plan. The Construction Traffic Management Plan shall include details of:

a. Demolition, excavation and construction vehicle numbers, type, routing by phase (Demolition, Excavation and Construction);

b. Traffic management requirements by phase (Demolition, Excavation and Construction) [Linked to Condition 3];

c. Construction and storage compounds (including areas designated for car parking);

d. Siting and details of wheel washing facilities;

e. Cleaning of site entrances, site tracks and the adjacent public highway;

f. Provision of sufficient on-site parking prior to commencement of construction activities;

g. Post construction restoration/reinstatement of the working areas and temporary access to the public highway.

Reason: In order to protect highway safety and the amenity of other users of the public highway and rights of way.

Condition 3: Construction Traffic - Transport Assessment

Construction of the development hereby approved shall not commence until a Transport Assessment has been submitted to and approved in writing by the Local Planning Authority. The purpose of the Transport Assessment is to ensure that the impact of the construction activity at the site will not have detrimental impact on the safety and operation of the local highway network. The Transport Assessment will assist to establish the necessity for and type of required Traffic Management.

Reason: In order to protect highway safety and the amenity of other users of the public highway and rights of way.

Condition 4: Travel Plan

At least two months prior to first occupation, a Full Travel Plan for the residential element of the development and Travel Plan Statements for the office and gym elements of the development, must be submitted to and approved in writing by the Local Planning Authority and should be prepared in accordance with Hertfordshire’s Travel Plan Guidance. Implementation of the plans shall follow a timescale to be agreed by the Local Planning Authority.

Reason: To promote sustainable transport measures to the development.

Condition 5: Detailed Plans

Prior to the commencement of the development hereby permitted full details in the form of scaled plans and written specifications shall be submitted to and approved in writing by the Local Planning Authority to illustrate the following:

i.  	Roads, footways, foul and on-site water drainage.

ii.	Existing and proposed access arrangements including visibility splays.

iii.  	Parking provision in accordance with adopted standard.

iv.  	Cycle parking provision in accordance with adopted standard.

v.  	Servicing areas, loading areas and turning areas for all vehicles.

Condition 6: Swept Path Assessments

Prior to commencement of the development, the applicant shall submit swept path assessments for the following parts of the proposed development. There are to be submitted to and approved in writing by the Local Planning Authority:

-  Refuse and servicing bays to demonstrate that the largest vehicles accessing the bays can safely access the bays to depart the site in a forward gear;

-  Car parking internal layout to demonstrate that a vehicle can enter, manoeuver through and depart in a forward gear; and,

-  Car parking spaces to demonstrate that all car parking spaces can be safely accessing by a large car.

Reason: In the interests of maintaining safe access to the proposed development by all users.

Condition 7: Delivery and Servicing Plan

Prior to commencement of the development, the applicant shall submit a Delivery and Servicing Plan to be submitted to and approved in writing by the Local Planning Authority. The Delivery and Servicing Plan shall contain the following:

-  The delivery and servicing requirements (including refuse collection) for the proposed uses;

-  A scheme for coordinating deliveries and servicing for the proposed development;

-  Areas within the development site that will be used for loading and manoeuvring of delivery and servicing vehicles; and,

-  Access to / from the site for delivery and servicing vehicles.

Reason: In the interests of maintaining highway efficiency and safety.

Condition 8: Road Safety Audit

Before commencement of any part of the development, a Stage 1 Road Safety is required for all access arrangements to be submitted to and approved in writing by the Local Planning Authority.

Reason: To ensure that the proposed access arrangements are safe and suitable for their intended use.

HIGHWAY INFORMATIVES:

HCC recommend inclusion of the following Advisory Notes (ANs) to ensure that any works as part of this development are carried out in accordance with the provisions of the Highways Act 1980 and other relevant processes.

AN1) Storage of materials: The applicant is advised that the storage of materials associated with the construction of this development should be provided within the site on land which is not public highway, and the use of such areas must not interfere with the public highway. If this is not possible, authorisation should be sought from the Highway Authority before construction works commence. Further information is available via the website http://www.hertsdirect.org/services/transtreets/highways/ or by telephoning 0300 1234047.

AN2) Obstruction of public highway land: It is an offence under section 137 of the Highways Act 1980 for any person, without lawful authority or excuse, in any way to wilfully obstruct the free passage along a highway or public right of way. If this development is likely to result in the public highway or public right of way network becoming routinely blocked (fully or partly) the applicant must contact the Highway Authority to obtain their permission and requirements before construction works commence. Further information is available via the website http://www.hertsdirect.org/services/transtreets/highways/ or by telephoning 0300 1234047.

AN3) Road Deposits: It is an offence under section 148 of the Highways Act 1980 to deposit mud or other debris on the public highway, and section 149 of the same Act gives the Highway Authority powers to remove such material at the expense of the party responsible.  Therefore, best practical means shall be taken at all times to ensure that all vehicles leaving the site during construction of the development are in a condition such as not to emit dust or deposit mud, slurry or other debris on the highway. Further information is available via the website http://www.hertsdirect.org/services/transtreets/highways/ or by telephoning 0300 1234047.

AN4) Construction standards for works within the highway: All works to be undertaken on the adjoining highway shall be constructed to the satisfaction and specification of the Highway Authority, by an approved contractor, and in accordance with Hertfordshire County Council’s publication "Roads in Hertfordshire - Highway Design Guide (2011)". Before works commence the applicant will need to apply to the Highway Authority to obtain their permission and requirements. Further information is available via the website http://www.hertsdirect.org/services/transtreets/highways/ or by telephoning 0300 1234047.

S278 Agreement

Any works within the highway boundary (including alterations to the footway and the proposed site access) will need to be secured and approved via a S278 Agreement with the HCC.

S106 Agreement

A S106 agreement will be required to secure a Construction and Logistics Plan Planning Obligation and support for the Travel Plan.

The S106 agreement should include support of the proposed bus service and a contribution towards the provision of a foot and cycle way along the northern verge of Tolpits Lane linking the site with the Moor Park estate and the underground station within it.

Community Infrastructure Levy (CIL)

The application site is located within Area C where the TRDC CIL charge is nil.  As such no CIL contribution would be required in relation to the proposed development.

Description of the Proposal

The proposal is for the demolition of two 3-storey offices with underground parking to be replaced by a single 5 storey building with underground parking for 661 cars.

The proposed development includes a 5-storey mixed use building. The building will comprise:

-  274 flats: 26 studio, 134 1-bed; 106 2-bed; and, 8 3-bed;

-  Office: 2500sqm GFA;

-  Café: 250sqm GFA;

-  Food Retail: 300sqm GFA;

-  Leisure (Gym, swimming, etc.): 700sqm GFA;

-  Leisure (Cinema): 300sqm GFA;

-  Training and Education: 300sqm; and,

-  Underground parking with provisions for 661 cars, including 35 disabled spaces and 20 electric car spaces.

Site Description

The development site is located in the Wolsey Business Park at the northwest corner of the eastern junction of Tolpits Lane (A4145) and Dwight Road in Watford. At present, the site is occupied by 2 3-storey buildings. Unit 3, Symbio Point is currently used as a secondary school with a temporary planning permission and Unit 4, York House, is used as office space.

The site is bordered to the east and west by Dwight Road, to the south by Tolpits Lane (A4145) and to the north by the private site access road. Dwight Road is an unclassified local access road with a 30mph speed limit and Tolpits Lane is a principal A-class main distributor road with a 40mph speed limit.

Analysis

As part of the planning application package, the applicant has provided a Transport Assessment (TA) to provide evidence to demonstrate the likely impact of the proposed development on the local highway network. This is as required in Roads in Hertfordshire (section 1 chapter 7).

A Design and Access Statement (DAS) is required for all planning applications that have an impact on the highway, as outlined in Roads in Hertfordshire: Design Guide (section 1 chapter 7). A DAS has been provided for the proposed development and is considered appropriate for the purposes of this planning application.

Policy Review

As part of the Transport Assessment provided for consideration, the applicant has provided evidence of review of the following policy documents:

-  National Planning Policy Framework (2012)

-  National Planning Practice Guidance

-  Hertfordshire County Council (HCC) highway design guide Roads in Hertfordshire 3rd edition

-  Relevant Three Rivers District Council core strategies relating to transport and parking requirements

-  Hertfordshire County Council (HCC) Local Transport Plan 3: 2011-2031

Trip Generation and Distribution

The applicant has provided a Transport Assessment (TA) with the application package for consideration.

Trip Generation

A trip generation profile for the existing site and the proposed development are provided as part of the TA.

Existing Situation

The TA provides existing trip generation profiles considering previously agreed trip rates submitted as part of planning applications with reference numbers: 15/1281/PDR and 15/1184/PDR. This is considered acceptable.

The TA outlines the existing permitted trip generation profiles for each of the buildings on sites, Units 3 and 4 Wolsey Business Park. The lowest approved trip generation profile for each site was used to establish an existing trip generation profile for the total site. This approach is considered acceptable as it presents a worst case scenario when compared with the proposed development.

Unit 3 has 3 permitted uses: Office, Residential and Reach Free School (temporary). The trip profile for each site was provided in the TA and the lowest trip profile was for the permitted residential scheme and is as follows:

-  AM Peak: 6 arrivals, 22 departures giving 28 two-way movements

-  PM Peak: 19 arrivals, 8 departures giving 27 two-way movements

Unit 4 has 2 permitted uses: Office and Residential. The trip profile for each site was provided in the TA and the lowest trip profile was for the permitted residential scheme and is as follows:

-  AM Peak: 3 arrivals, 3 departures giving 6 two-way movements

-  PM Peak: 12 arrivals, 8 departures giving 20 two-way movements

Therefore, the total trip generation profile for the existing land uses for the whole site is as follows:

-  AM Peak: 9 arrivals, 25 departures giving 34 two-way movements

-  PM Peak: 31 arrivals, 16 departures giving 47 two-way movements

Proposed Land Uses

The trip rates of the proposed development were generated by interrogating TRICS database. This approach is considered acceptable.

The TA used the same TRICS outputs provided as part of the original application submission for application reference 15/1935/FUL. These TRICS outputs were not disputed and are therefore considered acceptable.

The following parameters were used for the TRICS interrogation for all land uses:

-  England only, excluding Greater London;

-  Excluding weekends;

-  Average trip rates adopted; and,

-  Peak Hours 08:00 - 09:00 and 17:00 - 18:00.

The following parameter was used to establish the residential trip generation profile: Residential land use 03 - Residential - C Flats Privately Owned.

The following parameter was used for the TRICS interrogation to establish the office trip generation profile: Employment land use 02 - Employment - A Office.

The following parameter was used for the TRICS interrogation to establish the café trip generation profile: Café land use 03 - Hotel, Food and Drink - E Roadside food.

The following parameter was used for the TRICS interrogation to establish the retail trip generation profile: Retail land use 01 - Retail - O Convenience Store.

The following parameter was used for the TRICS interrogation to establish the gym trip generation profile: Leisure - Gym land use 07 - Leisure - K Fitness Club.

The following trips rates are associated with the above TRICS interrogation outputs:

-  Residential:

-  AM Peak: 0.068 in / 0.239 out

-  PM Peak: 0.220 in / 0.096 out

-  Office:

-  AM Peak: 1.232 in / 0.236 out

-  PM Peak: 0.173 in / 1.027 out

-  Café:

-  AM Peak: 3.057 in / 3.057 out

-  PM Peak: 3.806 in / 3.922 out

-  Retail:

-  AM Peak: 7.424 in / 7.348 out

-  PM Peak: 7.576 in / 7.652 out

-  Gym:

-  AM Peak: 0.951 in / 1.302 out

-  PM Peak: 3.040 in / 1.661 out

The applicant does not provide trip generation analysis for the Leisure - Cinema and Training & Education land uses due to their sole use by the proposed development’s residents. This is considered acceptable, subject to a suitable condition to guarantee that the proposed land uses will remain for the sole use of users of the office and residential land uses.

Using the predicted rates and proposed development areas the associated trips generated by the proposed development at this location would therefore be:

Residential

-  AM Peak: 19 arrivals, 66 departures giving 85 two-way

-  PM Peak: 61 arrivals, 27 departures giving 88 two-way

Office

-  AM Peak: 31 arrivals, 6 departures giving 37 two-way

-  PM Peak: 5 arrivals, 26 departures giving 31 two-way

Café

-  AM Peak: 8 arrivals, 8 departures giving 16 two-way

-  PM Peak: 10 arrivals, 10 departures giving 20 two-way

Food Retail

-  AM Peak: 23 arrivals, 23 departures giving 46 two-way

-  PM Peak: 23 arrivals, 23 departures giving 46 two-way

Leisure (Gym)

-  AM Peak: 7 arrivals, 10 departures giving 17 two-way

-  PM Peak: 22 arrivals, 12 departures giving 34 two-way

The total proposed trips for the proposed development are as follows:

-  AM Peak: 88 arrivals, 113 departures giving 201 two-way

-  PM Peak: 121 arrivals, 98 departures giving 219 two-way

The total proposed trip generation profile was compared to the existing trip generation profile to establish the net impact at the site. The net trip generation profile for the site is as follows:

-  AM Peak: 79 arrivals, 88 departures giving 167 two-way movements

-  PM Peak: 90 arrivals, 82 departures giving 172 two-way movements

Multi-modal Trip Generation

In addition to the vehicular trip generation profile, the applicant has provided a multimodal trip generation profile for Method of Travel to Work based on 2011 census data for the Moor Park and Eastbury ward. The multi-modal trip generation profile is provided to present the likely methods of travel to / from the site for work purposes. This approach is considered appropriate. The vehicular trips estimated as part of the vehicular trip generation profile for the proposed development were used to extrapolate the number of users by mode. The multi-modal trip generation profile shows a demand for public transport and footway access. Therefore, contributions are being sought to improve connections to the site on foot and cycle and by public transport.

Trip Distribution

Trip distribution assumptions for the proposed development traffic have been included as part of the TA and trip distributions proportions are derived from observed and synthesised turning movements at the Dwight Road / Tolpits Lane roundabout and the priority junction of Dwight Road with the site access road. This is considered an acceptable approach.

Impact on the Highway

Junction Assessment

Junction Modelling

The applicant has provided junction capacity assessments for the priority junction access road with Dwight Road and the Tolpits Lane/ Dwight Road roundabout junction. As requested by HCC in discussions following the original application submission (application reference 15/1935/FUL) the TA includes standalone junction models for each junction and junction modelling where the two junctions are connected. The junction modelling was carried out using industry-standard Junctions 9 software.

It was agreed as part of discussions following the original application that any resubmission would include an assessment of the opening year plus a 5 year horizon year. As part of the TA for this application, the year of opening would be 2020 and the proposed 5-year horizon year 2025. This is considered acceptable for the proposed assessment.

The applicant has provided growth rates for the horizon years using industry-standard TEMPRO database. This is considered an appropriate method.

The scenarios modelled as part of the assessment include:

-  Do Nothing 2020: Base traffic 2020;

-  Do Nothing 2025: Base traffic 2025;

-  Do Something 2020: Base traffic 2020 + development traffic; and,

- Do Something 2025: Base traffic 2025 + development.

The TA summarises the results of both the unlinked and linked junction assessments. The junction model outputs are presented in an appendix of the TA.

The operational capacity of the unlinked junctions was considered using the RFC (Ratio of Flow to Capacity), queuing and delay for consistency with other planning applications; however, Level of Service (LOS) was also considered for the unlinked junction operational capacity results because the linked junction operational capacity assessments cannot produce a RFC value for comparison. This is explained in the summary in the TA for the linked junction assessment which states that as the modelling for the linked junctions uses the lane simulation mode, the results do not specify a RFC value for the junctions. Therefore, the operational capacity of the junctions was considered using the Level of Service (LOS), queuing and delay. Therefore, the LOS is considered for both of the junction capacity assessment scenarios. This is considered acceptable for the purposes of the TA.

Typically the Level of Service (LOS) of a junction in the range A to D is considered to be acceptable, with LOS A/B most desirable and LOS C/D acceptable. A LOS of E would imply that the operation of the junction is beginning to fail and likely to be operating with congestion and delays. A LOS of F implies that the junction has reached or is over its capacity and is likely to be experiencing severe delays and congestion. The thresholds A-F are based on the queuing delay on each arm. The definitions can be summarised as follows:

A = Free flow

B = Reasonably free flow

C = Stable flow

D = Approaching unstable flow

E = Unstable flow

F = Forced or breakdown flow

Please note that the PCU (Passenger Car Unit) is the equivalent of 1 passenger car or 5.75m in length.

Unlinked Junction Capacity Assessments

The junction capacity results for the unconnected Tolpits Lane roundabout with Dwight Road demonstrate that in both the AM and PM peak periods in both horizon years and for the Do-Nothing and Do-Something scenarios, the junction will continue to operate within capacity. The junction is likely to experience a maximum RFC of 0.63 with associated LOS of A, a queue of 1.7 PCUs and a predicted total junction arm delay of 6.35 seconds. This will occur during the 2025 AM peak, Do Something scenario.

The junction capacity results for the unconnected Dwight Road priority T-junction with the site access road demonstrate that in both the AM and PM peak periods in both horizon years and for the Do-Nothing and Do-Something scenarios, the junction is likely to continue to operate within capacity. The junction would experience a maximum RFC of 0.58 with associated LOS of C, a queue of 1.3 PCUs and a predicted total junction arm delay of 17.66 seconds. This would occur during the 2025 PM peak, Do Something scenario.

Linked Junction Capacity Assessments

The LOS for the Tolpits Lane roundabout is predicted to be A for all junction arms in the AM and PM peak hours. Maximum queues experienced at this junction occur in the AM peak for Tolpits Lane East arm with 1.7 PCUs for the 2025 horizon year and a maximum predicted total junction arm delay of 6.25 seconds.

The LOS for the Dwight Road and site access road priority T-junction is predicted to be A for all junction arms for the 2020 and 2025 horizon years in the AM and PM peak hours for the Do-Nothing scenario apart from the site access road where the LOS is B for both 2020 and 2025 scenarios. The junction has a LOS of A for both the Dwight Road junction arms for the Do-Something scenario in both the AM and PM peaks for both horizon years. The site access junction arm was predicted to operate with a LOS of B for Do-Nothing scenario in the 2020 and 2025 horizon years for the PM peak and with a LOS of C for the 2020 PM peak and C for the 2025 PM peak. However, queuing at this junction would not exceed 1.5 PCUs with an associated maximum predicted total junction arm delay of 17.63 seconds.

Summary

The results indicate that the junctions are likely to operate within desired thresholds with LOS better than D, acceptable levels of queuing and delays for all public highway links.

The junction capacity test results indicate that the additional traffic added to the network as a consequence of the proposed development is not likely to have a detrimental impact on the operation of the highway network in the vicinity of the proposed development.

Highway Safety

The applicant has provided detailed collision data as part of the TA for the surrounding road network. This approach is considered acceptable. A review of the 5 years of collision data from 1st April 2011 to 31st March 2016, obtained from HCC reveals that there were 26 collisions resulting in injury in the vicinity of the site, all of which are designated slight collisions. There have been a total of 41 personal injuries resulting from these collisions.

There were 5 locations identified as part of the review where more than one collision occurred, as provided below:

-  A4145 Tolpits Lane, North of Croxley View junction: 3 PICs all human error, failing to look, reckless driving, following too close and loss of control.

-  A4145 Tolpits Lane/Croxley View: 7 PICs, 3 involved pedestrians. 5 collisions involved parking manoeuvres outside shops in the vicinity of the junction. Contributing factors include failing to look properly, reckless driving and wet road surface.

-  A4145 Tolpits Lane/ Chaffinch Lane: 2 PICs. One due to loss of control on wet surface and the other a consequence of driver travelling too close to a stopping vehicle.

-  A4145 Tolpits Lane, west of Dwight Road: 5 PICs, 1 injury to cyclist, 1 to motorcyclist and rest to car users. 4 PICs involved turning manoeuvres at entry to or exit from access roads from Tolpits Lane and were caused by misjudgement of speed or failure to look. 1 PIC involved vehicle overtaking cyclist.

-  A4145 Moor Lane/Sandy Lodge Road: 3 PICs. Contributing factors, failure to look properly, excessive speed, loss of control and slippery road surface.

The review of the collision data in the vicinity of the proposed development site does not present any safety issues with the highway and all collisions appear to have human judgement errors as contributing factors. Therefore, the proposed development is not expected to impact significantly on the safe operation of the local highway.

Refuse and Service Delivery

The applicant has stated in the TA that there will be 3 servicing areas for servicing, delivery and collection for the development site. Locations of the refuse store and collection have not been provided as part of the servicing and delivery arrangements. This is required to demonstrate that refuse collection will be in line with standards for refuse collection. A Refuse, Servicing and Delivery Plan is required to ensure that suitable vehicles would be able to access the appropriate bays, etc.  It is stated in the TA that vehicle swept path assessments within the site at ground and basement levels will be submitted under reserved matters. Due to the nature of the application as outline, this is considered acceptable.

Highway Layout

Vehicle Access

The proposed development proposes to utilise the existing site access road from Dwight Road. The proposed development includes new accesses to the different blocks and the underground car park of the proposed development.

The internal layout of the basement level appears to operate under a one-way system. However, the layout at this stage is not finalised and would be agreed as part of reserved matters. Exact dimensions of the parking bays as well as the spacing between them are required to be provided by the applicant at the reserved matters stage. In addition, swept path assessments would be required to demonstrate that cars could enter, manoeuvre through and depart the site in a forward gear.

The HCC DM Implementation team were consulted for the same access arrangements proposed for a full planning application, reference number: 16/2497/FUL, on the same site. As the site access road is not adopted by HCC and is not likely to be so they had no formal comments to make; however, there is concern related to the stop and reverse requirement for an HGV accessing the servicing areas for the proposed development. Therefore, evidence should be provided under reserved matters to support that the largest servicing vehicles can access the site safely and will not seek to park on Dwight Road to carry out any refuse collection, servicing or delivery at the site.

Pedestrian Access

It is proposed that the development would utilise the existing site access road and existing pedestrian facilities. It is unclear from the drawings if pedestrian facilities would be provided along the site frontage, adjacent to the site access road. Provisions for safe pedestrian access to the site are required. Additional details with regards to pedestrian access and footway provisions would be required at the reserved matters stage.

Swept Path Analysis

The applicant has not provided swept path assessments for the servicing areas, or access to the car park and car parking spaces. However, as this is an outline application this is considered necessary and swept path assessments would be required at the reserved matters stage.

Road Safety Audit

A Stage 1 Road Safety Audit would be required to illustrate that the new access arrangements are safe and appropriate for the size of the development. As outlined in the meeting minutes provided in the appendices from a meeting with HCC, the transport consultant and the developer, ‘the execution of a Stage 1 Road Safety Audit as part of the S38/S278 process will be stated in the TA for the resubmission’. The applicant states that a Stage 1 road safety audit for the site access would be conducted as under a Section 38 agreement or a Section 278 agreement. This is considered appropriate.

Parking

The applicant has stated that the proposed development would provide 661 car parking spaces, including 35 disabled spaces and 20 electric car spaces.

Three Rivers District Council’s parking standards set out the following maximum car parking provisions:

-  Residential: 1.75 space per 1-bed flats, 2 spaces per 2-bed flats and 2.25 spaces for 3-bed flats;

-  Office: 1 space per 30sqm;

-  Café: 1 space per 5sqm of floorspace of dining area plus 3 spaces per 4 employees;

-  Retail: 1 space per 30sqm for retail units of up to 500sqm GFA; and,

-  Gym:  1 space per 22sqm.

The cinema and training facilities would be for the use of residents and occupants of the office space and therefore will not require additional parking.

As the site is located in Zone 4 of the TRDC Accessibility zones, the parking provisions can be within 75-100% of the maximum car parking standards.

According to the Three Rivers District Council’s parking standards, the maximum permitted car parking spaces for the proposed developments has been calculated in the TA to be equal to 686 spaces. However, this is considered an over estimate as the café land use car parking provision was calculated using the whole of the café GFA and should have only considered the dining area. The applicant proposes the provision of 661 car parking spaces, equal to 99.3 per cent of the maximum allowed car parking provision. This is considered acceptable. However, it is ultimately the decision of the LPA to determine the suitability of the proposed car parking provisions.

Parking Layout

The exact dimensions of the proposed parking bays, as well as the spacing between them, has not been provided. As this is an outline planning application, this is considered acceptable. However, as part of reserved matters, the applicant would be required to provide dimensions and suitable swept path assessments demonstrating that vehicles can safely enter and depart from the car parking spaces safely and both enter the site and manoeuvre within it to depart in a forward gear.

Cycle Parking Provisions

The applicant stated in the TA that provision of 486 cycle parking spaces would be included as part of the proposed development.

Three Rivers District Council’s parking standards set out the following minimum cycle parking standards:

-  1 long-term space per 2 residential flats;

-  1 short-term space per 500sqm of GFA office space plus 1 long-term space per 10 full time staff;

-  1 short-term space per 100sqm of café GFA plus 1 long-term space per 10 maximum staff on site at one time;

-  1 short-term space per 150sqm of retail GFA plus 1 long-term space per 10 maximum staff on site at any time; and,

-  1 space per 25sqm of gym GFA.

The minimum required cycle parking spaces, according to Three Rivers District Council, is 185 assuming staff occupancy of 1 per 25sqm of GFA. Therefore, the provision of 468 cycle parking spaces proposed is considered acceptable. However, it is ultimately the decision of the LPA to determine the suitability of the proposed cycle parking provisions.

The provision of cycling parking spaces would be implemented via the provision of carousel compounds across the proposed development. Additionally, the provision of a cycle club of 60 electric bicycles is proposed in the submitted TA. The effective and sustainable management of the club should be set out as part of the Travel Plan for the proposed scheme.

Motorcycle Parking Provisions

The applicant has proposed 40 motorcycle parking spaces for the proposed development. TRDC standards state that 5% of car parking available should be motorcycle parking. Therefore, with a provision of 661 car parking spaces, 33 motorcycle spaces should be included. Therefore, 40 motorcycle spaces is acceptable.

Accessibility

The site has a low level of accessibility. This is demonstrated by the fact that it lies in a Zone 4 area (the lowest measured level) on map 10 of TRDC’s parking standards in appendix 7 of the Core Strategy.

Public Transport

The proposed development site is not currently serviced by any bus services.

At present, the site serviced by Croxley and Moor Park Metropolitan Line stations and is midway between the two. There are footpaths across the Moor that would provide walking or cycling access to Croxley station from the site. Moor Park is currently accessed by people walking along the unsuitable north verge of Tolpits Lane.

The area will soon benefit from the planned Metropolitan Line Extension (formerly known as the Croxley Rail Link) which will connect Watford

Junction and Croxley stations and will include two new stations in west Watford. The nearest, Cassiobridge station, will be some 1500m or 28 minutes walking distance from the application site.

A bus service is proposed as part of the application submission. This would enhance the sustainability of the Moors site. The proposals are provided in a document with associated costs in the appendix of the TA. The proposed bus routes would be:

- Wolsey Business Park (The Moors) - Rickmansworth station

- Wolsey Business Park (The Moors) - Watford Junction station

The proposed bus services are the same as those submitted as part of a previous application, reference 16/2497/FUL, have been reviewed and bus operator Arriva was contacted to discuss the proposals. It was noted that the fees associated with the cost breakdown in the appendix would need to be amended as they are dated July 2015 and costs will need to be derived to reflect the time when the service would start. It should also be noted that the patronage of the bus service may be affected with the extension of the Metropolitan Line.

HCC are aware, from contacts at the business park, of an interest in improved bus services. At present there is a shuttle bus operated by Camelot for their staff, it is unknown whether this would continue with the implementation of improved bus service. As was previously expressed, HCC are in favour of the Watford service option and the option 2 timetable offering higher frequency services in the AM and PM peak periods which may be attractive to surrounding businesses as well as commuters from the new development. HCC would accept a lower frequency on Saturday/Sunday, i.e. hourly. In relation to viability, the greater the number of units, the greater the potential patronage of the bus service especially for daytime services.

HCC passenger transport liaison team would prefer that the developer’s contribution towards this service were made a payment to them via a S106 agreement. That would enable them to put on something potentially more appropriate to the network at the time and responsive to demand.

Walking and Cycling

At present, the neighbourhood is flat and the applicant considers it walkable. There are no footways on Tolpits Lane. Footways are provided on both sides of Dwight Road from the roundabout junction with Tolpits Lane to the junction with Dwight Road and Dwight Road running east to west has a footway on its south side.

At present, pedestrians from the existing site can access traffic free footpaths that connect to the east, west and north of the site. The footpath routes are as follows:

-  Ebury Way to Rickmansworth and Watford; and,

-  Public footpath Croxley Green 017 across Common Moor to Croxley.

Existing arrangements for cyclists include easy access to the Ebury Way cycle route that forms part of the National Cycle Route 6 which connects from Uxbridge in the south and Watford and St Albans to the northeast. As previously mentioned, this is a traffic-free route. The route would connect cyclists to Uxbridge, Rickmansworth, Watford and St. Albans. Tolpits Lane is not conducive to cycling.

Travel Plan

As part of the TA, the consultant has included a Framework Travel Plan (FTP). The FTP is suitable to provide information to demonstrate that a full Travel Plan (TP) for the residential element of the site and Travel Plan Statements for the office and gym uses could be implemented. At the appropriate time a full Travel Plan and Travel Plan Statements should be required to encourage the use of sustainable transport modes and to reduce the reliance on private cars and to ensure minimal impact to the highway safety and function as a consequence of the development. Any TP submitted will have to be in line with HCC’s guidance set out in the County Council’s document ‘Hertfordshire’s Travel Plan Guidance for Business and Residential Development’ as set out at

http://www.hertfordshire.gov.uk/services/transtreets/highways/highwaysinfo/hiservicesforbus/devmanagment/greentravelplans1/.

The FTP has been reviewed and the following comments should be considered for preparation of the future full Travel Plan(s).

General

-  National and local policy background to be included in Travel Plan

Travel Plan management

-  Travel Plan co-ordinator details to be provided on appointment and details of secondary contact in case of personnel changes

-  An outline of the duties assigned to the co-ordinator, time allocated to the role, frequency on site.

-  A statement from a member of senior management demonstrating commitment to the success of the plan.

-  A Travel Plan group is mentioned - details are needed of potential membership, frequency of meetings

-  Identification of relevant stakeholders/external partners whose involvement will be required for the success of the plan - eg bus operators, cycling groups, cycle shops, car club provider etc

Measures

-  Residential travel pack contribution - discount vouchers mentioned for public transport/cycling/walking - suggested contribution contained in Appendix E of the HCC guidance.

Targets, monitoring and action plan

-  Template to be completed.

-  Baseline mode split data to be established on first survey.

-  Interim modal shift targets - target only provided for reduction in car mode share over whole plan period - need yearly targets or as minimum for years 1, 3 and 5 and give targets for increases in other sustainable modes.

-  Confirmed mode shift targets - to be submitted following initial survey.

-  Monitoring - full surveys required to obtain meaningful numbers of responses.  Multi-modal counts would also be useful to provide a fuller picture in years 1, 3 and 5.

-  Travel plan review - review of travel plan and progress towards targets and identification of additional measures necessary in year 1, 3 and 5.

-  Evaluation and support fee to be provided

Construction

As part of the TA, the applicant has provided an indicative construction programme for consideration and the information provided is considered appropriate. A full Construction Traffic Management Plan will be required to ensure construction vehicles will not have a detrimental impact on the vicinity of the site and a condition will be required to provide adequate parking for construction vehicles on-site to prevent on-street conflict and impacts to the highway safety.

A Construction Traffic Management Plan would be required for all phases of the construction, including demolition, excavation of the substantial basement and construction of all elements of the building (basement car parking levels through to the buildings above. It is expected that for a basement car park of this magnitude, there would be significant trips associated with the excavation phase. Measures would also be required to protect users of the local road network from hazards arising from undue damage caused from its use by large numbers of HGVs associated with the demolition and excavation phases of the project.

Planning Obligations / Community Infrastructure Levy (CIL)

It is the policy of the County and District Councils to seek planning obligations to mitigate the effects of development. HCC’s requirements in respect of highways and transport are set out in section 11 of the document ‘Planning Obligations Guidance - Toolkit for Hertfordshire (Hertfordshire County Council's requirements)’. This can be read and downloaded from http://www.hertsdirect.org/your-council/hcc/resandperf/hertsprop/planningobs/.

A S106 Agreement will be required to secure a Construction Traffic Management Plan Planning Obligation and support for the Travel Plan.

The applicant has proposed the funding of a bus service passing the site. This should be secured by S106 agreement. The S106 agreement should include a contribution towards the provision of a foot and cycleway along the northern verge of Tolpits Lane.

It is noted that draft S106 agreements have been received for the scheme. The agreements are templates with details of the development and the signatories added but no figures for financial contributions inserted and no specific obligations for monitoring fees for Travel Plan monitoring and Construction Traffic Management Plan monitoring. The template for the S106 Agreement will need to be modified to an up to date version at the point where S106 Agreement is under discussion.

Summary

HCC as highway authority has reviewed the application submission and raise no objection of the development, subject to the imposition of suitable planning conditions.

APPLICATION B [Objection]

Notice is given under article 18 of the Town and Country Planning (Development Management Procedure) (England) Order 2015 that the Hertfordshire County Council as Highway Authority recommends that permission be refused for the following reasons:

Decision

Hertfordshire County Council as Highway Authority wishes to raise an objection to the proposed development due to overprovision of car parking. The applicant needs to demonstrate that the proposals are not contrary to local and national planning policies which seek to promote sustainable transport use, inclusive of national planning guidance ‘National Planning Policy Framework’, in particular paragraphs 17, 30, 32, 35 and 39, and HCC’s Local Transport Plan 3 policies 3.8 and 3.15. HCC would be willing to reconsider our recommendation should the applicant submit an amended scheme with reduced car parking to be in line with the maximum car parking standards set out by Three Rivers District council. However, it should be noted that it would be for the LPA to determine whether such an amendment would be acceptable to consider for the current application.

Description of the Proposal

The proposal is for the demolition of two 3-storey offices with underground parking to be replaced by a 5-storey building and two 3-storey buildings, all with shared underground parking.

The proposed development includes:

-   One 5-storey residential building with a total of 332 flats and a gym, cinema and training and education facility on the ground floor;

-  Cinema - 300 sqm;

-  Training and Education Facility - 300 sqm;

-  Gym - 800 sqm; and,

-  Flats: 50 studio, 155 1-bed; 119 2-bed; and, 8 3-bed.

-  Two 3-storey office buildings;

-  Office Space: 2,500 sqm

-  Fitness Area

-  Café: 150sqm café

-  Food Retail: 300 sqm

-  Underground parking with provisions for 851 cars, which includes 35 disabled spaces and 20 electric car spaces.

Site Description

The development site is located in the Wolsey Business Park at the northwest corner of the eastern junction of Tolpits Lane (A4145) and Dwight Road in Watford. At present, the site is occupied by 2 3-storey buildings. Unit 3, Symbio Point is currently used as a secondary school with a temporary planning permission and Unit 4, York House is used as office space.

The site is bordered to the east and west by Dwight Road, to the south by Tolpits Lane (A4145) and to the north by the private site access road. Dwight Road is an unclassified local access road with a 30mph speed limit and Tolpits Lane is a principal A-class main distributor road with a 40mph speed limit.

Analysis

As part of the planning application package, the applicant has provided a Transport Assessment (TA) to demonstrate the likely impact of the proposed development on the local highway network. This is as required in Roads in Hertfordshire (section 1 chapter 7).

A Design and Access Statement (DAS) is required for all planning applications that have an impact on the highway, as outlined in Roads in Hertfordshire: Design Guide (section 1 chapter 7). A DAS has been provided for the proposed development and is considered appropriate for the purposes of this planning application.

Policy Review

As part of the Transport Assessment provided for consideration, the applicant has provided evidence of review of the following policy documents:

-  National Planning Policy Framework (2012)

-  National Planning Practice Guidance

-  Hertfordshire County Council (HCC) highway design guide Roads in Hertfordshire 3rd edition

-  Relevant Three Rivers District Council core strategies relating to transport and parking requirements

-  Hertfordshire County Council (HCC) Local Transport Plan 3: 2011-2031

Trip Generation and Distribution

The applicant has provided a Transport Assessment (TA) with the application package for consideration.

Trip Generation

A trip generation profile for the existing site and the proposed development are provided as part of the TA.

Existing Situation

The TA provides existing trip generation profiles considering previously agreed trip rates submitted and agreed as part of planning application submissions for application reference numbers: 15/1281/PDR and 15/1184/PDR. This is considered acceptable.

The TA outlines the existing permitted trip generation profiles for each of the buildings on sites, Units 3 and 4  Wolsey Business Park. The lowest approved trip generation profile for each site was used to establish an existing trip generation profile for the total site. This approach is considered acceptable as it presents a worst case scenario when compared with the proposed development.

Unit 3 has 3 permitted uses: Office, Residential and Reach Free School (temporary). The trip profile for each site was provided in the TA and the lowest trip profile was for the permitted residential scheme and is as follows:

-  AM Peak: 6 arrivals, 22 departures giving 28 two-way movements

-  PM Peak: 19 arrivals, 8 departures giving 27 two-way movements

Unit 4 has 2 permitted uses: Office and Residential. The trip profile for each site was provided in the TA and the lowest trip profile was for the permitted residential scheme and is as follows:

-  AM Peak: 3 arrivals, 3 departures giving 6 two-way movements

-  PM Peak: 12 arrivals, 8 departures giving 20 two-way movements

Therefore, the total trip generation profile for the existing land uses for the site is as follows:

-  AM Peak: 9 arrivals, 25 departures giving 34 two-way movements

-  PM Peak: 31 arrivals, 16 departures giving 47 two-way movements

Proposed Land Uses

The trip rates of the proposed development were generated by interrogating the TRICS database. This approach is considered acceptable.

The TA used the same TRICS outputs provided as part of the original application submission for application reference 15/1935/FUL. These TRICS outputs were not disputed and are therefore considered acceptable.

The following parameters were used for the TRICS interrogation for all land uses:

-  England only, excluding Greater London;

-  Excluding weekends;

-  Average trip rates adopted; and,

-  Peak Hours 08:00 - 09:00 and 17:00 - 18:00.

The following parameter was used to establish the residential trip generation profile: Residential land use 03 - Residential - C Flats Privately Owned.

The following parameter was used for the TRICS interrogation to establish the office trip generation profile: Employment land use 02 - Employment - A Office.

The following parameter was used for the TRICS interrogation to establish the café trip generation profile: Café land use 03 - Hotel, Food and Drink - E Roadside food.

The following parameter was used for the TRICS interrogation to establish the retail trip generation profile: Retail land use 01 - Retail - O Convenience Store.

The following parameter was used for the TRICS interrogation to establish the gym trip generation profile:   Leisure - Gym land use 07 - Leisure - K Fitness Club.

The following trips rates are associated with the above TRICS interrogation outputs:

The proposed trips rates generated by the TRICS assessment for the proposed development are as follows:

Residential

-  AM Peak: 0.068 arrivals, 0.239 departures

-  PM Peak: 0.220 arrivals, 0.096 departures

Office

-  AM Peak: 1.232 arrivals, 0.236 departures

-  PM Peak: 0.173 arrivals, 1.027 departures

Café

-  AM Peak: 3.057 arrivals, 3.057 departures

-  PM Peak: 3.806 arrivals, 3.922 departures

Food Retail

-  AM Peak: 7.424 arrivals, 7.348 departures

-  PM Peak: 7.576 arrivals, 7.652 departures

Leisure (Gym)

-  AM Peak: 0.951 arrivals, 1.302 departures

-  PM Peak: 3.040 arrivals, 1.661 departures

The applicant does not provide trip generation analysis for the Leisure - Cinema and Training & Education land uses due to their sole use by the proposed development’s residents. This is considered acceptable, subject to a suitable condition to guarantee that the proposed land uses will remain for the sole use of employees of the office land use and residents of the residential land use.

Using the predicted rates and proposed development areas the associated trips generated by the proposed development at this location would therefore be:

Residential

-  AM Peak: 23 arrivals, 80 departures giving 103 two-way

-  PM Peak: 74 arrivals, 32 departures giving 106 two-way

Office

-  AM Peak: 31 arrivals, 6 departures giving 37 two-way

-  PM Peak: 5 arrivals, 26 departures giving 31 two-way

Café

-  AM Peak: 5 arrivals, 5 departures giving 10 two-way

-  PM Peak: 6 arrivals, 6 departures giving 12 two-way

Food Retail

-  AM Peak: 23 arrivals, 23 departures giving 46 two-way

-  PM Peak: 23 arrivals, 23 departures giving 46 two-way

Leisure (Gym)

-  AM Peak: 8 arrivals, 11 departures giving 19 two-way

-  PM Peak: 25 arrivals, 14 departures giving 39 two-way

The total proposed trips for the proposed development are as follows:

-  AM Peak: 90 arrivals, 125 departures giving 215 two-way

-  PM Peak: 133 arrivals, 101 departures giving 234 two-way

The total proposed trip generation profile was compared to the existing trip generation profile to establish the net impact at the site. The net trip generation profile for the site is as follows:

-  AM Peak: 81 arrivals, 100 departures giving 181 two-way

-  PM Peak: 102 arrivals, 85 departures giving 187 two-way

Multi-modal Trip Generation

In addition to the vehicular trip generation profile, the applicant has provided a multimodal trip generation profile for Method of Travel to Work based on 2011 census data for the Moor Park and Eastbury ward. The multi-modal trip generation profile is provided to present the likely methods of travel to / from the site for work purposes. This approach is considered appropriate. The vehicular trips estimated as part of the vehicular trip generation profile for the proposed development were used to extrapolate the number of users by mode. The multi-modal trip generation profile shows a demand for public transport and footway access. Therefore, contributions are being be sought to improve connections to the site on foot and cycle and by public transport.

Trip Distribution

Trip distribution assumptions for the proposed development traffic have been included as part of the TA and trip distributions proportions are derived from observed and synthesised turning movements at the Dwight Road / Tolpits Lane roundabout and the priority junction of Dwight Road with the site access road. This is considered an acceptable approach.

Impact on the Highway

Junction Assessment

Junction Modelling

The applicant has provided junction capacity assessments for the priority junction access road with Dwight Road and the Tolpits Lane and Dwight Road roundabout junction. As requested by HCC in discussions following the original application submission (application reference 15/1935/FUL) the TA includes junction modelling standalone junction assessments and the scenario where the two junctions are connected. The junction modelling was carried out using industry standard Junctions 9 software.

It was agreed as part of discussions following the original application that any resubmission would include an assessment of the opening year plus a 5 year horizon year. As part of the TA for this application, the year of opening would be 2020 and the proposed 5-year horizon year would be 2025. This is considered acceptable for the proposed assessment.

The applicant has provided growth rates for the horizon years using industry standard TEMPRO database. This is considered an appropriate method.

The scenarios modelled as part of the assessment include:

-  Do Nothing 2020: Base traffic 2020;

-  Do Nothing 2025: Base traffic 2025;

-  Do Something 2020: Base traffic 2020 + development traffic; and,

- Do Something 2025: Base traffic 2025 + development.

The TA summarises the results of both the unlinked and linked junction assessments. The junction model outputs are presented in an appendix of the TA.

The operational capacity of the unlinked junctions was considered using the RFC (Ratio of Flow to Capacity), queuing and delay for consistency with other planning applications; however, Level of Service (LOS) was also considered for the unlinked junction operational capacity results because the linked junction operational capacity assessments could not produce a RFC value for comparison. This is explained in the summary in the TA for the linked junction assessment which states that as the modelling for the linked junctions uses the lane simulation mode, the results do not specify a RFC value for the junctions. Therefore, the operational capacity of the junctions was considered using the Level of Service (LOS), queuing and delay. Therefore, the LOS is considered for both of the junction capacity assessment scenarios. This is considered acceptable for the purposes of the TA.

Typically the Level of Service (LOS) of a junction in the range A to D is considered to be acceptable, with LOS of A/B most desirable and LOS of C/D acceptable. A LOS of E would imply that the operation of the junction is beginning to fail and likely to be operating with congestion and delays. A LOS of F implies that the junction has reached or is over its capacity and is likely to be experiencing severe delays and congestion. The thresholds A-F are based on the queuing delay on each arm. The definitions can be summarised as follows:

A = Free flow

B = Reasonably free flow

C = Stable flow

D = Approaching unstable flow

E = Unstable flow

F = Forced or breakdown flow

Please note that the PCU (Passenger Car Unit) is the equivalent of 1 passenger car or 5.75m in length.

Unlinked Junction Capacity Assessments

The junction capacity results for the unconnected Tolpits Lane roundabout with Dwight Road demonstrate that in both the AM and PM peak periods in both horizon years and for the Do-Nothing and Do-Something scenarios, the junction will continue to operate within capacity. The junction is likely to experience a maximum RFC of 0.63 with associated LOS of A, a queue of 1.7 PCUs and a predicted total junction arm delay of 6.41 seconds. This will occur during the 2025 AM peak, Do Something scenario.

The junction capacity results for the unconnected Dwight Road priority T-junction with the site access road demonstrate that in both the AM and PM peak periods in both horizon years and for the Do-Nothing and Do-Something scenarios, the junction is likely to continue to operate within capacity. The junction would experience a maximum RFC of 0.58 with associated LOS of C, a queue of 1.4 PCUs and a predicted total junction arm delay of 18.05 seconds. This would occur during the 2025 PM peak, Do Something scenario.

Link Junction Capacity Assessments

The LOS for the Tolpits Lane roundabout is a predicted to be A for all junction arms in the AM and PM peak hours. Maximum queues experienced at this junction occur in the AM peak for Tolpits Lane East arm with 1.7 PCUs for the 2025 horizon year and a maximum predicted total junction arm delay of 6.34 seconds.

The LOS for the Dwight Road and site access road priority T-junction is predicted to be A for all junction arms for the 2020 and 2025 horizon years in the AM and PM peak hours for the Do-Nothing scenario. The junction has a LOS of A for both the Dwight Road junction arms for the Do-Something scenario in both the AM and PM peaks for both horizon years. The site access junction arm was predicted to operate with a LOS of B for Do-Something scenario in the 2020 and 2025 horizon years for the AM peak and with a LOS of B for the 2020 PM peak and C for the 2025 PM peak. However, queuing at this junction would not exceed 1.6 PCUs with an associated maximum predicted total junction arm delay of 18.41s.

Summary

The results indicate that the junction is likely to operate within desired thresholds with LOS better than D, acceptable levels of queuing and delays for all public highway links.

The junction capacity test results indicate that the additional traffic added to the network as a consequence of the proposed development is not likely to have a detrimental impact on the operation of the highway network in the vicinity of the proposed development.

Highway Safety

The applicant has provided detailed collision data as part of the TA for the surrounding road network. This approach is considered acceptable. A review of the 5 years of collision data from 1st April 2011 to 31st March 2016, obtained from HCC reveals that there were 26 collisions resulting in injury in the vicinity of the site, all of which are designated slight collisions. There have been a total of 41 personal injuries resulting from these collisions.

There were 5 locations identified as part of the review where more than one collision occurred, as provided below:

-  A4145 Tolpits Lane, North of Croxley View junction: 3 PICs all human error, failing to look, reckless driving, following too close and loss of control.

-  A4145 Tolpits Lane/Croxley View: 7 PICs, 3 involved pedestrians. 5 collisions involved parking manoeuvres outside shops in the vicinity of the junction. Contributing factors include failing to look properly, reckless driving and wet road surface.

-  A4145 Tolpits Lane/ Chaffinch Lane: 2 PICs. One due to loss of control on wet surface and the other a consequence of driver travelling too close to a stopping vehicle.

-  A4145 Tolpits Lane, west of Dwight Road: 5 PICs, 1 injury to cyclist, 1 to motorcyclist and rest to car users. 4 PICs involved turning manoeuvres at entry to or exit from access roads from Tolpits Lane and were caused by misjudgement of speed or failure to look. 1 PIC involved vehicle overtaking cyclist.

-  A4145 Moor Lane/Sandy Lodge Road: 3 PICs. Contributing factors, failure to look properly, excessive speed, loss of control and slippery road surface.

The review of the collision data in the vicinity of the proposed development site does not present any safety issues with the highway and all collisions appear to have human judgement errors as contributing factors. Therefore, the proposed development is not expected to impact significantly on the safe operation of the local highway.

Refuse and Service Delivery

The applicant has stated in the TA that there will be 3 servicing areas for servicing, delivery and collection for the development site. Locations of the refuse collection have not been provided as part of the servicing and delivery arrangements. This is required to demonstrate that refuse collection will be in line with highway standards. A Refuse, Servicing and Delivery Plan is required to restrict the use of the bays by size of vehicle, etc. It is stated in the TA that vehicle swept path assessments within the site at ground and basement levels will be submitted under reserved matters. Due to the nature of the application as outline, this is considered acceptable.

Highway Layout

Vehicle Access

The proposed development would utilise the existing site access road from Dwight Road. The proposed development includes new accesses to the different blocks and the underground car park of the proposed development.

The internal layout of the basement level appears to operate under a one-way system. However, the layout at this stage is not finalised and would be agreed as part of reserved matters. Exact dimensions of the parking bays as well as the spacing between them are required to be provided by the applicant at the reserved matters stage. In addition, swept path assessments would be required to demonstrate that cars could enter, manoeuvre through and depart the site in a forward gear.

The HCC DM Implementation team were consulted for the same access arrangements proposed for a full planning application, reference number: 16/2497/FUL, on the same site. As the site access road is not adopted by HCC, and it is not likely to be, so they had no formal comments to make; however, there is concern related to the stop and reverse requirement for an HGV accessing the servicing areas for the proposed development. Therefore, evidence should be provided under reserved matters to support that the largest servicing vehicles can access the site safely and will not seek to park on Dwight Road to carry out any refuse collection, servicing or delivery at the site.

Pedestrian Access

It is proposed that the development would utilise the existing site access road and existing pedestrian facilities. It is unclear from the drawings if pedestrian facilities would be provided along the site frontage, adjacent to the site access road. Provisions for safe pedestrian access to the site are required. Additional details with regards to pedestrian access and footway provisions would be required at the reserved matters stage.

Swept Path Analysis

The applicant has not provided swept path assessments for the servicing areas, or access to the car park and car parking spaces. However, as this is an outline application this is considered necessary and swept path assessments would be required at the reserved matters stage.

Road Safety Audit

A Stage 1 Road Safety Audit would be required to illustrate that the new access arrangements are safe and appropriate for the size of the development. As stated in  the meeting minutes recorded from a meeting held between HCC, the transport consultant and the developer, it is agreed that ‘the execution of a Stage 1 Road Safety Audit as part of the S38/S278 process will be stated in the TA for the resubmission’. The applicant states that a Stage 1 Road Safety Audit for the site access would be conducted under a Section 38 agreement or a Section 278 agreement. This is considered appropriate.

Parking

The applicant has stated that the proposed development would provide 851 car parking spaces, which includes 35 disabled spaces and 20 electric car spaces.

Three Rivers District Council’s parking standards set out the following maximum car parking provisions:

-  Residential: 1.75 space per 1-bed flats, 2 spaces per 2-bed flats and 2.25 spaces for 3-bed flats;

-  Office: 1 space per 30sqm;

-  Café: 1 space per 5sqm of floor space of dining area plus 3 spaces per 4 employees;

-  Retail: 1 space per 30sqm for retail units of up to 500sqm GFA; and,

-  Gym:  1 space per 22sqm.

The cinema and training facilities would be for the use of residents and occupants of the office space and therefore will not require additional parking.

As the site is located in Zone 4 of the TRDC Accessibility zones, the parking provisions can be within 75-100% of the maximum car parking standards.

The maximum car parking provision, as stated in TRDCs parking standards, would equate to 776 car parking spaces. 851 is over this amount, at 109% of the total maximum allowed. It is considered that provision of extra parking will encourage the use of a personal vehicle as a mode of transport and a reduced car parking provision should be applied. Therefore, it is not considered appropriate to provide overprovision of car parking at this location. HCC are not satisfied with the overprovision of parking spaces for the proposals. However, it is ultimately the decision of the LPA to determine the suitability of the proposed car parking provisions.

Parking Layout

The exact dimensions of the proposed parking bays, as well as the spacing between them, have not been provided. As this is an outline planning application, this is considered acceptable. However, as part of reserved matters, the applicant would be required to provide dimensions or suitable swept path assessments demonstrating that vehicles can safely enter and depart from the car parking spaces safely and both enter the site and manoeuvre within it to depart in a forward gear.

Cycle Parking Provisions

The applicant stated in the TA that provision of 516 cycle parking spaces would be included as part of the proposed development.

Three Rivers District Council’s parking standards set out the following minimum cycle parking standards:

-  1 long-term space per 2 residential flats;

-  1 short-term space per 500sqm of GFA office space plus 1 l/t space per 10 full time staff;

-  1 short-term space per 100sqm of café GFA plus 1 l/t space per 10 maximum staff on site at one time;

-  1 short-term space per 150sqm of retail GFA plus 1 long term space per 10 maximum staff on site at any time; and,

-  1 space per 25sqm of gym GFA.

The minimum required cycle parking spaces, according to Three Rivers District Council, were calculated in the TA to be 217 assuming staff occupancy of 1 per 25sqm of GFA. Therefore, this it is considered acceptable to provide 516 cycle parking spaces. However, it is ultimately the decision of the LPA to determine the suitability of the proposed cycle parking provisions.

The provision of cycling parking spaces would be implemented via the provision of carousel compounds across the proposed development. Additionally, the provision of a cycle club of 60 electric bicycles is proposed in the submitted TA. The effective and sustainable management of the club should be set out as part of the Travel Plan for the proposed scheme.

Motorcycle Parking Provisions

The applicant has proposed 40 motorcycle parking spaces for the proposed development. TRDC standards state that 5% of car parking available should be motorcycle parking. Therefore, with a provision of 776 car parking spaces, 39  motorcycle spaces should be included. Alternatively, if the maximum standards from TRDC are considered at 851 total parking spaces, 43 should be motorcycle spaces. Therefore, 40 motorcycle spaces is acceptable.  However, it is ultimately the decision of the LPA to determine the suitability of the proposed cycle parking provisions.

Accessibility

The site has a low level of accessibility. This is demonstrated by the fact that it lies in a Zone 4 area (the lowest measured level) on map 10 of TRDC’s parking standards in appendix 7 of the Core Strategy.

Public Transport

The proposed development site is not currently serviced by any bus services.

At present, the site serviced by Croxley and Moor Park Metropolitan Line stations and is midway between the two. There are footpaths across the Moor that would provide walking or cycling access to Croxley station from the site. Moor Park is currently accessed by people walking along the unsuitable north verge of Tolpits Lane.

The area will soon benefit from the planned Metropolitan Line Extension (formerly known as the Croxley Rail Link) which will connect Watford

Junction and Croxley stations and will include two new stations in west Watford. The nearest, Cassiobridge station, will be some 1500m or 28 minutes walking distance from the application site.

A bus service is proposed as part of the application submission. This would enhance the sustainability of the Moors site. The proposals are provided in a document with associated costs in the appendix of the TA. The proposed bus routes would be:

- Wolsey Business Park (The Moors) - Rickmansworth station

- Wolsey Business Park (The Moors) - Watford Junction station

The proposed bus services are the same as those submitted as part of a previous application, reference 16/2497/FUL, have been reviewed and bus operator Arriva was contacted to discuss the proposals. It was noted that the fees associated with the cost breakdown in the appendix would need to be amended as they are dated July 2015 and costs will need to be derived to reflect the time when the service would start. It should also be noted that the patronage of the bus service may be affected with the extension of the Metropolitan Line.

HCC are aware, from contacts at the business park, of an interest in improved bus services. At present there is a shuttle bus operated by Camelot for their staff, it is unknown whether this would continue with the implementation of improved bus service. As was previously expressed, HCC are in favour of the Watford service option and the option 2 timetable offering higher frequency services in the AM and PM peak periods which may be attractive to surrounding businesses as well as commuters from the new development. HCC would accept a lower frequency on Saturday/Sunday, i.e. hourly. In relation to viability, the greater the number of units, the greater the potential patronage of the bus service especially for daytime services.

HCC passenger transport liaison team would prefer that the developer’s contribution towards this service were made a payment to them via a S106 agreement. That would enable them to put on something potentially more appropriate to the network at the time and responsive to demand.

Walking and Cycling

At present, the neighbourhood is flat and the applicant considers it walkable. There are no footways on Tolpits Lane. Footways are provided on both sides of Dwight Road from the roundabout junction with Tolpits Lane to the junction with Dwight Road and Dwight Road running east to west has a footway on its south side.

At present, pedestrians from the existing site can access traffic free footpaths that connect to the east, west and north of the site. The footpath routes are as follows:

-  Ebury Way to Rickmansworth and Watford; and,

-  Public footpath Croxley Green 017 across Common Moor to Croxley.

Existing arrangements for cyclists include easy access to the Ebury Way cycle route that forms part of the National Cycle Route 6 which connects from Uxbridge in the south and Watford and St Albans to the northeast. As previously mentioned, this is a traffic-free route. The route would connect cyclists to Uxbridge, Rickmansworth, Watford and St. Albans. Tolpits Lane is not conducive to cycling.

Travel Plan

As part of the TA, the consultant has included a Framework Travel Plan (FTP). The FTP is suitable to provide information to demonstrate that a full Travel Plan (TP) for the residential element of the site and Travel Plan Statements for the office and gym uses, could be implemented. At the appropriate time, a full Travel Plan and Travel Plan Statements would be required to encourage the use of sustainable transport modes and to reduce the reliance on private cars and to ensure minimal impact to the highway safety and function as a consequence of the development. Any TP submitted will have to be in line with HCC’s guidance set out in the County Council’s document ‘Hertfordshire’s Travel Plan Guidance for Business and Residential Development’ as set out at

http://www.hertfordshire.gov.uk/services/transtreets/highways/highwaysinfo/hiservicesforbus/devmanagment/greentravelplans1/.

The FTP has been reviewed, and the following comments should be considered for preparation of the future full Travel Plan(s).

General

-  National and local policy background to be included in Travel Plan

Travel Plan management

-  Travel Plan co-ordinator details to be provided on appointment and details of secondary contact in case of personnel changes

-  An outline of the duties assigned to the co-ordinator, time allocated to the role, frequency on site.

-  A statement from a member of senior management demonstrating commitment to the success of the plan.

-  A Travel Plan group is mentioned - details are needed of potential membership, frequency of meetings

-  Identification of relevant stakeholders/external partners whose involvement will be required for the success of the plan - eg bus operators, cycling groups, cycle shops, car club provider etc

Measures

-  Residential travel pack contribution - discount vouchers mentioned for public transport/cycling/walking - suggested contribution contained in Appendix E of the HCC guidance.

Targets, monitoring and action plan

-  Template to be completed.

-  Baseline mode split data to be established on first survey.

-  Interim modal shift targets - target only provided for reduction in car mode share over whole plan period - need yearly targets or as minimum for years 1, 3 and 5 and give targets for increases in other sustainable modes.

-  Confirmed mode shift targets - to be submitted following initial survey.

-  Monitoring - full surveys required to obtain meaningful numbers of responses.  Multi-modal counts would also be useful to provide a fuller picture in years 1, 3 and 5.

-  Travel plan review - review of travel plan and progress towards targets and identification of additional measures necessary in year 1, 3 and 5.

-  Evaluation and support fee to be provided

Construction

As part of the TA, the applicant has provided an indicative construction programme for consideration and the information provided is considered appropriate. A full Construction Traffic Management Plan will be required to ensure construction vehicles will not have a detrimental impact on the vicinity of the site and a condition will be required to provide adequate parking for construction vehicles on-site to prevent on-street conflict and impacts to the highway safety.

A Construction Traffic Management Plan would be required for all phases of the construction, including demolition, excavation of the substantial basement and construction of all elements of the building (basement car parking levels through to the buildings above. It is expected that for a basement car park of this magnitude, there would be significant trips associated with the excavation phase. Measures would also be required to protect users of the local road network from hazards arising from undue damage caused from its use by large numbers of HGVs associated with the demolition and excavation phases of the project.

Planning Obligations / Community Infrastructure Levy (CIL)

It is the policy of the County and District Councils to seek planning obligations to mitigate the effects of development. HCC’s requirements in respect of highways and transport are set out in section 11 of the document ‘Planning Obligations Guidance - Toolkit for Hertfordshire (Hertfordshire County Council's requirements)’. This can be read and downloaded from http://www.hertsdirect.org/your-council/hcc/resandperf/hertsprop/planningobs/.

A S106 Agreement will be required to secure a Construction Traffic Management Plan Planning Obligation and support for the Travel Plan.

The applicant has proposed the funding of a bus service passing the site. This should be secured by S106 agreement. The S106 agreement should include a contribution towards the provision of a foot and cycle way along the northern verge of Tolpits Lane.

It is noted that draft S106 agreements have been received for the scheme. The agreements are templates with details of the development and the signatories added but no figures for financial contributions inserted and no specific obligations for monitoring fees for Travel Plan monitoring and Construction Traffic Management Plan monitoring. The template for the S106 Agreement will need to be modified to an up to date version at the point where S106 Agreement is under discussion.

Summary

HCC as highway authority has reviewed the application submission and wishes to raise an objection to the development due to overprovision of car parking.

APPLICATION C [Objection]

Notice is given under article 18 of the Town and Country Planning (Development Management Procedure) (England) Order 2015 that the Hertfordshire County Council as Highway Authority recommends that permission be refused for the following reasons:

Decision

Hertfordshire County Council as Highway Authority wishes to raise an objection to the proposed development due to overprovision of car parking. The applicant needs to demonstrate that the proposals are not contrary to local and national planning policies which seek to promote sustainable transport use, inclusive of national planning guidance ‘National Planning Policy Framework’, in particular paragraphs 17, 30, 32, 35 and 39, and HCC’s Local Transport Plan 3 policies 3.8 and 3.15. HCC would be willing to reconsider our recommendation should the applicant submit an amended scheme with reduced car parking to be in line with the maximum car parking standards set out by Three Rivers District Council. However, it should be noted that it would be for the LPA to determine whether such an amendment would be acceptable to consider for the current application.

Description of the Proposal

The proposal is for the demolition of two 3-storey offices with underground parking to be replaced by a 6-storey building and two 3-storey buildings, all with shared underground parking for 943 cars.

The proposed development includes:

-   One 6-storey residential building with a total of 403 flats and a gym, cinema and training and education facility on the ground floor;

-  Cinema - 300 sqm;

-  Training and Education Facility - 300sqm;

-  Gym - 800sqm; and,

-  Flats: 56 studio, 190 1-bed; 149 2-bed; and, 8 3-bed.

-  Two 3-storey office buildings;

-  Office Space: 2500sqm;

-  Fitness Area;

-  Coffee Shop and Amenity Area: 150sqm café; and,

-  Food Retail: 300 sqm.

-  Underground parking with provisions for 943 cars, including 35 disabled spaces and 20 electric car spaces.

Site Description

The development site is located in the Wolsey Business Park at the northwest corner of the eastern junction of Tolpits Lane (A4145) and Dwight Road in Watford. At present, the site is occupied by 2 3-storey buildings. Unit 3, Symbio Point is currently used as a secondary school with a temporary planning permission and Unit 4, York House is used as office space.

The site is bordered to the east by Dwight Road, to the west by Dwight Road, to the south by Tolpits Lane (A4145) and to the north by the private site access road. Dwight Road is an unclassified local access road with a 30mph speed limit and Tolpits Lane is a principal A-class main distributor road with a 40mph speed limit.

Analysis

As part of the planning application package, the applicant has provided a Transport Assessment (TA) to demonstrate the likely impact of the proposed development on the local highway network. This is as required in Roads in Hertfordshire (section 1 chapter 7).

A Design and Access Statement (DAS) is required for all planning applications that have an impact on the highway, as outlined in Roads in Hertfordshire: Design Guide (section 1 chapter 7).A DAS has been provided for the proposed development and is considered appropriate for the purposes of this planning application.

Policy Review

As part of the Transport Assessment provided for consideration, the applicant has provided evidence of review of the following policy documents:

-  National Planning Policy Framework (2012)

-  National Planning Practice Guidance

-  Hertfordshire County Council (HCC) highway design guide Roads in Hertfordshire 3rd edition

-  Relevant Three Rivers District Council core strategies relating to transport and parking requirements

-  Hertfordshire County Council (HCC) Local Transport Plan 3: 2011-2031

Trip Generation and Distribution

The applicant has provided a Transport Assessment (TA) with the application package for consideration.

Trip Generation

Existing Land Uses

The TA provides existing trip generation profiles considering previously agreed trip rates submitted and agreed as part of planning application submissions for application reference numbers: 15/1281/PDR and 15/1184/PDR. This is considered acceptable.

The TA outlines the existing permitted trip generation profiles for each of the buildings on sites, Units 3 and 4  Wolsey Business Park. The lowest approved trip generation profile for each site was used to establish an existing trip generation profile for the total site. This approach is considered acceptable as it presents a worst case scenario when compared with the proposed development.

Unit 3 has 3 permitted uses: Office, Residential and Reach Free School (temporary). The trip profile for each site was provided in the TA and the lowest trip profile was for the permitted residential scheme and is as follows:

-  AM Peak: 6 arrivals, 22 departures giving 28 two-way

-  PM Peak: 19 arrivals, 8 departures giving 27 two-way

Unit 4 has 2 permitted uses: Office and Residential. The trip profile for each site was provided in the TA and the lowest trip profile was for the permitted residential scheme and is as follows:

-  AM Peak: 3 arrivals, 3 departures giving 6 two-way

-  PM Peak: 12 arrivals, 8 departures giving 20 two-way

Therefore, the total trip generation profile for the existing land uses for the site is as follows:

-  AM Peak: 9 arrivals, 25 departures giving 34 two-way

-  PM Peak: 31 arrivals, 16 departures giving 47 two-way

Proposed Land Uses

The trip rates of the proposed development were generated by interrogating the TRICS database. This approach is considered acceptable.

The TA used the same TRICS outputs provided as part of the original application submission for application reference 15/1935/FUL. These TRICS outputs were not disputed and are therefore considered.

The following parameters were used for the TRICS interrogation for all land uses:

-  England only, excluding Greater London;

-  Excluding weekends;

-  Average trip rates adopted; and,

-  Peak Hours 08:00 - 09:00 and 17:00 - 18:00.

The following parameter was used to establish the residential trip generation profile: Residential land use 03 - Residential - C Flats Privately Owned.

The following parameter was used for the TRICS interrogation to establish the office trip generation profile: Employment land use 02 - Employment - A Office.

The following parameter was used for the TRICS interrogation to establish the café trip generation profile:   Café land use 03 - Hotel, Food and Drink - E Roadside food.

The following parameter was used for the TRICS interrogation to establish the retail trip generation profile: Retail land use 01 - Retail - O Convenience Store.

The following parameter was used for the TRICS interrogation to establish the gym trip generation profile:   Leisure - Gym land use 07 - Leisure - K Fitness Club.

The following trips rates are associated with the above TRICS interrogation outputs:

-  Residential:

-  AM Peak: 0.068 in / 0.239 out

-  PM Peak: 0.220 in / 0.096 out

-  Office:

-  AM Peak: 1.232 in / 0.236 out

-  PM Peak: 0.173 in / 1.027 out

-  Café:

-  AM Peak: 3.057 in / 3.057 out

-  PM Peak: 3.806 in / 3.922 out

-  Retail:

-  AM Peak: 7.424 in / 7.348 out

-  PM Peak: 7.576 in / 7.652 out

-  Gym:

-  AM Peak: 0.951 in / 1.302 out

-  PM Peak: 3.040 in / 1.661 out

The applicant does not provide trip generation analysis for the Leisure - Cinema and Training & Education land uses due to their sole use by the proposed development’s residents. This is considered acceptable, subject to a suitable condition to guarantee that the proposed land uses will remain for the sole  use of users of the office and residential land uses.

Using the predicted rates and proposed development areas, the associated trips generated by the proposed development are as follows:

-  Residential:

-  AM Peak: 28 arrivals, 97 departures giving 125 two-way

-  PM Peak: 89 arrivals, 39 departures giving 128 two-way

-  Office:

-  AM Peak: 31 arrivals, 6 departures giving 37 two-way

-  PM Peak: 5 arrivals, 26 departures giving 31 two-way

-  Café:

-  AM Peak: 5 arrivals, 5 departures giving 10 two-way

-  PM Peak: 6 arrivals, 6 departures giving 12 two-way

-  Retail:

-  AM Peak: 23 arrivals, 23 departures giving 46 two-way

-  PM Peak: 23 arrivals, 23 departures giving 46 two-way

-  Gym:

-  AM Peak: 8 arrivals, 11 departures giving 19 two-way

-  PM Peak: 25 arrivals, 14 departures giving 39 two-way

Total:

AM Peak: 95 arrivals, 142 departures giving 237 two-way

PM Peak: 148 arrivals, 108 departures giving 256 two- way

The total proposed trip generation profile was compared to the existing trip generation profile to establish the net impact at the site. The net trip generation profile for the site is as follows:

-  AM Peak: 86 arrivals, 117 departures giving 203 two-way

-  PM Peak: 117 arrivals, 92 departures giving 209 two-way

Multi-Modal Trip Generation

In addition to the vehicular trip generation profile, the applicant has provided a multimodal trip generation profile for Method of Travel to Work based on 2011 census data for the Moor Park and Eastbury ward. The multi-modal trip generation profile is provided to present the likely methods of travel to / from the site for work purposes. This approach is considered appropriate. The vehicular trips estimated as part of the vehicular trip generation profile for the proposed development were used to extrapolate the number of users by mode. The multi-modal trip generation profile shows a demand for public transport and footway access. Therefore, contributions are being sought to improve connections to the site on foot and cycle and by public transport.

Trip Distribution

Trip distribution assumptions for the proposed development traffic have been included as part of the TA and trip distributions proportions are derived from observed and synthesised turning movements at the Dwight Road / Tolpits Lane roundabout and the priority junction of Dwight Road with the site access road. This is considered an acceptable approach.

Impact on Highway Network

Junction Assessment

The applicant has provided junction assessments for the priority junction of the access road with Dwight Road and the Tolpits Lane and Dwight Road roundabout junction. As requested by HCC in discussions following the original application submission (application reference 15/1935/FUL), the TA includes junction modelling for each junction and assessment where the two junctions are connected. The junction modelling was carried out using industry-standard Junctions 9 software.

It was agreed as part of discussions following the original application that any resubmission would include an assessment of the opening year plus a 5-year horizon year. As part of the TA for this application, the year of opening would be 2020 and the proposed 5-year horizon year would be 2025. This is considered acceptable for the proposed assessment.

The applicant has provided growth rates for the horizon years using industry-standard TEMPRO database. This is considered an appropriate method.

The scenarios modelled as part of the assessment include:

-  Do Nothing 2020: Base traffic 2020;

-  Do Nothing 2025: Base traffic 2025;

-  Do Something 2020: Base traffic 2020 + development traffic; and,

- Do Something 2025: Base traffic 2025 + development.

The TA summarises the results of both the unlinked and linked junction assessments. The junction model outputs are presented in an appendix of the TA.

The operational capacity of the unlinked junctions was considered using the RFC (Ratio of Flow to Capacity), queuing and delay for consistency with other planning applications; however, Level of Service (LOS) was also considered for the unlinked junction operational capacity results because the linked junction operational capacity assessments could not produce a RFC value for comparison. This is explained in the summary in the TA for the linked junction assessment which states that as the modelling for the linked junctions uses the lane simulation mode, the results do not specify a RFC value for the junctions. Therefore, the operational capacity of the junctions was considered using the Level of Service (LOS), queuing and delay. Therefore, the LOS is considered for both of the junction capacity assessment scenarios. This is considered acceptable for the purposes of the TA.

Typically the Level of Service (LOS) of a junction in the range of A to D are acceptable, with LOS A/B most desirable and LOS C/D acceptable. A LOS of E would imply that the operation of the junction is beginning to fail and likely to be operating with congestion and delays. A LOS of F implies that the junction has reached or is over its capacity and is likely to be experiencing severe delays and congestion. The thresholds A-F are based on the queuing delay on each arm. The definitions can be summarised as follows:

A = Free flow

B = Reasonably free flow

C = Stable flow

D = Approaching unstable flow

E = Unstable flow

F = Forced or breakdown flow

Please note that the PCU (Passenger Car Unit) is the equivalent of 1 passenger car or 5.75m in length.

Unlinked Junction Capacity Assessments

The junction capacity results for the unconnected Tolpits Lane roundabout with Dwight Road demonstrate that in both the AM and PM peak periods in both horizon years and for the Do-Minimum and Do-Something scenarios, the junction will continue to operate within capacity. The junction is likely to experience a maximum RFC of 0.64 with associated LOS of A, a queue of 1.8 PCUs and a predicted total junction arm delay of 6.49 seconds. This will occur during the 2025 AM peak Do Something scenario.

The junction capacity results for the unconnected Dwight Road priority T-junction with the site access road demonstrate that in both the AM and PM peak periods in both horizon years and for the Do-Minimum and Do-Something scenarios, the junction is likely to continue to operate within capacity. The junction will experience a maximum RFC of 0.60 with associated LOS of C, a queue of 1.5 PCUs and a predicted total junction arm delay of 18.92 seconds. This would occur during the 2025 PM peak, Do Something scenario.

Link Junction Capacity Assessments

The LOS for the Tolpits Lane roundabout is predicted to be  A for all junction arms in the AM and PM peak hours. Maximum queues experienced at this junction occur in the AM peak for Tolpits Lane East arm with 1.8  PCUs for the 2025 horizon year and a maximum predicted total junction arm delay of 6.5 seconds.

The LOS for the Dwight Road and site access road priority T-junction is predicted to be A for all junction arms for the 2020  and 2025 horizon years in the AM and PM peak hours for the Do-Minimum scenario. The junction has a LOS of A for both the Dwight Road junction arms for the Do-Something scenario in both the AM and PM peak hours for both horizon years. The site access junction arm was predicted to operate with a LOS of B for Do-Something scenario in the 2020 and 2025 horizon years for the AM peak; however, will operate with a LOS of C for the Do Something scenario in the 2020 and 2025 horizon years in the PM peak. However, queuing at this junction would not likely exceed 1.8 PCUs at the junction with a maximum predicted total junction arm delay of 19 seconds.

Summary

The results indicate that the junction is likely to operate within desired thresholds with LOS under D, acceptable levels of queuing and delays for all public highway links.

The junction capacity test results indicate that the additional traffic added to the network as a consequence of the proposed development is not likely to have a detrimental impact on the operation of the highway network in the vicinity of the proposed development.

Highway Layout

Highway Safety

The applicant has provided detailed collision data as part of the TA for the surrounding road network.  This approach is considered acceptable. A review of the 5 years of collision data from 1st April 2011 to 31st March 2016, obtained from HCC reveals that there were 26 collisions resulting in injury in the vicinity of the site, all of which are designated slight collisions. There has been a total of 41 personal injuries resulting from these collisions.

There were 5 locations identified as part of the review where more than one collision occurred, as provided below:

-  A4145 Tolpits Lane, North of Croxley View junction: 3 PICs all human error, failing to look, reckless driving, following too close and loss of control.

-  A4145 Tolpits Lane/Croxley View: 7 PICs, 3 involved pedestrians. 5 collisions involved parking manoeuvres outside shops in the vicinity of the junction. Contributing factors include failing to look properly, reckless driving and wet road surface.

-  A4145 Tolpits Lane/ Chaffinch Lane: 2 PICs. One due to loss of control on wet surface and the other a consequence of driver travelling too close to a stopping vehicle.

-  A4145 Tolpits Lane, west of Dwight Road: 5 PICs, 1 injury to cyclist, 1 to motorcyclist and rest to car users. 4 PICs involved turning manoeuvres at entry to or exit from access roads from Tolpits Lane and were caused by misjudgement of speed or failure to look. 1 PIC involved vehicle overtaking cyclist.

-  A4145 Moor Lane/Sandy Lodge Road: 3 PICs. Contributing factors, failure to look properly, excessive speed, loss of control and slippery road surface.

The review of the collision data in the vicinity of the proposed development site does not present any safety issues with the highway and all collisions appear to have human judgement errors as contributing factors. Therefore, the proposed development is not expected to impact significantly on the safe operation of the local highway.

Highway Layout

Vehicle Access

The proposed development would utilise the existing site access road from Dwight Road. The proposed development includes a new access from the site access road, providing access to an underground level which includes underground parking for motorcycles, electric cars and access to the car parking spaces.

The internal layout of the basement level appears to operate under a one-way system. However, the layout at this stage is not finalised and would be agreed at the reserved matters stage. It should be noted that exact dimensions of the underground parking bays are required to be provided by the applicant as part of reserved matters, as well as the spacing between them. In addition, swept path assessments would be  required to demonstrate that vehicles could enter and leave the site in a forward gear as well as manoeuvre inside.

The HCC DM Implementation team were consulted on the access arrangements for the proposed development site for a similar scheme at this location with the same access arrangements. As the site access road is not adopted by HCC, and is not likely to, so they had no formal comments to make; however, there is concern related to the stop and reverse requirement for an HGV accessing the servicing areas for the proposed development. Therefore, evidence should be provided under reserved matters to support that the largest servicing vehicles can be access the site safely and will not seek to park on Dwight Road to carry out any refuse collection, servicing or delivery at the site.

Pedestrian Access

It is proposed that the development would utilise the existing site access road and existing pedestrian facilities. It is unclear from the drawings if pedestrian facilities would be provided along the site frontage, adjacent to the site access road. Provisions for safe pedestrian access to the site are required. Additional details with regards to pedestrian access and footway provisions would be required at the reserved matters stage.

Swept Path Analysis

The applicant has not provided swept path assessments for the servicing areas, or access to the car park and car parking spaces. However, as this is an outline application this is considered necessary and swept path assessments would be required at the reserved matters stage.

Road Safety Audit

A Stage 1 Road Safety Audit would be required as there are proposed changes to the existing access arrangement from the site to the highway network. As stated in  the meeting minutes recorded from a meeting held between HCC, the transport consultant and the developer, it is agreed that ‘the execution of a Stage 1 Road Safety Audit as part of the S38/S278 process will be stated in the TA for the resubmission’. The applicant states that a Stage 1 Road Safety Audit for the site access would be conducted under a Section 38 agreement or a Section 278 agreement. This is considered appropriate.

Refuse and Service Delivery

The applicant has stated in the TA that there will be 3 servicing areas for servicing, delivery and collection for the development site. Locations of the refuse store and collection have not been provided as part of the servicing and delivery arrangements. This is required to demonstrate that refuse collection will be in line with standards for refuse collection. A Refuse, Servicing and Delivery Plan is required to ensure that suitable vehicles would be able to access the appropriate bays, etc.  It is stated in the TA that vehicle swept path assessments within the site at ground and basement levels will be submitted under reserved matters. Due to the nature of the application as outline, this is considered acceptable.

Parking

The applicant has stated that the proposed development would provide 943 car parking spaces, including 35 disabled spaces and 20 electric car spaces.

Three Rivers District Council’s parking standards set out the following maximum car parking provisions:

-  Residential: 1.75 space per 1-bed flats, 2 spaces per 2-bed flats and 2.25 spaces for 3-bed flats;

-  Office: 1 space per 30sqm;

-  Café: 1 space per 5sqm of floor space of dining area plus 3 spaces per 4 employees;

-  Retail: 1 space per 30sqm for retail units of up to 500sqm GFA; and,

-  Gym:  1 space per 22sqm.

The cinema and training facilities would be for the use of residents and occupants of the office space and therefore will not require additional parking.

The maximum car parking provision, as calculated in the TA, would equate to 908 car parking spaces. The maximum parking provision would likely be less than 908 spaces as the car parking spaces for the café land use are calculated using the whole GFA for the café and it should only be calculated using the dining room GFA. 943 is over this amount, at 104% of the total maximum allowed. It is considered that provision of extra parking will encourage the use of a personal vehicle as a mode of transport and a reduced car parking provision should be applied. Therefore, it is not considered appropriate to provide overprovision of car parking at this location. However, it is ultimately the decision of the LPA to determine the suitability of the proposed car parking provisions.

Parking Layout

The exact dimensions of the proposed parking bays, as well as the spacing between them, have not been provided. As this is an outline planning application, this is considered acceptable. However, as part of reserved matters, the applicant would be required to provide dimensions or suitable swept path assessments demonstrating that vehicles can safely enter and depart from the car parking spaces safely and both enter the site and manoeuvre within it to depart in a forward gear.

Cycle Parking Provisions

The applicant stated in the TA that provision of 516 cycle parking spaces would be included as part of the proposed development.

Three Rivers District Council’s parking standards set out the following minimum cycle parking standards:

-  1 long-term space per 2 residential flats;

-  1 short-term space per 500sqm of GFA office space plus 1 long-term space per 10 full time staff;

-  1 short-term space per 100sqm of café GFA plus 1 long-term space per 10 maximum staff on site at one time;

-  1 short-term space per 150sqm of retail GFA plus 1 long-term space per 10 maximum staff on site at any time; and,

-  1 space per 25sqm of gym GFA.

The minimum required cycle parking spaces, according to Three Rivers District Council, were calculated in the TA to be 253 cycle parking spaces. Therefore, the cycle parking provisions are above the minimum standards set out by TRDC.  However, it is ultimately the decision of the LPA to determine the suitability of the proposed cycle parking provisions.

The provision of cycling parking spaces would be implemented via the provision of carousel compounds across the proposed development. Additionally, the provision of a cycle club of 60 electric bicycles is proposed in the submitted TA. The effective and sustainable management of the club should be set out as part of the Travel Plan for the proposed scheme.

Motorcycle Parking Provisions

The applicant has proposed 40 motorcycle parking spaces for the proposed development. TRDC standards state that 5% of car parking available should be motorcycle parking. Therefore, with a provision of 943 car parking spaces, 47 motorcycle spaces should be included. Alternatively, if the maximum standards from TRDC are considered at 908 total parking spaces, 45 should be motorcycle spaces. Therefore, 40 motorcycle spaces is below the minimum standards. However, it is ultimately the decision of the LPA to determine the suitability of the proposed motorcycle parking provisions.

Accessibility

Public Transport

The proposed development site is not currently serviced by any bus services.

At present, the site serviced by  Croxley and Moor Park Metropolitan Line stations and is midway between the two. There are footpaths across the Moor that would provide walking or cycling access to Croxley station from the site. Moor Park is currently accessed by people walking along the unsuitable north verge of Tolpits Lane.

The area will soon benefit from the planned Metropolitan Line Extension (formerly known as the Croxley Rail Link) which will connect Watford

Junction and Croxley stations and will include two new stations in west Watford. The nearest, Cassiobridge station, will be some 1500m or 28 minutes walking distance from the application site.

A bus service is proposed as part of the application submission. This  would enhance the sustainability of the Moors site. The proposals are provided in a document with associated costs in the appendix of the TA. The proposed bus routes would be:

- Wolsey Business Park (The Moors) - Rickmansworth station

- Wolsey Business Park (The Moors) - Watford Junction station

The proposed bus services are the same as those submitted as part of a previous application, reference 16/2497/FUL, have been reviewed and bus operator Arriva was contacted to discuss the proposals. It was noted that the fees associated with the cost breakdown in the appendix would need to be amended as they are dated July 2015 and costs will need to be derived to reflect the time when the service would start. It should also be noted that the patronage of the bus service may be affected with the extension of the Metropolitan Line.

HCC are aware, from contacts at the business park, of an interest in improved bus services. At present there is a shuttle bus operated by Camelot for their staff, it is unknown whether this would continue with the implementation of improved bus service. As was previously expressed, HCC are in favour of the Watford service option and the option 2 timetable offering higher frequency services in the AM and PM peak periods which may be attractive to surrounding businesses as well as commuters from the new development. HCC would accept a lower frequency on Saturday/Sunday, i.e. hourly. In relation to viability, the greater the number of units, the greater the potential patronage of the bus service especially for daytime services.

HCC passenger transport liaison team would prefer that the developer’s contribution towards this service were made a payment to them via a S106 agreement. That would enable them to put on something potentially more appropriate to the network at the time and responsive to demand.

Walking and Cycling

At present, the neighbourhood is flat and the applicant considers it walkable. There are no footways on Tolpits Lane. Footways are provided on both sides of Dwight Road from the roundabout junction with Tolpits Lane to the junction with Dwight Road and Dwight Road running east to west has a footway on its south side.

At present, pedestrians from the existing site can access traffic free footpaths that connect to the east, west and north of the site. The footpath routes are as follows:

-  Ebury Way to Rickmansworth and Watford; and,

-  Public footpath Croxley Green 017 across Common Moor to Croxley.

Existing arrangements for cyclists include easy access to the Ebury Way cycle route that forms part of the National Cycle Route 6 which connects from Uxbridge in the south and Watford and St Albans to the northeast. As previously mentioned, this is a traffic-free route. The route would connect cyclists to Uxbridge, Rickmansworth, Watford and St. Albans. Tolpits Lane is not conducive to cycling.

Travel Plan

As part of the TA, the consultant has included a Framework Travel Plan (FTP). The FTP is suitable to provide information to demonstrate that a full Travel Plan (TP) for the residential element of the site and Travel Plan Statements for the office and gym uses, would be implemented. At the appropriate time, a full Travel Plan and Travel Plan Statements would be required to encourage the use of sustainable transport modes and to reduce the reliance on private cars and to ensure minimal impact to the highway safety and function as a consequence of the development. Any TP submitted will have to be in line with HCC’s guidance set out in the County Council’s document ‘Hertfordshire’s Travel Plan Guidance for Business and Residential Development’ as set out at

http://www.hertfordshire.gov.uk/services/transtreets/highways/highwaysinfo/hiservicesforbus/devmanagment/greentravelplans1/.

The FTP has been reviewed, and the following comments should be considered for preparation of the future full Travel Plan(s).

General

-  National and local policy background to be included in Travel Plan

Travel Plan management

-  Travel Plan co-ordinator details to be provided on appointment and details of secondary contact in case of personnel changes

-  An outline of the duties assigned to the co-ordinator, time allocated to the role, frequency on site.

-  A statement from a member of senior management demonstrating commitment to the success of the plan.

-  A Travel Plan group is mentioned - details are needed of potential membership, frequency of meetings

-  Identification of relevant stakeholders/external partners whose involvement will be required for the success of the plan - eg bus operators, cycling groups, cycle shops, car club provider etc

Measures

-  Residential travel pack contribution - discount vouchers mentioned for public transport/cycling/walking - suggested contribution contained in Appendix E of the HCC guidance.

Targets, monitoring and action plan

-  Template to be completed.

-  Baseline mode split data to be established on first survey.

-  Interim modal shift targets - target only provided for reduction in car mode share over whole plan period - need yearly targets or as minimum for years 1, 3 and 5 and give targets for increases in other sustainable modes.

-  Confirmed mode shift targets - to be submitted following initial survey.

-  Monitoring - full surveys required to obtain meaningful numbers of responses.  Multi-modal counts would also be useful to provide a fuller picture in years 1, 3 and 5.

-  Travel plan review - review of travel plan and progress towards targets and identification of additional measures necessary in year 1, 3 and 5.

-  Evaluation and support fee to be provided

Construction

As part of the TA, the applicant has provided an indicative construction programme for consideration and the information provided is considered appropriate. A full Construction Traffic Management Plan will be required to ensure construction vehicles will not have a detrimental impact on the vicinity of the site and a condition will be required to provide adequate parking for construction vehicles on-site to prevent on-street conflict and impacts to the highway safety.

A Construction Traffic Management Plan would be required for all phases of the construction, including demolition, excavation of the substantial basement and construction of all elements of the building (basement car parking levels through to the buildings above. It is expected that for a basement car park of this magnitude, there would be significant trips associated with the excavation phase. Measures would also be required to protect users of the local road network from hazards arising from undue damage caused from its use by large numbers of HGVs associated with the demolition and excavation phases of the project.

Planning Obligations / Community Infrastructure Levy (CIL)

It is the policy of the County and District Councils to seek planning obligations to mitigate the effects of development. HCC’s requirements in respect of highways and transport are set out in section 11 of the document ‘Planning Obligations Guidance - Toolkit for Hertfordshire (Hertfordshire County Council's requirements)’. This can be read and downloaded from http://www.hertsdirect.org/your-council/hcc/resandperf/hertsprop/planningobs/.

A S106 Agreement will be required to secure a Construction Traffic Management Plan Planning Obligation and support for the Travel Plan.

The applicant has proposed the funding of a bus service passing the site. This should be secured by S106 agreement. The S106 agreement should include a contribution towards the provision of a foot and cycle way along the northern verge of Tolpits Lane.

It is noted that draft S106 agreements have been received for the scheme. The agreements are templates with details of the development and the signatories added but no figures for financial contributions inserted and no specific obligations for monitoring fees for Travel Plan monitoring and Construction Traffic Management Plan monitoring. The template for the S106 Agreement will need to be modified to an up to date version at the point where S106 Agreement is under discussion.

Summary

HCC as highway authority has reviewed the application submission and wishes to raise an objection to the development due to overprovision of car parking.

APPLICATION D: [No objection, conditions requested]

Notice is given under article 18 of the Town and Country Planning (Development Management Procedure) (England) Order 2015 that the Hertfordshire County Council as Highway Authority does not wish to restrict the grant of permission subject to the following conditions:

Decision

Hertfordshire County Council as Highway Authority does not wish to raise an objection to the proposed development, subject to suitable conditions.

Conditions

Condition 1: Cinema and Training and Education Land Uses

The land uses ‘Leisure - Cinema’ and ‘Training and Education’ are to be for the sole use of residential and office users, as stated in submitted Transport Assessment application document by Nichols Consulting. Should the Cinema and Training and Education land uses be open to the public, a Transport Assessment will be required to demonstrate the impact of the added traffic on the local highway network.

Reason: In order to protect the highway safety and amenity of other users of the public highway and rights of way.

Condition 2: Construction Traffic Management Plan

Construction of the development hereby approved shall not commence until a Construction Traffic Management Plan has been submitted to and approved in writing by the Local Planning Authority. Thereafter, the construction of the development shall only be carried out in accordance with the approved Plan. The Construction Traffic Management Plan shall include details of:

a. Demolition, excavation and construction vehicle numbers, type, routing by phase (Demolition, Excavation and Construction);

b. Traffic management requirements by phase (Demolition, Excavation and Construction) [Linked to Condition 3];

c. Construction and storage compounds (including areas designated for car parking);

d. Siting and details of wheel washing facilities;

e. Cleaning of site entrances, site tracks and the adjacent public highway;

f. Provision of sufficient on-site parking prior to commencement of construction activities;

g. Post construction restoration/reinstatement of the working areas and temporary access to the public highway.

Reason: In order to protect highway safety and the amenity of other users of the public highway and rights of way.

Condition 3: Construction Traffic - Transport Assessment

Construction of the development hereby approved shall not commence until a Transport Assessment has been submitted to and approved in writing by the Local Planning Authority. The purpose of the Transport Assessment is to ensure that the impact of the construction activity at the site will not have detrimental impact on the safety and operation of the local highway network. The Transport Assessment will assist to establish the necessity for and type of required Traffic Management.

Reason: In order to protect highway safety and the amenity of other users of the public highway and rights of way.

Condition 4: Travel Plan

At least two months prior to first occupation, a Full Travel Plan for the residential element of the development and Travel Plan Statements for the office and gym elements of the development, must be submitted to and approved in writing by the Local Planning Authority and should be prepared in accordance with Hertfordshire’s Travel Plan Guidance. Implementation of the plans shall follow a timescale to be agreed by the Local Planning Authority.

Reason: To promote sustainable transport measures to the development.

Condition 5: Detailed Plans

Prior to the commencement of the development hereby permitted full details in the form of scaled plans and written specifications shall be submitted to and approved in writing by the Local Planning Authority to illustrate the following:

i.  	Roads, footways, foul and on-site water drainage.

ii.	Existing and proposed access arrangements including visibility splays.

iii.  	Parking provision in accordance with adopted standard.

iv.  	Cycle parking provision in accordance with adopted standard.

v.  	Servicing areas, loading areas and turning areas for all vehicles.

Condition 6: Swept Path Assessments

Prior to commencement of the development, the applicant shall submit swept path assessments for the following parts of the proposed development. There are to be submitted to and approved in writing by the Local Planning Authority:

-  Refuse and servicing bays to demonstrate that the largest vehicles accessing the bays can safely access the bays to depart the site in a forward gear;

-  Car parking internal layout to demonstrate that a vehicle can enter, manoeuver through and depart in a forward gear; and,

-  Car parking spaces to demonstrate that all car parking spaces can be safely accessing by a large car.

Reason: In the interests of maintaining safe access to the proposed development by all users.

Condition 7: Delivery and Servicing Plan

Prior to commencement of the development, the applicant shall submit a Delivery and Servicing Plan to be submitted to and approved in writing by the Local Planning Authority. The Delivery and Servicing Plan shall contain the following:

-  The delivery and servicing requirements (including refuse collection) for the proposed uses;

-  A scheme for coordinating deliveries and servicing for the proposed development;

-  Areas within the development site that will be used for loading and manoeuvring of delivery and servicing vehicles; and,

-  Access to / from the site for delivery and servicing vehicles.

Reason: In the interests of maintaining highway efficiency and safety.

Condition 8: Road Safety Audit

Before commencement of any part of the development, a Stage 1 Road Safety is required for all access arrangements to be submitted to and approved in writing by the Local Planning Authority.

Reason: To ensure that the proposed access arrangements are safe and suitable for their intended use.

HIGHWAY INFORMATIVES:

HCC recommend inclusion of the following Advisory Notes (ANs) to ensure that any works as part of this development are carried out in accordance with the provisions of the Highways Act 1980 and other relevant processes.

AN1) Storage of materials: The applicant is advised that the storage of materials associated with the construction of this development should be provided within the site on land which is not public highway, and the use of such areas must not interfere with the public highway. If this is not possible, authorisation should be sought from the Highway Authority before construction works commence. Further information is available via the website http://www.hertsdirect.org/services/transtreets/highways/ or by telephoning 0300 1234047.

AN2) Obstruction of public highway land: It is an offence under section 137 of the Highways Act 1980 for any person, without lawful authority or excuse, in any way to wilfully obstruct the free passage along a highway or public right of way. If this development is likely to result in the public highway or public right of way network becoming routinely blocked (fully or partly) the applicant must contact the Highway Authority to obtain their permission and requirements before construction works commence. Further information is available via the website http://www.hertsdirect.org/services/transtreets/highways/ or by telephoning 0300 1234047.

AN3) Road Deposits: It is an offence under section 148 of the Highways Act 1980 to deposit mud or other debris on the public highway, and section 149 of the same Act gives the Highway Authority powers to remove such material at the expense of the party responsible.  Therefore, best practical means shall be taken at all times to ensure that all vehicles leaving the site during construction of the development are in a condition such as not to emit dust or deposit mud, slurry or other debris on the highway. Further information is available via the website http://www.hertsdirect.org/services/transtreets/highways/ or by telephoning 0300 1234047.

AN4) Construction standards for works within the highway: All works to be undertaken on the adjoining highway shall be constructed to the satisfaction and specification of the Highway Authority, by an approved contractor, and in accordance with Hertfordshire County Council’s publication "Roads in Hertfordshire - Highway Design Guide (2011)". Before works commence the applicant will need to apply to the Highway Authority to obtain their permission and requirements. Further information is available via the website http://www.hertsdirect.org/services/transtreets/highways/ or by telephoning 0300 1234047.

S278 Agreement

Any works within the highway boundary (including alterations to the footway and the proposed site access) will need to be secured and approved via a S278 Agreement with the HCC.

S106 Agreement

A S106 agreement will be required to secure a Construction and Logistics Plan Planning Obligation and support for the Travel Plan.

The S106 agreement should include support of the proposed bus service and a contribution towards the provision of a foot and cycle way along the northern verge of Tolpits Lane linking the site with the Moor Park estate and the underground station within it.

Community Infrastructure Levy (CIL)

The application site is located within Area C where the TRDC CIL charge is nil.  As such no CIL contribution would be required in relation to the proposed development.

Description of the Proposal

The proposal is for the demolition of two 3-storey offices with underground parking to be replaced by a single 7 storey building all with shared underground parking for 943 cars.

The proposed development includes a 7-storey mixed use building. The building will comprise:

-  416 Flats: 38 studio, 204 1-bed, 166 2-bed, and, 8 3-bed.

-  Office Space: 2500sqm

-  Coffee Shop and Amenity Area: 250sqm café;

-  Food Retail: 250sqm;

-  Gym and Fitness Area: 700sqm;

-  Cinema: 300sqm; and,

-  Training and Education Facility: 300sqm;

-  Underground parking with provisions for 943 cars, including 35 disabled spaces and 20 electric car spaces.

Site Description

The development site is located in the Wolsey Business Park at the northwest corner of the eastern junction of Tolpits Lane (A4145) and Dwight Road in Watford. At present, the site is occupied by 2 3-storey buildings. Unit 3, Symbio Point is currently used as a secondary school with a temporary planning permission and Unit 4, York House, is used as office space.

The site is bordered to the east and west by Dwight Road, to the south by Tolpits Lane (A4145) and to the north by the private site access road. Dwight Road is an unclassified local access road with a 30mph speed limit and Tolpits Lane is a principal A-class main distributor road with a 40mph speed limit.

Analysis

As part of the planning application package, the applicant has provided a Transport Assessment (TA) to demonstrate the likely impact of the proposed development on the local highway network. This is as required in Roads in Hertfordshire (section 1 chapter 7).

A Design and Access Statement (DAS) is required for all planning applications that have an impact on the highway, as outlined in Roads in Hertfordshire: Design Guide (section 1 chapter 7). A DAS has been provided for the proposed development and is considered appropriate for the purposes of this planning application.

Policy Review

As part of the Transport Assessment provided for consideration, the applicant has provided evidence of review of the following policy documents:

-  National Planning Policy Framework (2012)

-  National Planning Practice Guidance

-  Hertfordshire County Council (HCC) highway design guide Roads in Hertfordshire 3rd edition

-  Relevant Three Rivers District Council core strategies relating to transport and parking requirements

-  Hertfordshire County Council (HCC) Local Transport Plan 3: 2011-2031

Trip Generation and Distribution

The applicant has provided a Transport Assessment (TA) with the application package for consideration.

Trip Generation

Existing Land Uses

The TA provides existing trip generation profiles considering previously agreed trip rates submitted and agreed as part of planning application submissions for application reference numbers: 15/1281/PDR and 15/1184/PDR. This is considered acceptable.

The TA outlines the existing permitted trip generation profiles for each of the buildings on sites, Units 3 and Unit 4, of the Wolsey Business Park. The lowest approved trip generation profile for each site was used to establish an existing trip generation profile for the total site. This approach is considered acceptable as it presents a worst case scenario when compared to the proposed development.

Unit 3 has 3 permitted uses: Office, Residential and Reach Free School (temporary). The trip profile for each site was provided in the TA and the lowest trip profile was for the permitted residential scheme and is as follows:

-  AM Peak: 6 arrivals, 22 departures giving 28 two-way

-  PM Peak: 19 arrivals,  8 departures giving 27 two-way

Unit 4 has 2 permitted uses: Office and Residential. The trip profile for each site was provided in the TA and the lowest trip profile was for the permitted residential scheme and is as follows:

-  AM Peak: 3 arrivals, 3 departures giving 6 two-way

-  PM Peak: 12 arrivals, 8 departures giving 20 two-way

Therefore, the total trip generation profile for the existing land uses for the site is as follows:

-  AM Peak: 9 arrivals, 25 departures giving 34 two-way

-  PM Peak: 31 arrivals, 16 departures giving 47 two-way

Proposed Land Uses

The trip rates of the proposed development were generated by interrogating TRICS database. This approach is considered acceptable.

The TA used the same TRICS outputs provided as part of the original application submission for application reference 15/1935/FUL. These TRICS outputs were not disputed and are therefore considered acceptable.

The following parameters were used for the TRICS interrogation for all land uses:

-  England only, excluding Greater London;

-  Excluding weekends;

-  Average trip rates adopted; and,

-  Peak Hours 08:00 - 09:00 and 17:00 - 18:00.

The following parameter was used to establish the residential trip generation profile: Residential land use 03 - Residential - C Flats Privately Owned.

The following parameter was used for the TRICS interrogation to establish the office trip generation profile: Employment land use 02 - Employment - A Office.

The following parameter was used for the TRICS interrogation to establish the café trip generation profile:   Café land use 03 - Hotel, Food and Drink - E Roadside food.

The following parameter was used for the TRICS interrogation to establish the retail trip generation profile: Retail land use 01 - Retail - O Convenience Store.

The following parameter was used for the TRICS interrogation to establish the gym trip generation profile:   Leisure - Gym land use 07 - Leisure - K Fitness Club.

The following trips rates are associated with the above TRICS interrogation outputs:

-  Residential:

-  AM Peak: 0.068 in / 0.239 out

-  PM Peak: 0.220 in / 0.096 out

-  Office:

-  AM Peak: 1.232 in / 0.236 out

-  PM Peak: 0.173 in / 1.027 out

-  Café:

-  AM Peak: 3.057 in / 3.057 out

-  PM Peak: 3.806 in / 3.922 out

-  Retail:

-  AM Peak: 7.424 in / 7.348 out

-  PM Peak: 7.576 in / 7.652 out

-  Gym:

-  AM Peak: 0.951 in / 1.302 out

-  PM Peak: 3.040 in / 1.661 out

The trips rates were agreed as part of the original application and are therefore considered acceptable for the purposes of this assessment. The applicant does not provide trip generation analysis for the Leisure - Cinema and Training & Education land uses due to their sole use by the proposed development’s residents. This is considered acceptable, subject to a suitable condition to guarantee that the proposed land uses will remain for the sole use of users of the office and residential land uses.

Using the predicted rates and proposed development areas the associated trips generated by the proposed development at this location would therefore be:

-  Residential:

-  AM Peak: 29 arrivals, 100 departures giving 129 two-way

-  PM Peak: 92 arrivals, 40 departures giving 132 two-way

-  Office:

-  AM Peak: 31 arrivals, 6 departures giving 37 two-way

-  PM Peak: 5 arrivals, 26 departures giving 31 two-way

-  Café:

-  AM Peak: 8 arrivals, 8 departures giving 16 two-way

-  PM Peak: 10 arrivals, 10 departures giving 20 two-way

-  Food Retail:

-  AM Peak: 19 arrivals, 19 departures giving 38 two-way

-  PM Peak: 19 arrivals, 20 departures giving 39 two-way

-  Gym:

-  AM Peak: 7 arrivals, 10 departures giving 17 two-way

-  PM Peak: 22 arrivals, 12 departures giving 34 two-way

Total:

AM Peak: 94 arrivals, 143 departures giving 237 two-way

PM Peak: 148 arrivals, 108 departures giving 256 two- way

The total proposed trip generation profile was compared to the existing trip generation profile to establish the net impact at the site. The net trip generation profile for the site is as follows:

-  AM Peak: 85 arrivals, 118 departures giving 203 two-way

-  PM Peak: 117 arrivals, 92 departures giving 209 two-way

Multi-modal Trip Generation

In addition to the vehicular trip generation profile, the applicant has provided a multimodal trip generation profile for Method of Travel to Work based on 2011 census data for the Moor Park and Eastbury ward. The multi-modal trip generation profile is provided to present the likely methods of travel to / from the site for work purposes. This approach is considered appropriate. The vehicular trips estimated as part of the vehicular trip generation profile for the proposed development were used to extrapolate the number of users by mode. The multi-modal trip generation profile shows a demand for public transport and footway access. Therefore, contributions are being sought to improve connections to the site on foot and cycle and by public transport.

Trip Distribution

Trip distribution assumptions for the proposed development traffic have been included as part of the TA and trip distributions proportions are derived from observed and synthesised turning movements at the Dwight Road / Tolpits Lane roundabout junction and priority junction of Dwight Road with the site access road. This is considered an acceptable approach.

Impact on Highway Network

Junction Assessment

The applicant has provided junction assessments for the priority junction of access road with Dwight Road and the Tolpits Lane and Dwight Road roundabout junction. As requested by HCC in discussions following the original application submission, application reference 15/1935/FUL, the TA includes junction modelling for each junction and assessment where the two junctions are connected. The junction modelling was carried out using industry-standard Junctions 9 software.

It was agreed as part of discussions following the original application that any resubmission would include an assessment of the opening year plus a 5-year horizon year. As part of the TA for this application, the year of opening would be 2020 and the proposed 5year horizon year 2025. This is considered acceptable for the proposed assessment.

The applicant has provided growth rates for the horizon years using industry-standard TEMPRO database. This is considered an appropriate method.

The scenarios modelled as part of the assessment include:

-  Do Nothing 2020: Base traffic 2020;

-  Do Nothing 2025: Base traffic 2025;

-  Do Something 2020: Base traffic 2020 + development traffic; and,

-  Do Something 2025: Base traffic 2025 + development.

The TA summarises the results of both the unlinked and linked junction assessments. The junction model outputs are presented in an appendix of the TA.

The operational capacity of the unlinked junctions was considered using the RFC (Ratio of Flow to Capacity), queuing and delay for consistency with other planning applications; however, Level of Service (LOS) was also considered for the unlinked junction operational capacity results because the linked junction operational capacity assessments could not produce a RFC value for comparison. This is explained in the summary in the TA for the linked junction assessment which states that as the modelling for the linked junctions uses the lane simulation mode, the results do not specify a RFC value for the junctions. Therefore, the operational capacity of the junctions was considered using the Level of Service (LOS), queuing and delay. Therefore, the LOS is considered for both of the junction capacity assessment scenarios. This is considered acceptable for the purposes of the TA.

Typically the Level of Service (LOS) of a junction in the range of A-D are considered to be acceptable, with LOS A/B most desirable and LOS C/D acceptable. A LOS of E would imply that the operation of the junction is beginning to fail and likely to be operating with congestion and delays. A LOS of F implies that the junction has reached or is over its capacity and is likely to be experiencing severe delays and congestion. The thresholds A-F are based on the queuing delay on each arm. The definitions can be summarised as follows:

A = Free flow

B = Reasonably free flow

C = Stable flow

D = Approaching unstable flow

E = Unstable flow

F = Forced or breakdown flow

Please note that the PCU (Passenger Car Unit) is the equivalent of 1 passenger car or 5.75m in length.

Unlinked Junction Capacity Assessments

The junction capacity results for the unconnected Tolpits Lane roundabout with Dwight Road demonstrate that in both the AM and PM peak hours in both horizon years and for the Do-Minimum and Do-Something scenarios, the junction will continue to operate within capacity. The junction is likely to experience a maximum RFC of 0.64 with associated LOS of A, a queue of 1.8 PCUs and a predicted total junction arm delay of 6.5 seconds. This will occur during the 2025 AM peak, Do Something scenario.

The junction capacity results for the unconnected Dwight Road priority T-junction with the site access road demonstrate that in both the AM and PM peak periods in both horizon years and for the Do-Minimum and Do-Something scenarios, the junction is likely to continue to operate within capacity. The junction would experience a maximum RFC of 0.60 with associated LOS of C, a queue of 1.5 PCUs and a predicted total junction arm delay of 18.92 seconds. This would occur during the 2025 PM peak, Do Something scenario.

Linked Junction Capacity Assessments

The LOS for the Tolpits Lane roundabout is predicted to be A for all junction arms in the AM and PM peak hours. Maximum queues experienced at this junction occur in the AM peak for Tolpits Lane East arm with 2.3 PCUs for the 2025 horizon year and a maximum predicted total junction arm delay of 6.46 seconds.

The LOS for the Dwight Road and site access road priority T-junction is predicted to be A for all junction arms for the 2020  and 2025 horizon years in the AM and PM peak hours for the Do-Minimum scenario. The junction will likely have a LOS of A for both the Dwight Road junction arms for the Do-Something scenario in both the AM and PM peaks for both horizon years. The site access junction arm was predicted to operate with a LOS of B for Do-Something scenario in the 2020 and 2025 horizon years for the AM peak; however, will operate with a LOS of C for the Do Something scenario in the 2020 and 2025 horizon years in the PM peak. However, queuing at this junction would not exceed 1.8 PCUs at the junction with a maximum predicted total junction arm delay of 20.49 seconds.

Summary

The results indicate that the junction is likely to operate within desired thresholds with LOS under D,  acceptable levels of queuing and delays for all public highway links.

The junction capacity test results indicate that the additional traffic added to the network as a consequence of the proposed development is not likely to have a detrimental impact on the operation of the highway network in the vicinity of the proposed development.

Highway Layout

Highway Safety

The applicant has provided detailed collision data as part of the TA for the surrounding road network. This approach is considered acceptable.  A review of the 5 years of collision data from 1st April 2011 to 31st March 2016, obtained from HCC reveals that there were 26 collisions resulting in injury in the vicinity of the site, all of which are designated slight collisions. There have been a total of 41 personal injuries resulting from these collisions.

There were 5 locations identified as part of the review where more than one collision occurred, as provided below:

-  A4145 Tolpits Lane, North of Croxley View junction: 3 PICs all human error, failing to look, reckless driving, following too close and loss of control.

-  A4145 Tolpits Lane/Croxley View: 7 PICs, 3 involved pedestrians. 5 collisions involved parking manoeuvres outside shops in the vicinity of the junction. Contributing factors include failing to look properly, reckless driving and wet road surface.

-  A4145 Tolpits Lane/ Chaffinch Lane: 2 PICs. One due to loss of control on wet surface and the other a consequence of driver travelling too close to a stopping vehicle.

-  A4145 Tolpits Lane, west of Dwight Road: 5 PICs, 1 injury to cyclist, 1 to motorcyclist and rest to car users. 4 PICs involved turning manoeuvres at entry to or exit from access roads from Tolpits Lane and were caused by misjudgement of speed or failure to look. 1 PIC involved vehicle overtaking cyclist.

-  A4145 Moor Lane/Sandy Lodge Road: 3 PICs. Contributing factors, failure to look properly, excessive speed, loss of control and slippery road surface.

The review of the collision data in the vicinity of the proposed development site does not present any safety issues with the highway and all collisions appear to have human judgement errors as contributing factors. Therefore, the proposed development is not expected to impact significantl on the safe operation of the local highway.

Vehicle Access

The proposed development would utilise the existing site access road from Dwight Road. The proposed development includes a new access from the site access road, providing access to an underground level which includes underground parking for motorcycles, electric cars and access to the car parking spaces.

The internal layout of the basement level appears to operate under a one-way system. However, the layout at this stage is not finalised and would be agreed as part of reserved matters. Exact dimensions of the parking bays as well as the spacing between them are required to be provided by the applicant at the reserved matters stage. In addition, swept path assessments would be required to demonstrate that cars could enter, manoeuvre through and depart the site in a forward gear.

The HCC DM Implementation team were consulted on the access arrangements for the proposed development site for a similar scheme at this location with the same access arrangements. As the site access road is not adopted by HCC, and not likely to be, so they had no formal comments to make; however, there is concern related to the stop and reverse requirement for an HGV accessing the servicing areas for the proposed development. Therefore, evidence should be provided under the reserved matters to support that the largest servicing vehicles can be access the site safely and will not seek to park on Dwight Road to carry out any refuse collection, servicing or delivery at the site.

Pedestrian Access

It is proposed that the development would utilise the existing site access road and existing pedestrian facilities. It is unclear from the drawings if pedestrian facilities would be provided along the site frontage, adjacent to the site access road. Provisions for safe pedestrian access to the site are required. Additional details with regards to pedestrian access and footway provisions would be required at the reserved matters stage.

Swept Path Analysis

The applicant has not provided swept path assessments for the servicing areas, or access to the car park and car parking spaces. However, as this is an outline application this is considered necesssary and swept path assessments would be required at the reserved matters stage.

Road Safety Audit

A Stage 1 Road Safety Audit would be required to illustrate that the new access arrangements are safe and appropriate for the size of the development. As outlined in the meeting minutes provided in the appendices from a meeting with HCC, the transport consultant and the developer, ‘the execution of a Stage 1 Road Safety Audit as part of the S38/S278 process will be stated in the TA for the resubmission’. The applicant states that a Stage 1 road safety audit for the site access would be conducted under a Section 38 agreement or a Section 278 agreement. This is considered appropriate.

Refuse and Service Delivery

The applicant has stated in the TA that there will be 3 servicing areas for servicing, delivery and collection for the development site. Locations of the refuse store and collection have not been provided as part of the servicing and delivery arrangements. This is required to demonstrate that refuse collection will be in line with standards for refuse collection. A Refuse, Servicing and Delivery Plan is required to ensure that suitable vehicles would be able to access the appropriate bays, etc.  It is stated in the TA that vehicle swept path assessments within the site at ground and basement levels will be submitted under reserved matters. Due to the nature of the application as outline, this is considered acceptable.

Parking

The applicant has stated that the proposed development would provide 943 car parking spaces, including 35 disabled spaces and 20 electric car spaces.

Three Rivers District Council’s parking standards set out the following maximum car parking provisions:

-  Residential: 1.75 space per 1-bed flats, 2-spaces per 2 bed flats and 2.25 spaces for 3-bed flats;

-  Office: 1 space per 30sqm;

-  Café: 1 space per 5sqm of floorspace of dining area plus 3 spaces per 4 employees;

-  Retail: 1 space per 30sqm for retail units of up to 500sqm GFA; and,

-  Gym:  1 space per 22sqm.

The cinema and training facilities would be for the use of residents and occupants of the office space and therefore will not require additional parking.

As the site is located in Zone 4 of the TRDC Accessibility zones, the parking provisions can be within 75-100% of the maximum car parking standards.

According to the Three Rivers District Council’s parking standards, the maximum permitted car parking spaces for the proposed developments has been calculated in the TA to be equal to 949 spaces. However, this is considered an over estimate as the café land use car parking provision was calculated using the whole of the café GFA and should have only considered the dining area. The applicant proposes the provision of 943 car parking spaces, equal to 99.3 per cent of the maximum allowed car parking provision. This is considered acceptable. However, it is ultimately the decision of the LPA to determine the suitability of the proposed car parking provisions.

Parking Layout

The exact dimensions of the proposed parking bays, as well as the spacing between them, has not been provided. As this is an outline planning application, this is considered acceptable. However, as part of reserved matters, the applicant would be required to provide dimensions and suitable swept path assessments demonstrating that vehicles can safely enter and depart from the car parking spaces safely and both enter the site and manoeuvre within it to depart in a forward gear.

Cycle Parking Provisions

The applicant stated in the TA that provision of 486 cycle parking spaces would be included as part of the proposed development.

Three Rivers District Council’s parking standards set out the following minimum cycle parking standards:

-  1 long-term space per 2 residential flats;

-  1 short-term space per 500sqm of GFA office space plus 1 long-term space per 10 full time staff;

-  1 short-term space per 100sqm of café GFA plus 1 l long-term space per 10 maximum staff on site at one time;

-  1 short-term space per 150sqm of retail GFA plus 1 long-term space per 10 maximum staff on site at any time; and,

-  1 space per 25sqm of gym GFA.

The minimum required cycle parking spaces, according to Three Rivers District Council, is  256 assuming staff occupancy of 1 per 25sqm of GFA. Therefore, the provision of 468 cycle parking spaces proposed is considered acceptable. However, it is ultimately the decision of the LPA to determine the suitability of the proposed cycle parking provisions.

The provision of cycling parking spaces would be implemented via the provision of carousel compounds across the proposed development. Additionally, the provision of a cycle club of 60 electric bicycles is proposed in the submitted TA. The effective and sustainable management of the club should be set out as part of the Travel Plan for the proposed scheme.

MotorCycle Parking Provisions

The applicant has proposed 40 motorcycle parking spaces for the proposed development. TRDC standards state that 5% of car parking available should be motorcycle parking. Therefore, with a provision of 943 car parking spaces, 47 motorcycle spaces should be included. Alternatively, if the maximum standards from TRDC are considered at 949 total parking spaces, 48 should be motorcycle spaces. Therefore, 40 motorcycle spaces is below the minimum standards. However, it is ultimately the decision of the LPA to determine the suitability of motorcycle parking provisions.

Accessibility

Public Transport

The proposed development site is not currently serviced by any bus services.

At present, the site serviced by  Croxley and Moor Park Metropolitan Line stations and is midway between the two. There are footpaths across the Moor that would provide walking or cycling access to Croxley station from the site. Moor Park is currently accessed by people walking along the unsuitable north verge of Tolpits Lane.

The area will soon benefit from the planned Metropolitan Line Extension (formerly known as the Croxley Rail Link) which will connect Watford

Junction and Croxley stations and will include two new stations in west Watford. The nearest, Cassiobridge station, will be some 1500m or 28 minutes walking distance from the application site.

A bus service is proposed as part of the application submission. This would enhance the sustainability of the Moors site. The proposals are provided in a document with associated costs in the appendix of the TA. The proposed bus routes would be:

- Wolsey Business Park (The Moors) - Rickmansworth station

- Wolsey Business Park (The Moors) - Watford Junction station

The proposed bus services are the same as those submitted as part of a previous application, reference 16/2497/FUL, have been reviewed and bus operator Arriva was contacted to discuss the proposals. It was noted that the fees associated with the cost breakdown in the appendix would need to be amended as they are dated July 2015 and costs will need to be derived to reflect the time when the service would start. It should also be noted that the patronage of the bus service may be affected with the extension of the Metropolitan Line.

HCC are aware, from contacts at the business park, of an interest in improved bus services. At present there is a shuttle bus operated by Camelot for their staff, it is unknown whether this would continue with the implementation of improved bus service. As was previously expressed, HCC are in favour of the Watford service option and the option 2 timetable offering higher frequency services in the AM and PM peak periods which may be attractive to surrounding businesses as well as commuters from the new development. HCC would accept a lower frequency on Saturday/Sunday, i.e. hourly. In relation to viability, the greater the number of units, the greater the potential patronage of the bus service especially for daytime services.

HCC passenger transport liaison team would prefer that the developer’s contribution towards this service were made a payment to them via a S106 agreement. That would enable them to put on something potentially more appropriate to the network at the time and responsive to demand.

Walking and Cycling

At present, the neighbourhood is flat and the applicant considers it walkable. There are no footwayson Tolpits Lane. Footways are provided on both sides of Dwight Road from the roundabout junction with Tolpits Lane to the junction with Dwight Road and Dwight Road running east to west has a footway on its south side.

At present, pedestrians from the existing site can access traffic free footpaths that connect to the east, west and north of the site. The footpath routes are as follows:

-  Ebury Way to Rickmansworth and Watford; and,

-  Public footpath Croxley Green 017 across Common Moor to Croxley.

Existing arrangements for cyclists include easy access to the Ebury Way cycle route that forms part of the National Cycle Route 6 which connects from Uxbridge in the south and Watford and St Albans to the northeast. As previously mentioned, this is a traffic-free route. The route would connect cyclists to Uxbridge, Rickmansworth, Watford and St. Albans. Tolpits Lane is not conducive to cycling.

Travel Plan

As part of the TA, the consultant has included a Framework Travel Plan (FTP). The FTP is suitable to provide information to demonstrate that a full Travel Plan (TP) for the residential element of the site and Travel Plan Statements for the office and gym uses, would be implemented. At the appropriate time, afull Travel Plan and Travel Plan Statements will be required to encourage the use of sustainable transport modes and to reduce the reliance on private cars and to ensure minimal impact to the highway safety and function as a consequence of the development. Any TP submitted will have to be in line with HCC’s guidance set out in the County Council’s document ‘Hertfordshire’s Travel Plan Guidance for Business and Residential Development’ as set out at

http://www.hertfordshire.gov.uk/services/transtreets/highways/highwaysinfo/hiservicesforbus/devmanagment/greentravelplans1/.

The FTP has been reviewed, and the following comments should be considered for preparation of the future full Travel Plan(s).

General

-  National and local policy background to be included in Travel Plan

Travel Plan management

-  Travel Plan co-ordinator details to be provided on appointment and details of secondary contact in case of personnel changes

-  An outline of the duties assigned to the co-ordinator, time allocated to the role, frequency on site.

-  A statement from a member of senior management demonstrating commitment to the success of the plan.

-  A Travel Plan group is mentioned - details are needed of potential membership, frequency of meetings

-  Identification of relevant stakeholders/external partners whose involvement will be required for the success of the plan - eg bus operators, cycling groups, cycle shops, car club provider etc

Measures

-  Residential travel pack contribution - discount vouchers mentioned for public transport/cycling/walking - suggested contribution contained in Appendix E of the HCC guidance.

Targets, monitoring and action plan

-  Template to be completed.

-  Baseline mode split data to be established on first survey.

-  Interim modal shift targets - target only provided for reduction in car mode share over whole plan period - need yearly targets or as minimum for years 1, 3 and 5 and give targets for increases in other sustainable modes.

-  Confirmed mode shift targets - to be submitted following initial survey.

-  Monitoring - full surveys required to obtain meaningful numbers of responses.  Multi-modal counts would also be useful to provide a fuller picture in years 1, 3 and 5.

-  Travel plan review - review of travel plan and progress towards targets and identification of additional measures necessary in year 1, 3 and 5.

-  Evaluation and support fee to be provided

Construction

As part of the TA, the applicant has provided an indicative construction programme for consideration and the information provided is considered appropriate. A full Construction Traffic Management Plan will be required to ensure construction vehicles will not have a detrimental impact on the vicinity of the site and a condition will be required to provide adequate parking for construction vehicles on-site to prevent on-street conflict and impacts to the highway safety.

A Construction Traffic Management Plan would be required for all phases of the construction, including demolition, excavation of the substantial basement and construction of all elements of the building (basement car parking levels through to the buildings above. It is expected that for a basement car park of this magnitude, there would be significant trips associated with the excavation phase. Measures would also be required to protect users of the local road network from hazards arising from undue damage caused from its use by large numbers of HGVs associated with the demolition and excavation phases of the project.

Planning Obligations / Community Infrastructure Levy (CIL)

It is the policy of the County and District Councils to seek planning obligations to mitigate the effects of development. HCC’s requirements in respect of highways and transport are set out in section 11 of the document ‘Planning Obligations Guidance - Toolkit for Hertfordshire (Hertfordshire County Council's requirements)’. This can be read and downloaded from http://www.hertsdirect.org/your-council/hcc/resandperf/hertsprop/planningobs/.

A S106 Agreement will be required to secure a Construction Traffic Management Plan Planning Obligation and support for the Travel Plan.

The applicant has proposed the funding of a bus service passing the site. This should be secured by S106 agreement. The S106 agreement should include a contribution towards the provision of a foot and cycle way along the northern verge of Tolpits Lane.

It is noted that draft S106 agreements have been received for the scheme. The agreements are templates with details of the development and the signatories added but no figures for financial contributions inserted and no specific obligations for monitoring fees for Travel Plan monitoring and Construction Traffic Management Plan monitoring. The template for the S106 Agreement will need to be modified to an up to date version at the point where S106 Agreement is under discussion.

Summary

HCC as highway authority has reviewed the application submission and raise no objection of the development, subject to the imposition of suitable planning conditions.

Housing Development Officer:

APPLICATION A

Policy CP4 of the Adopted Core Strategy requires 45% of new housing to be provided as Affordable Housing, unless it can be clearly demonstrated with financial evidence that this is not viable. As a guide the tenure split should be 70% social rented and 30% intermediate.

Policy CP3 of the adopted Core Strategy (2011) sets out the proportions that should form the basis for housing mix in development proposals submitted to Three Rivers District Council. Proposals should broadly be for 30% 1-bed units, 35% 2-bed units, 34% 3-bed units and 1% 4+ bed units.

However, identified need for affordable housing suggests the following preferred mix: 22% 1-bed units, 50% 2-bed units, 24% 3 bed units and 4% 4 + bed units.  This mix only applies to the sites affordable units.

As per the proposed affordable housing statement, 4.01% has been offered. Also the statement indicates that the 11 dwellings will be provided on a 23% socially rented and 77% as intermediate social housing on a shared ownership scheme. I therefore would like these reconsidered, as per our policy. A cash S106 contribution equivalent to the social housing ratio to be agreed by TRDC and the applicant would be considered.

APPLICATION B

Policy CP4 of the Adopted Core Strategy requires 45% of new housing to be provided as Affordable Housing, unless it can be clearly demonstrated with financial evidence that this is not viable. As a guide the tenure split should be 70% social rented and 30% intermediate.

Policy CP3 of the adopted Core Strategy (2011) sets out the proportions that should form the basis for housing mix in development proposals submitted to Three Rivers District Council. Proposals should broadly be for 30% 1-bed units, 35% 2-bed units, 34% 3-bed units and 1% 4+ bed units.

However, identified need for affordable housing suggests the following preferred mix: 22% 1-bed units, 50% 2-bed units, 24% 3 bed units and 4% 4 + bed units.  This mix only applies to the sites affordable units.

As per the proposed affordable housing statement, 6.02% has been offered. Also the statement indicates that the 20 dwellings will be provided on a 23% socially rented and 77% as intermediate social housing on a shared ownership scheme. I therefore would like these reconsidered, as per our policy.

APPLICATION C

Policy CP4 of the Adopted Core Strategy requires 45% of new housing to be provided as Affordable Housing, unless it can be clearly demonstrated with financial evidence that this is not viable. As a guide the tenure split should be 70% social rented and 30% intermediate.

Policy CP3 of the adopted Core Strategy (2011) sets out the proportions that should form the basis for housing mix in development proposals submitted to Three Rivers District Council. Proposals should broadly be for 30% 1-bed units, 35% 2-bed units, 34% 3-bed units and 1% 4+ bed units.

However, identified need for affordable housing suggests the following preferred mix: 22% 1-bed units, 50% 2-bed units, 24% 3 bed units and 4% 4 + bed units.  This mix only applies to the sites affordable units.

As per the proposed affordable housing statement, 10.17% has been offered.  I therefore would like this reconsidered, as per our policy.  Also the statement makes reference to the Local Authority having a need for 3 and 4 bedroom units.  Please be aware that the main requirement is for 2 bed 4 person units.

APPLICATION D

Policy CP4 of the Adopted Core Strategy requires 45% of new housing to be provided as Affordable Housing, unless it can be clearly demonstrated with financial evidence that this is not viable. As a guide the tenure split should be 70% social rented and 30% intermediate.

Policy CP3 of the adopted Core Strategy (2011) sets out the proportions that should form the basis for housing mix in development proposals submitted to Three Rivers District Council. Proposals should broadly be for 30% 1-bed units, 35% 2-bed units, 34% 3-bed units and 1% 4+ bed units.

However, identified need for affordable housing suggests the following preferred mix: 22% 1-bed units, 50% 2-bed units, 24% 3 bed units and 4% 4 + bed units.  This mix only applies to the sites affordable units.

As per the proposed affordable housing statement, 15.86% has been offered. Also the statement indicates that the 66 dwellings will be provided on a 23% socially rented and 77% as intermediate social housing on a shared ownership scheme. I therefore would like these reconsidered, as per our policy.

Housing Manager: No comments received.

Herts Archaeology: [Comments for all applications, no objection]

The development is large in scale and close to Area of Archaeological Significance 15, as identified in the Local Plan, an area with a considerable number of prehistoric, Roman and Medieval sites and findspots.

The development itself, however, is located within the footprint of a former asbestos factory, and an Environmental Search submitted with the application stated that the property 'may constitute "contaminated land" as defined by Part2A of the Environmental Protection Act 1990' (P. 3).  A desk-top contamination report by Brownfield Solutions Ltd also submitted with this planning application says that ground levels have been reduced across the site to remove asbestos contamination. The report also says that further asbestos contamination may still be present (paragraph 3.5.1).

Therefore on the basis of the above information saying that the site is contaminated and has been the subject of decontamination works including ground reduction, as well as the impact of the existing buildings on the site, in this instance I consider that the development is unlikely to have a significant impact on heritage assets of archaeological interest, and I have no comment to make upon the proposal.

Herts Property Services: [Comments for all applications, no comment to make]

Herts Property Services do not have any comments to make in relation to financial contributions required by the Toolkit, as this development is situated within Three Rivers' CIL Area C and does not fall within any of the CIL Reg123 exclusions.  Notwithstanding this, we reserve the right to seek Community Infrastructure Levy contributions towards the provision of infrastructure as outlined in your R123 List through the appropriate channels.

HCC Flood Risk Management Team: [Comments for all applications, no objection subject to conditions]

Following a review of the Flood Risk and Hydrogeology Assessment carried out by Envirocentre reference 7135 dated April 2016 and the Drainage Strategy reference 7131 dated April 2016, we can confirm that we have no objection in principle on flood risk grounds and advise the LPA that the proposed development site can be adequately drained and mitigate any potential existing surface water flood risk if carried out in accordance with the overall drainage strategy.

The proposed drainage strategy is based upon attenuation and discharge into Thames Water sewer restricted to 10l/s with the required storage volumes 720m3 to be provided on site. We acknowledge that Thames Water have been contacted and have no objection in principle to the scheme.

As the proposed scheme has yet to provide the final detail and in order to secure the principles of the current proposed scheme we recommend the following planning condition to the LPA should planning permission be granted:

LLFA position

We consider that outline planning permission could be granted to the proposed development if the following planning condition is included as set out below.

Condition

No development approved by this planning permission shall take place until a detailed surface water drainage scheme has been submitted to, and approved in writing by, the local planning authority. The surface water drainage system will be based on the submitted Flood Risk and Hydrogeology Assessment carried out by Envirocentre reference 7138 dated April 2016 and the Drainage Strategy reference 7134 dated April 2016

The surface water drainage scheme should include;

1.	Limiting the surface water run-off generated by the 1 in 100 year + climate change critical storm so that it will not exceed the run-off from the undeveloped site and not increase the risk of flooding off-site.

2.	Providing attenuation to ensure no increase in surface water run-off volumes for all rainfall events up to and including the 1 in 100 year + climate change event

3.	Implementing the appropriate drainage strategy based on attenuation and discharge, using appropriate SuDS measures.

4.	Detailed surface water calculations and modelling for all rainfall events up to and including the 1 in 100 year + climate change event.

5.	Details of how the scheme shall be maintained and managed after completion.

6.	Detailed engineered drawings of the proposed SuDS measures

The scheme shall be fully implemented and subsequently maintained, in accordance with the timing / phasing arrangements embodied within the scheme, or within any other period as may subsequently be agreed, in writing, by the local planning authority.

Reason

1.	To prevent flooding by ensuring the satisfactory storage of/disposal of surface water from the site.

2.	To reduce the risk of flooding to the proposed development and future users.

Although we are satisfied at this stage that the proposed development could be allowed in principle, the applicant will need to provide further information to ensure that the proposed development can go ahead without posing an unacceptable flood risk.

Informative to the LPA

For further guidance on HCC's policies on SuDS, HCC Developers Guide and Checklist and links to national policy and industry best practice guidance please refer to our surface water drainage webpage

http://www.hertfordshire.gov.uk/services/envplan/water/floods/surfacewaterdrainage/

HCC Footpath: No comments received.

HCC Waste & Minerals: No comments received.

Herts Ecology: [Comments for all applications, no objection subject to conditions]

The application site sits adjacent to the southern boundary of the Croxley Common Moor SSSI and LNR, and I am aware that there are outstanding issues regarding potential increase in recreational use and as a commuting route to the nearby Croxley Underground Station. These issues will need to be resolved in consultation with Natural England who are the statutory body ultimately responsible for the SSSI.

South of the application site is the Hamper Mill Lakes Local Wildlife Site (LWS). This area has been designated for is mosaic of habitats including open and running water, semi-improved neutral grassland and marginal habitats. There are also records of rare species such as bee orchids. As with the SSSI/LNR a building of this many residential dwellings could see an increase in recreational pressure and the number of dogs and cats present within the area which could impact both the habitats and some of the species present. Careful consideration should be taken by the applicant on how they intend to limit potential impacts on this area.

At previous points when similar applications at this location have been submitted Hertfordshire Ecology has advised that a biodiversity sympathetic lighting scheme is necessary considering the habitats to the north and south of the application site. A lighting report has been submitted with the application but no lighting plan. The lighting report still discusses a ‘glow in the dark’ footpath across Croxley Common which I believe has now been dropped from the plans. I would therefore recommend that the following condition be included with any planning decision.

Prior to occupation a “lighting design strategy for biodiversity” for features or areas to be lit, shall be submitted to and approved in writing by the local planning authority. The strategy shall:

a) Identify those areas/features on site that are particularly sensitive for nocturnal species and that are likely to cause disturbance in or around their breeding sites and resting places or along important routes used to access key areas of their territory, for example, for foraging; and

b) show how and where external lighting will be installed (through the provision of appropriate lighting contour plans and technical specification) so that it can be clearly demonstrated that areas to be lit will not disturb or prevent the above species using their territory or having access to their breeding sites and resting places.

All external lighting shall be installed in accordance with the specifications and locations set out in the strategy, and these shall be maintained thereafter in accordance with the strategy. Under no circumstances should any other external lighting be installed without prior consent from the local planning authority.

Herts & Middlesex Wildlife Trust: No comments received.

Fire Protection Department: [Advisory comments that are applicable for all applications]

We have examined the application and make the following comments:

ACCESS AND FACILITIES

1. Access for fire fighting vehicles should be in accordance with The Building Regulations 2010 Approved Document B (ADB), section B5, sub-section 16.

2. Access routes for Hertfordshire Fire and Rescue Service vehicles should achieve a minimum carrying capacity of 18 tonnes.

3. Turning facilities should be provided in any dead-end route that is more than 20m long. This can be achieved by a hammer head or a turning circle designed on the basis of Table 20 in section B5.

WATER SUPPLIES

4. Water supplies should be provided in accordance with BS 9999.

5. This authority would consider the following hydrant provision adequate:

 Not more than 60m from an entry to any building on the site.

 Not more than 120m apart for residential developments or 90m apart for commercial developments.

 Preferably immediately adjacent to roadways or hard-standing facilities provided for fire service appliances.

 Not less than 6m from the building or risk so that they remain usable during a fire.

 Hydrants should be provided in accordance with BS 750 and be capable of

providing an appropriate flow in accordance with National Guidance documents.

 Where no piped water is available, or there is insufficient pressure and flow in

the water main, or an alternative arrangement is proposed, the alternative source of supply should be provided in accordance with ADB Vol 2, Section B5, Sub section 15.8.

6. In addition, buildings fitted with fire mains must have a suitable hydrant sited within 18m of the hard standing facility provided for the fire service pumping appliance.

The comments made by this Fire Authority do not prejudice any further requirements that may be necessary to comply with the Building Regulations.

Herts Constabulary: [Advisory comments for all applications]

Comments:

- Employment site:  The site is a site for various commercial employments.  The proposal of this application is as regards residential, and due to the number of bedrooms for some flats there will be children and youths residential on site.  With such a substantial residential development on what in effect is a factory / warehouse / office estate means that youths and children from the proposed residential will likely explore the estate and be a cause for damage, ASB, etc to the commercial premises. Also such commercial sites have large lorries manoeuvring and youths / children on cycles will need to be particularly careful, to avoid conflict with such HGV movements.

- Basement Parking:

  - Underground / basement car parking will be problematic if not adequately secured and users will feel vulnerable.  It was not clear from the plans if any form of access control to this area from the public domain is planned?      A barrier arm would not be sufficient and the entrance exit should be controlled by a visually permeable roller shutter or similar.  The shutters should start to close within 5 seconds of vehicles leaving or entering the parking area, so as to prevent unauthorised persons tailgating into the parking area or on foot sneaking into the parking area when a vehicle leaves.  The sensor that detects motion to close the gate should also be mounted on the inside of the gates to prevent tampering by offenders. If the car park were left open I would object due to the likelihood of crime.

  - Painting the walls and ceiling white of the underground car parking area can greatly aid the use of CCTV, and provide good even lighting, which reduces the fear of crime. Lighting levels should be to the appropriate British Standard. The bottom 0.5m of wall can be painted black or dark grey so exhaust fumes do not mark the wall.

  - Stair cores from the parking area to the residential blocks should be secure with doors to BS Pas 24:2012 with access control which only residents can use these doors to gain access.  This is to prevent offenders forcing open these doors if they are able to tailgate their way into the undercroft parking area.

- Physical Security – ADQ and SBD:

In  October 2015, Approved Document Q (ADQ) came into force that requires under Building Regulations dwellings are built to “Prevent Unauthorised Access”.  This applies to any “dwelling and any part of a building from which access can be gained to a flat within the building”.   Achieving the Secured by Design (SBD) award meets the requirements of Approved Document Q (ADQ), and there is no charge for applying for the Secured by Design award.    Further details are available from Hertfordshire Police Crime Prevention Design Advisors at 01707-355226.
I would obviously be keen to see any development built to the physical security standards of Secured by Design which is the police approved minimum security standard, as this will reduce the potential for burglary by 50% to 75% and therefore demand on the Police as well as achieving ADQ.

I hope the above is of use to you in your deliberations and will help the development achieve that aims of the National Planning Policy Framework (NPPF).

- 69 – re safe and accessible environments where crime and disorder, and the fear of crime, do not undermine quality of life or community cohesion.

& the National Planning Practice Guidance (NPPG) under ‘Design’

- 010 – re Sec 17 of the Crime and Disorder Act 1998 – to prevent crime & disorder.

- 011 – re taking proportionate security measures being a central consideration to the planning and delivery of new developments and substantive retrofits.

And CP1 and CP12 of Three Rivers Core Strategy.

Environment Agency: No comments received.

Environmental Protection: No comments received.

Integrated Accommodation Commission: No comments received.

NHS Herts Valleys: No comments received.

Landscape Officer: [Comment for all applications, objection]

My comments are much the same as made on the applications 15/1935/FUL and 16/2497/FUL and can be applied here.

I note the reduction in storeys however, the proposals would still be the tallest buildings in the vicinity. I have read the Biodiversity Officer’s comments and fully support all the raised points in the report. Any threat to a Site of Special Scientific Interest (SSSI) and Local Nature Reserve is unacceptable. The light pollution, footfall, and the possible future pressure on surfaced paths within the SSSI site would pose a threat to the rare and threatened Red Book species on the site and a further threat to the SSSI site losing its status. Considering DEFRA have highlighted that 60% of SSSI sites are poorly managed, Croxley Common Moor is managed to high conservation standard and has community participatory involvement from The Friends of Croxley Common Moor.

Due to the above reasons I cannot support this outline application.

Herts Public Health: No comments received.

Highways Agency: [Comment for all applications, no objection]

Notice is hereby given that Highways England’s formal recommendation is that we offer no objection.

Natural England: No comments received.

Sustainable Projects Officer: No comments received.

Leisure Development Officer: [Advisory comments for all applications]

After full consideration of all of the documents relating to the above planning application, please note the following comments:

- There are no clear plans for leisure facilities. Although there are concept designs, these do not show a planned designed layout with details of the specifications of planned features.

- Particular attention should be given to DDA compliant access to any leisure areas including pathways, seating and equipment.

- Where leisure provision is made by the developers, design advice should be sought from Three Rivers Leisure team.

- Please consider vehicle access for maintenance of any leisure areas.

- Operator signs for any leisure areas will need to be included for reporting of maintenance or reparation issues.

- A full Risk assessment of any leisure equipment should be considered.

- A RoSPA report or assessment should take place of any proposed play and/or leisure equipment.

- Provision for the on-going maintenance of any leisure facilities should be detailed, particularly if it the developer plans to formally hand over to the leisure facility to the Parish Council or Three Rivers District Council.

These comments are given to help the development achieve the aims of Three Rivers District Council’s Local and Strategic plans and National Policy Framework sections detailed below:

- Achieving Sustainable Development paragraph 6-10,

- Promoting Healthy Communities paragraphs 69, 70, 73

- Health and Wellbeing paragraph 171

Local Plans:

APPLICATION A

Representation: The adopted Core Strategy under Policy CP6: Employment and Economic Development seeks to support the sustainable growth of the Three Rivers economy by continuing to focus employment use within the key employment areas that are located in the District. Criterion (j) of this policy identifies Tolpits Lane as a key employment area within Three Rivers. Criterion (n) of this policy states that office space will be considered for release from employment use where a surplus to employment needs across the plan period is expected, as indicated by an up-to-date Employment Land Study.

The South West Herts Economic Study (published in February 2016) indicates that there will be an estimated total jobs growth in the District of 8,400 during the period 2013-2036. 3,600 or 43% is estimated to require ‘B class’ employment space that includes office accommodation. This roughly equates to an estimated 7.1ha of additional land that is needed for office space within District, or just under 60,000sqm of office space during this period.

The adopted Site Allocations LDD formally identifies Tolpits Lane as an employment site allocation (site ref: E(b)). The site is not allocated as a mixed use site as claimed in the submitted Design and Access Statement on p.39. This allocation is supported by Policy SA2: Employment Site Allocations, which states that: ‘allocated employment sites will be safeguarded for business, industrial and storage or distribution uses.’

In addition to the above, and in recognition of the need to protect employment sites, the Council made a non-immediate Article 4(1) Direction on 5 August 2016 which removes the permitted development rights for change of use from office to residential and light industrial to residential within the Tolpits Lane Employment Area. This direction was confirmed on 13 October 2016 and is due to come into force on 5 August 2017 (office to residential) and 30 September 2017 (light industrial to residential). The Council considers the Article 4(1) Direction to be necessary to protect this important location for employment within the District and to secure the viability of businesses within the employment area.

Due to the site’s formal allocation for employment uses and the need to allocate further land for ‘B class’ jobs growth within the District, Tolpits Lane Employment Site is not considered suitable for residential.

The Design & Access Statement on p.155 states that the existing office floorspace is 51,000ft2 (4738.06m2) and will be replaced with 24,017.07ft2 (2231.26m2) which is a net loss of 2506.8m2. However, this is contradicted in the conclusions section on p.196, which states that the current office employment space is only 17,000ft2 (1579.35m2). The CIL Forms submitted with the application states that the existing office floorspace is 5,837.86m2 Although the form states there will be no loss of non-residential floorspace, the total gross internal floorspace proposed for non-residential will be 2,231.24m2 which is a reduction of 3,606.62m2. The proposals will result in the net loss of office floorspace and are therefore contrary to Policy SA2 of the Site Allocations LDD and Policy CP6 ((j) and (n)) of the Core Strategy .

As stated above the site is not allocated for housing in the Site Allocations LDD and does not form part of the District’s housing supply and must therefore be considered as a windfall housing site. Policy CP2 of the Core Strategy states that applications for windfall sites will be considered on a case by case basis having regard to:

- The location of the proposed development, taking into account the Spatial Strategy

- The sustainability of the development and its contribution to meeting local housing needs

- Infrastructure requirements and the impact on the delivery of allocated housing sites

- Monitoring information relating to housing supply and the Three Rivers housing target.

The Spatial Strategy states that new development will be directed towards previously developed land and appropriate infilling opportunities within the urban areas of the Principal Town (Rickmansworth) and Key Centres (South Oxhey, Croxley Green, Abbots Langley, Chorleywood, Leavesden and Garston and Mill End) which have been identified as the most sustainable locations within the District. More limited levels of new development will be directed towards Secondary Centres (Kings Langley, Carpenders Park, Eastbury, Oxhey Hall, Maple Cross and Moor Park). Place shaping policies PSP1, PSP2 and PSP3 set out the proportion of development that will be directed towards each.

The proposed site is not within or close to the boundaries of the Key Town, Key Centres or Secondary Centres identified by the Core Strategy. Essentially, Tolpits Lane Employment Area is an ‘out of town’ employment area and is in a relatively isolated location. The nearest station (Croxley Green) is just over 1km away as the crow flies, but in reality a walk to the station will be over 3km and involve walking along Tolpits Lane (which has no footpaths and no street lighting along stretches), a cycle path and then across a main carriageway. An alternative route of just over 1.1km (which was suggested by the applicant in a previous application), would involve walking through the employment area and across Croxley Common Moor (which is a Site of Special Scientific Interest and a Local Nature Reserve) where there is a public right of way but with no formal footpath and across two foot bridges over the canal.

Moor Park Station is approximately 1.8km away as the crow flies. Access to Moor Park Station as suggested by the applicants is via Tolpits Lane (no pavements or dedicated cycle route) and Sandy Lodge Road (a private road and often barrier controlled) on the private Moor Park Estate and would be 2.9km in distance. Realistically car access would be through the southern side of Moor Park via Astons Road off Batchworth Lane resulting in a car journey of 6.9km.

The Design & Access Statement states on p.87 that the proposed Vicarage Road Station (due to be functional in 2020) will be situated at the junction of Vicarage Road and Occupation Road some 1.8 miles (2.9 km) away and have provided indicative travel times by car, bicycle and walking. However, the location of a local car MOT service station has mistakenly been used for their calculations.

The proposed Vicarage Road Station will actually be situated closer to the proposed development site just off Hagden Lane approximately 1.4km away as the crow flies. The journey by car will be approximately 1.8 km, the walking distance would be the same. However walking to this station will involve walking along part of Tolpits Lane which has no footpaths. For cyclists, the same direct route could be used but there are no cycle paths. There is a suggestion that cyclists could use the Ebury Way (cycle route), however this is unlikely to be used for access to Vicarage Road Station as it would mean a travelling a distance of approximately 2.6km.

The proposed Cassiobridge Station (due to be functional in 2020) will be situated off Ascot Road and the most direct pedestrian and cyclist route (1.5km) would be along Tolpits Lane (where there are no footpaths) and then through the use of a cycle path.  The journey by car however would be substantially longer at a distance of 2.7km.

The site is not serviced by public bus routes, Tolpits Lane itself lacks footpaths and there is no specific provision for cyclists on the highway. The site is isolated from key services including schools and health care facilities and does not have easy access to existing community facilities and retail options. The nearest bus stop is nearly a mile away.

The proposed location for this residential development is considered unsustainable and contrary to the objectives set out in the Place Shaping Policies (PSP1, PSP2 and PSP3) and Policy CP1 (Overarching Policy on Sustainable Development) of the Core Strategy. Three Rivers District currently has identified a 9.8 year supply of housing land that meets the definition of deliverability set out in the NPPF and therefore the proposed residential development of 274 apartments is not required to meet the District Housing Targets.

It is therefore not considered that the proposal can be justified on the basis of the criteria contained in Policy CP2 in relation to windfall sites.

Policy CP10 (Transport and Travel), states that all development proposals should be designed and located to minimise the impacts of travel by motor vehicle. In particular, major development will be expected to be located in areas highly accessible by the most sustainable modes of transport, and to all people of abilities in a socially inclusive and safe manner in accordance with the following user hierarchy:

- Pedestrians, particularly people with restricted mobility

- Cyclists and where appropriate horse riders

- Public transport

- All forms of motor vehicle

The location of the proposal, the lack of footpaths and access to public transport (Bus and rail) result in the proposal being contrary to Policy CP10 of the Core Strategy.

The Transport Assessment on p.7 sets out that they (R Square Properties) and Arriva ‘is currently working closely with Arriva’ to provide a bus service and have sought views from HCC on their preferred route from the following two options:

- Wolsey Business Park to Watford Junction

- Wolsey Business Park to Rickmansworth Station

The Transport Assessment contains indicative timetables with bus services generally every 30 minutes. It is also stated (third paragraph) that RSquare Properties undertakes to underwrite the portion of the costs that is not covered by revenue from the first day of occupation of The Moors for a period of 5 years after The Moors has reached 85% occupation on one of the services.

From the email trail provided in the Transport Assessment, HCC have provided comments to say that the Wolsey Business Park to Watford Junction Service would be the most sustainable option but there is doubt that there would be sufficient demand to justify more than an hourly off peak and Saturday service.

Consideration may have to be given as to whether the ‘one’ bus service (either from Wolsey Business Park to Watford Junction or Wolsey Business Park to Rickmansworth Station), would address the accessibility issues to the site as set out above.  However, in the absence of a completed Section 106 Agreement and written confirmation from Arriva, confirming a preferred bus route and timetable, I am unable to consider this at this time.

Policy DM4 of the Development Management Policies LDD requires applicants to demonstrate that development proposals will produce 5% less carbon dioxide emissions than Building Regulations Part L requirements (2013). The information provided shows that the development proposals would meet these requirements and therefore meets Policy DM4. However, the development’s proposed sustainability credentials in relation to energy use and carbon emissions does not mean that the proposal meets the definition of sustainable development as set out in the NPPF. Paragraph 7 of the NPPF sets out that there are three dimensions to sustainable development: economic, social and environmental.

Housing Mix

Policy CP3 of the Core Strategy states that ‘The Council will require housing proposals to take into account the range of housing needs, in terms of size and type of dwellings as identified by the Strategic Housing Market Assessment and subsequent updates.’ The most recent SHMA was published in February 2016 and has identified the indicative targets for market and affordable sectors’ dwelling size within the Three Rivers District as follows:

1 bedroom 7.7% of dwellings

2 bedrooms 27.8% of dwellings

3 bedrooms 41.5% of dwellings

4+ bedrooms 23.0% of dwellings

The development proposes a total of 274 apartments which according to the application form would provide:

1-bed units 58% (160)

2-bed units 39% (106)

3-bed units 3% (8)

4-bed units 0% (0)

The proposed mix is not in accordance with Policy CP3 which seeks a much higher percentage of 3-bed and 4 bed units and a lower percentage of 1-bed and 2-bed units. Whilst it is possible to consider a variation to the percentages set out in Policy CP3 on a case by case basis, no evidence/justification has been provided to support the proposed housing mix.

Affordable Housing

Policy CP4 of the adopted Core Strategy (2011) requires 45% of all new housing to be provided as Affordable Housing, unless it can be clearly demonstrated with financial evidence that this is not viable. As a guide the tenure split should be 70% social rented and 30% intermediate.

According to the Affordable Housing Statement, the development proposes a total of 11 affordable units (4%) with a tenure split of (6 x 1 bed and 5 x 2 bed) to be provided on the basis of 100% socially rented.

Where non-viability is cited as the reason for a development proposal not complying with affordable housing requirements, applicants must submit financial evidence (viability appraisal) to justify this position. Subject to the independent review of the submitted Viability Report, Local Plans reserves comment as to whether the proposals meet Policy CP4 of the Core Strategy.

The development will be replacing two 3 storey buildings in an ‘out of town’ employment area. The other buildings in the employment area are also a similar size to the buildings being replaced by the development. The new development is larger in scale with the residential buildings being five storeys with a height of approximately 16 metres. Policy CP12 states that development proposals are expected to have regard to the local context and make efficient use of land whilst respecting the distinctiveness of the surrounding area in terms of scale, height and massing. It is considered that the development proposals do not take account of the local context in terms of scale and massing.

The proposals fail to meet policies CP1, CP2, CP3, possibly CP4, CP10 and CP12 of the Core Strategy and Policy SA2 of the Site Allocations LDD. Therefore there is a policy objection to the application.

APPLICATION B

Representation: The adopted Core Strategy under Policy CP6: Employment and Economic Development seeks to support the sustainable growth of the Three Rivers economy by continuing to focus employment use within the key employment areas that are located in the District. Criterion (j) of this policy identifies Tolpits Lane as a key employment area within Three Rivers. Criterion (n) of this policy states that office space will be considered for release from employment use where a surplus to employment needs across the plan period is expected, as indicated by an up-to-date Employment Land Study.

The South West Herts Economic Study (published in February 2016) indicates that there will be an estimated total jobs growth in the District of 8,400 during the period 2013-2036. 3,600 or 43% is estimated to require ‘B class’ employment space that includes office accommodation. This roughly equates to an estimated 7.1ha of additional land that is needed for office space within District, or just under 60,000sqm of office space during this period.

The adopted Site Allocations LDD formally identifies Tolpits Lane as an employment site allocation (site ref: E(b)). The site is not allocated as a mixed use site as claimed in the submitted Design and Access Statement on p.39. This allocation is supported by Policy SA2: Employment Site Allocations, which states that: ‘allocated employment sites will be safeguarded for business, industrial and storage or distribution uses.’

In addition to the above, and in recognition of the need to protect employment sites, the Council made a non-immediate Article 4(1) Direction on 5 August 2016 which removes the permitted development rights for change of use from office to residential and light industrial to residential within the Tolpits Lane Employment Area. This direction was confirmed on 13 October 2016 and is due to come into force on 5 August 2017 (office to residential) and 30 September 2017 (light industrial to residential). The Council considers the Article 4(1) Direction to be necessary to protect this important location for employment within the District and to secure the viability of businesses within the employment area.

Due to the site’s formal allocation for employment uses and the need to allocate further land for ‘B class’ jobs growth within the District, Tolpits Lane Employment Site is not considered suitable for residential.

The Design & Access Statement on p.155 states that the existing office floorspace is 51,000ft2 (4738.06m2) and will be replaced with 24,343.36ft2 (2261.57m2) which is a net loss of 2476.49m2. However, this is contradicted in the conclusions section on p.196, which states that the current office employment space is only 17,000ft2 (1579.35m2). The CIL Forms submitted with the application states that the existing office floorspace is 5,837.86m2 Although the form states there will be no loss of non-residential floorspace, the total gross internal floorspace proposed for non-residential will be 2,540.26m2 which is a reduction of 3,297.6m2. The proposals will result in the net loss of office floorspace and are therefore contrary to Policy SA2 of the Site Allocations LDD and Policy CP6 ((j) and (n)) of the Core Strategy .

As stated above the site is not allocated for housing in the Site Allocations LDD and does not form part of the District’s housing supply and must therefore be considered as a windfall housing site. Policy CP2 of the Core Strategy states that applications for windfall sites will be considered on a case by case basis having regard to:

- The location of the proposed development, taking into account the Spatial Strategy

- The sustainability of the development and its contribution to meeting local housing needs

- Infrastructure requirements and the impact on the delivery of allocated housing sites

- Monitoring information relating to housing supply and the Three Rivers housing target.

The Spatial Strategy states that new development will be directed towards previously developed land and appropriate infilling opportunities within the urban areas of the Principal Town (Rickmansworth) and Key Centres (South Oxhey, Croxley Green, Abbots Langley, Chorleywood, Leavesden and Garston and Mill End) which have been identified as the most sustainable locations within the District. More limited levels of new development will be directed towards Secondary Centres (Kings Langley, Carpenders Park, Eastbury, Oxhey Hall, Maple Cross and Moor Park). Place shaping policies PSP1, PSP2 and PSP3 set out the proportion of development that will be directed towards each.

The proposed site is not within or close to the boundaries of the Key Town, Key Centres or Secondary Centres identified by the Core Strategy. Essentially, Tolpits Lane Employment Area is an ‘out of town’ employment area and is in a relatively isolated location. The nearest station (Croxley Green) is just over 1km away as the crow flies, but in reality a walk to the station will be over 3km and involve walking along Tolpits Lane (which has no footpaths and no street lighting along stretches), a cycle path and then across a main carriageway. An alternative route of just over 1.1km (which was suggested by the applicant in a previous application), would involve walking through the employment area and across Croxley Common Moor (which is a Site of Special Scientific Interest and a Local Nature Reserve) where there is a public right of way but with no formal footpath and across two foot bridges over the canal.

Moor Park Station is approximately 1.8km away as the crow flies. Access to Moor Park Station as suggested by the applicants is via Tolpits Lane (no pavements or dedicated cycle route) and Sandy Lodge Road (a private road and often barrier controlled) on the private Moor Park Estate and would be 2.9km in distance. Realistically car access would be through the southern side of Moor Park via Astons Road off Batchworth Lane resulting in a car journey of 6.9km.

The Design & Access Statement states on p.87 that the proposed Vicarage Road Station (due to be functional in 2020) will be situated at the junction of Vicarage Road and Occupation Road some 1.8 miles (2.9 km) away and have provided indicative travel times by car, bicycle and walking. However, the location of a local car MOT service station has mistakenly been used for their calculations.

The proposed Vicarage Road Station will actually be situated closer to the proposed development site just off Hagden Lane approximately 1.4km away as the crow flies. The journey by car will be approximately 1.8 km, the walking distance would be the same. However walking to this station will involve walking along part of Tolpits Lane which has no footpaths. For cyclists, the same direct route could be used but there are no cycle paths. There is a suggestion that cyclists could use the Ebury Way (cycle route), however this is unlikely to be used for access to Vicarage Road Station as it would mean a travelling a distance of approximately 2.6km.

The proposed Cassiobridge Station (due to be functional in 2020) will be situated off Ascot Road and the most direct pedestrian and cyclist route (1.5km) would be along Tolpits Lane (where there are no footpaths) and then through the use of a cycle path.  The journey by car however would be substantially longer at a distance of 2.7km.

The site is not serviced by public bus routes, Tolpits Lane itself lacks footpaths and there is no specific provision for cyclists on the highway. The site is isolated from key services including schools and health care facilities and does not have easy access to existing community facilities and retail options. The nearest bus stop is nearly a mile away.

The proposed location for this residential development is considered unsustainable and contrary to the objectives set out in the Place Shaping Policies (PSP1, PSP2 and PSP3) and Policy CP1 (Overarching Policy on Sustainable Development) of the Core Strategy. Three Rivers District currently has identified a 9.8 year supply of housing land that meets the definition of deliverability set out in the NPPF and therefore the proposed residential development of 332 apartments is not required to meet the District Housing Targets.

It is therefore not considered that the proposal can be justified on the basis of the criteria contained in Policy CP2 in relation to windfall sites.

Policy CP10 (Transport and Travel), states that all development proposals should be designed and located to minimise the impacts of travel by motor vehicle. In particular, major development will be expected to be located in areas highly accessible by the most sustainable modes of transport, and to all people of abilities in a socially inclusive and safe manner in accordance with the following user hierarchy:

- Pedestrians, particularly people with restricted mobility

- Cyclists and where appropriate horse riders

- Public transport

- All forms of motor vehicle

The location of the proposal, the lack of footpaths and access to public transport (Bus and rail) result in the proposal being contrary to Policy CP10 of the Core Strategy.

The Transport Assessment on p.7 sets out that they (R Square Properties) and Arriva ‘is currently working closely with Arriva’ to provide a bus service and have sought views from HCC on their preferred route from the following two options:

- Wolsey Business Park to Watford Junction

- Wolsey Business Park to Rickmansworth Station

The Transport Assessment contains indicative timetables with bus services generally every 30 minutes. It is also stated (third paragraph) that RSquare Properties undertakes to underwrite the portion of the costs that is not covered by revenue from the first day of occupation of The Moors for a period of 5 years after The Moors has reached 85% occupation on one of the services.

From the email trail provided in the Transport Assessment, HCC have provided comments to say that the Wolsey Business Park to Watford Junction Service would be the most sustainable option but there is doubt that there would be sufficient demand to justify more than an hourly off peak and Saturday service.

Consideration may have to be given as to whether the ‘one’ bus service (either from Wolsey Business Park to Watford Junction or Wolsey Business Park to Rickmansworth Station), would address the accessibility issues to the site as set out above.  However, in the absence of a completed Section 106 Agreement and written confirmation from Arriva, confirming a preferred bus route and timetable, I am unable to consider this at this time.

Policy DM4 of the Development Management Policies LDD requires applicants to demonstrate that development proposals will produce 5% less carbon dioxide emissions than Building Regulations Part L requirements (2013). The information provided shows that the development proposals would meet these requirements and therefore meets Policy DM4. However, the development’s proposed sustainability credentials in relation to energy use and carbon emissions does not mean that the proposal meets the definition of sustainable development as set out in the NPPF. Paragraph 7 of the NPPF sets out that there are three dimensions to sustainable development: economic, social and environmental.

Housing Mix

Policy CP3 of the Core Strategy states that ‘The Council will require housing proposals to take into account the range of housing needs, in terms of size and type of dwellings as identified by the Strategic Housing Market Assessment and subsequent updates.’ The most recent SHMA was published in February 2016 and has identified the indicative targets for market and affordable sectors’ dwelling size within the Three Rivers District as follows:

1 bedroom 7.7% of dwellings

2 bedrooms 27.8% of dwellings

3 bedrooms 41.5% of dwellings

4+ bedrooms 23.0% of dwellings

The development proposes a total of 332 apartments which according to the application form would provide:

1-bed units 62% (205)

2-bed units 36% (119)

3-bed units 2% (8)

4-bed units 0% (0)

The proposed mix is not in accordance with Policy CP3 which seeks a much higher percentage of 3-bed and 4-bed units and a lower percentage of 1-bed and 2-bed units. Whilst it is possible to consider a variation to the percentages set out in Policy CP3 on a case by case basis, no evidence/justification has been provided to support the proposed housing mix.

Affordable Housing

Policy CP4 of the adopted Core Strategy (2011) requires 45% of all new housing to be provided as Affordable Housing, unless it can be clearly demonstrated with financial evidence that this is not viable. As a guide the tenure split should be 70% social rented and 30% intermediate.

According to the Affordable Housing Statement, the development proposes a total of 20 affordable units (6%) with a tenure split of (14 x 1 bed and 6 x 2 bed) to be provided on the basis of 100% socially rented.

Where non-viability is cited as the reason for a development proposal not complying with affordable housing requirements, applicants must submit financial evidence (viability appraisal) to justify this position. Subject to the independent review of the submitted Viability Report, Local Plans reserves comment as to whether the proposals meet Policy CP4 of the Core Strategy.

The development will be replacing two 3 storey buildings in an ‘out of town’ employment area. The other buildings in the employment area are also a similar size to the buildings being replaced by the development. The new development is larger in scale with the residential buildings being five storeys with a height of approximately nearly 16 metres. Policy CP12 states that development proposals are expected to have regard to the local context and make efficient use of land whilst respecting the distinctiveness of the surrounding area in terms of scale, height and massing. It is considered that the development proposals do not take account of the local context in terms of scale and massing.

The proposals fail to meet policies CP1, CP2, CP3, possibly CP4, CP10 and CP12 of the Core Strategy and Policy SA2 of the Site Allocations LDD. Therefore there is a policy objection to the application.

APPLICATION C

Representation: The adopted Core Strategy under Policy CP6: Employment and Economic Development seeks to support the sustainable growth of the Three Rivers economy by continuing to focus employment use within the key employment areas that are located in the District. Criterion (j) of this policy identifies Tolpits Lane as a key employment area within Three Rivers. Criterion (n) of this policy states that office space will be considered for release from employment use where a surplus to employment needs across the plan period is expected, as indicated by an up-to-date Employment Land Study.

The South West Herts Economic Study (published in February 2016) indicates that there will be an estimated total jobs growth in the District of 8,400 during the period 2013-2036. 3,600 or 43% is estimated to require ‘B class’ employment space that includes office accommodation. This roughly equates to an estimated 7.1ha of additional land that is needed for office space within District, or just under 60,000sqm of office space during this period.

The adopted Site Allocations LDD formally identifies Tolpits Lane as an employment site allocation (site ref: E(b)). The site is not allocated as a mixed use site as claimed in the submitted Design and Access Statement on p.39. This allocation is supported by Policy SA2: Employment Site Allocations, which states that: ‘allocated employment sites will be safeguarded for business, industrial and storage or distribution uses.’

In addition to the above, and in recognition of the need to protect employment sites, the Council made a non-immediate Article 4(1) Direction on 5 August 2016 which removes the permitted development rights for change of use from office to residential and light industrial to residential within the Tolpits Lane Employment Area. This direction was confirmed on 13 October 2016 and is due to come into force on 5 August 2017 (office to residential) and 30 September 2017 (light industrial to residential). The Council considers the Article 4(1) Direction to be necessary to protect this important location for employment within the District and to secure the viability of businesses within the employment area.

Due to the site’s formal allocation for employment uses and the need to allocate further land for ‘B class’ jobs growth within the District, Tolpits Lane Employment Site is not considered suitable for residential.

The Design & Access Statement on p.155 states that the existing office floorspace is 51,000ft2 (4738.06m2) and will be replaced with 27,343.36ft2 (2540.28m2) which is a net loss of 2197.78m2. However, this is contradicted in the conclusions section on p.196, which states that the current office employment space is only 17,000ft2 (1579.35m2). The CIL Forms submitted with the application states that the existing office floorspace is 5,837.86m2 Although the form states there will be no loss of non-residential floorspace, the total gross internal floorspace proposed for non-residential will be 2,540.26m2 which is a reduction of 3,297.6m2. The proposals will result in the net loss of office floorspace and are therefore contrary to Policy SA2 of the Site Allocations LDD and Policy CP6 ((j) and (n)) of the Core Strategy .

As stated above the site is not allocated for housing in the Site Allocations LDD and does not form part of the District’s housing supply and must therefore be considered as a windfall housing site. Policy CP2 of the Core Strategy states that applications for windfall sites will be considered on a case by case basis having regard to:

- The location of the proposed development, taking into account the Spatial Strategy

- The sustainability of the development and its contribution to meeting local housing needs

- Infrastructure requirements and the impact on the delivery of allocated housing sites

- Monitoring information relating to housing supply and the Three Rivers housing target.

The Spatial Strategy states that new development will be directed towards previously developed land and appropriate infilling opportunities within the urban areas of the Principal Town (Rickmansworth) and Key Centres (South Oxhey, Croxley Green, Abbots Langley, Chorleywood, Leavesden and Garston and Mill End) which have been identified as the most sustainable locations within the District. More limited levels of new development will be directed towards Secondary Centres (Kings Langley, Carpenders Park, Eastbury, Oxhey Hall, Maple Cross and Moor Park). Place shaping policies PSP1, PSP2 and PSP3 set out the proportion of development that will be directed towards each.

The proposed site is not within or close to the boundaries of the Key Town, Key Centres or Secondary Centres identified by the Core Strategy. Essentially, Tolpits Lane Employment Area is an ‘out of town’ employment area and is in a relatively isolated location. The nearest station (Croxley Green) is just over 1km away as the crow flies, but in reality a walk to the station will be over 3km and involve walking along Tolpits Lane (which has no footpaths and no street lighting along stretches), a cycle path and then across a main carriageway. An alternative route of just over 1.1km (which was suggested by the applicant in a previous application), would involve walking through the employment area and across Croxley Common Moor (which is a Site of Special Scientific Interest and a Local Nature Reserve) where there is a public right of way but with no formal footpath and across two foot bridges over the canal.

Moor Park Station is approximately 1.8km away as the crow flies. Access to Moor Park Station as suggested by the applicants is via Tolpits Lane (no pavements or dedicated cycle route) and Sandy Lodge Road (a private road and often barrier controlled) on the private Moor Park Estate and would be 2.9km in distance. Realistically car access would be through the southern side of Moor Park via Astons Road off Batchworth Lane resulting in a car journey of 6.9km.

The Design & Access Statement states on p.87 that the proposed Vicarage Road Station (due to be functional in 2020) will be situated at the junction of Vicarage Road and Occupation Road some 1.8 miles (2.9 km) away and have provided indicative travel times by car, bicycle and walking. However, the location of a local car MOT service station has mistakenly been used for their calculations.

The proposed Vicarage Road Station will actually be situated closer to the proposed development site just off Hagden Lane approximately 1.4km away as the crow flies. The journey by car will be approximately 1.8 km, the walking distance would be the same. However walking to this station will involve walking along part of Tolpits Lane which has no footpaths. For cyclists, the same direct route could be used but there are no cycle paths. There is a suggestion that cyclists could use the Ebury Way (cycle route), however this is unlikely to be used for access to Vicarage Road Station as it would mean a travelling a distance of approximately 2.6km.

The proposed Cassiobridge Station (due to be functional in 2020) will be situated off Ascot Road and the most direct pedestrian and cyclist route (1.5km) would be along Tolpits Lane (where there are no footpaths) and then through the use of a cycle path.  The journey by car however would be substantially longer at a distance of 2.7km.

The site is not serviced by public bus routes, Tolpits Lane itself lacks footpaths and there is no specific provision for cyclists on the highway. The site is isolated from key services including schools and health care facilities and does not have easy access to existing community facilities and retail options. The nearest bus stop is nearly a mile away.

The proposed location for this residential development is considered unsustainable and contrary to the objectives set out in the Place Shaping Policies (PSP1, PSP2 and PSP3) and Policy CP1 (Overarching Policy on Sustainable Development) of the Core Strategy. Three Rivers District currently has identified a 9.8 year supply of housing land that meets the definition of deliverability set out in the NPPF and therefore the proposed residential development of 403 apartments is not required to meet the District Housing Targets.

It is therefore not considered that the proposal can be justified on the basis of the criteria contained in Policy CP2 in relation to windfall sites.

Policy CP10 (Transport and Travel), states that all development proposals should be designed and located to minimise the impacts of travel by motor vehicle. In particular, major development will be expected to be located in areas highly accessible by the most sustainable modes of transport, and to all people of abilities in a socially inclusive and safe manner in accordance with the following user hierarchy:

- Pedestrians, particularly people with restricted mobility

- Cyclists and where appropriate horse riders

- Public transport

- All forms of motor vehicle

The location of the proposal, the lack of footpaths and access to public transport (Bus and rail) result in the proposal being contrary to Policy CP10 of the Core Strategy.

The Transport Assessment on p.7 sets out that they (R Square Properties) and Arriva ‘is currently working closely with Arriva’ to provide a bus service and have sought views from HCC on their preferred route from the following two options:

- Wolsey Business Park to Watford Junction

- Wolsey Business Park to Rickmansworth Station

The Transport Assessment contains indicative timetables with bus services generally every 30 minutes. It is also stated (third paragraph) that RSquare Properties undertakes to underwrite the portion of the costs that is not covered by revenue from the first day of occupation of The Moors for a period of 5 years after The Moors has reached 85% occupation on one of the services.

From the email trail provided in the Transport Assessment, HCC have provided comments to say that the Wolsey Business Park to Watford Junction Service would be the most sustainable option but there is doubt that there would be sufficient demand to justify more than an hourly off peak and Saturday service.

Consideration may have to be given as to whether the ‘one’ bus service (either from Wolsey Business Park to Watford Junction or Wolsey Business Park to Rickmansworth Station), would address the accessibility issues to the site as set out above.  However, in the absence of a completed Section 106 Agreement and written confirmation from Arriva, confirming a preferred bus route and timetable, I am unable to consider this at this time.

Policy DM4 of the Development Management Policies LDD requires applicants to demonstrate that development proposals will produce 5% less carbon dioxide emissions than Building Regulations Part L requirements (2013). The information provided shows that the development proposals would meet these requirements and therefore meets Policy DM4. However, the development’s proposed sustainability credentials in relation to energy use and carbon emissions does not mean that the proposal meets the definition of sustainable development as set out in the NPPF. Paragraph 7 of the NPPF sets out that there are three dimensions to sustainable development: economic, social and environmental.

Housing Mix

Policy CP3 of the Core Strategy states that ‘The Council will require housing proposals to take into account the range of housing needs, in terms of size and type of dwellings as identified by the Strategic Housing Market Assessment and subsequent updates.’ The most recent SHMA was published in February 2016 and has identified the indicative targets for market and affordable sectors’ dwelling size within the Three Rivers District as follows:

1 bedroom 7.7% of dwellings

2 bedrooms 27.8% of dwellings

3 bedrooms 41.5% of dwellings

4+ bedrooms 23.0% of dwellings

The development proposes a total of 403 apartments which according to the application form would provide:

1-bed units 61% (246)

2-bed units 37% (149)

3-bed units 2% (8)

4-bed units 0% (0)

The proposed mix is not in accordance with Policy CP3 which seeks a much higher percentage of 3-bed and 4-bed units and a lower percentage of 1-bed and 2-bed units. Whilst it is possible to consider a variation to the percentages set out in Policy CP3 on a case by case basis, no evidence/justification has been provided to support the proposed housing mix.

Affordable Housing

Policy CP4 of the adopted Core Strategy (2011) requires 45% of all new housing to be provided as Affordable Housing, unless it can be clearly demonstrated with financial evidence that this is not viable. As a guide the tenure split should be 70% social rented and 30% intermediate.

According to the Affordable Housing Statement, the development proposes a total of 41 affordable units (10%) with a tenure split of (29 x 1 bed and 12 x 2 bed) to be provided on the basis of 100% socially rented.

Where non-viability is cited as the reason for a development proposal not complying with affordable housing requirements, applicants must submit financial evidence (viability appraisal) to justify this position. Subject to the independent review of the submitted Viability Report, Local Plans reserves comment as to whether the proposals meet Policy CP4 of the Core Strategy.

The development will be replacing two 3 storey buildings in an ‘out of town’ employment area. The other buildings in the employment area are also a similar size to the buildings being replaced by the development. The new development is larger in scale with the residential buildings being six storeys with a height of approximately nearly 19 metres. Policy CP12 states that development proposals are expected to have regard to the local context and make efficient use of land whilst respecting the distinctiveness of the surrounding area in terms of scale, height and massing. It is considered that the development proposals do not take account of the local context in terms of scale and massing.

The proposals fail to meet policies CP1, CP2, CP3, possibly CP4, CP10 and CP12 of the Core Strategy and Policy SA2 of the Site Allocations LDD. Therefore there is a policy objection to the application.

APPLICATION D

Representation: The adopted Core Strategy under Policy CP6: Employment and Economic Development seeks to support the sustainable growth of the Three Rivers economy by continuing to focus employment use within the key employment areas that are located in the District. Criterion (j) of this policy identifies Tolpits Lane as a key employment area within Three Rivers. Criterion (n) of this policy states that office space will be considered for release from employment use where a surplus to employment needs across the plan period is expected, as indicated by an up-to-date Employment Land Study.

The South West Herts Economic Study (published in February 2016) indicates that there will be an estimated total jobs growth in the District of 8,400 during the period 2013-2036. 3,600 or 43% is estimated to require ‘B class’ employment space that includes office accommodation. This roughly equates to an estimated 7.1ha of additional land that is needed for office space within District, or just under 60,000sqm of office space during this period.

The adopted Site Allocations LDD formally identifies Tolpits Lane as an employment site allocation (site ref: E(b)). The site is not allocated as a mixed use site as claimed in the submitted Design and Access Statement on p.39. This allocation is supported by Policy SA2: Employment Site Allocations, which states that: ‘allocated employment sites will be safeguarded for business, industrial and storage or distribution uses.’

In addition to the above, and in recognition of the need to protect employment sites, the Council made a non-immediate Article 4(1) Direction on 5 August 2016 which removes the permitted development rights for change of use from office to residential and light industrial to residential within the Tolpits Lane Employment Area. This direction was confirmed on 13 October 2016 and is due to come into force on 5 August 2017 (office to residential) and 30 September 2017 (light industrial to residential). The Council considers the Article 4(1) Direction to be necessary to protect this important location for employment within the District and to secure the viability of businesses within the employment area.

Due to the site’s formal allocation for employment uses and the need to allocate further land for ‘B class’ jobs growth within the District, Tolpits Lane Employment Site is not considered suitable for residential.

The Design & Access Statement on p.155 states that the existing office floorspace is 51,000ft2 (4738.06m2) and will be replaced with 47,000ft2 (4366.44m2) which is a net loss of 371.62m2. However, this is contradicted in the conclusions section on p.196, which states that the current office employment space is only 17,000ft2 (1579.35m2). The CIL Forms submitted with the application states that the existing office floorspace is 5,837.86m2 and that 447.57m2 of office space will be lost by demolition or change of use. The proposals will result in the net loss of office floorspace and are therefore contrary to Policy SA2 of the Site Allocations LDD and Policy CP6 ((j) and (n)) of the Core Strategy .

As stated above the site is not allocated for housing in the Site Allocations LDD and does not form part of the District’s housing supply and must therefore be considered as a windfall housing site. Policy CP2 of the Core Strategy states that applications for windfall sites will be considered on a case by case basis having regard to:

- The location of the proposed development, taking into account the Spatial Strategy

- The sustainability of the development and its contribution to meeting local housing needs

- Infrastructure requirements and the impact on the delivery of allocated housing sites

- Monitoring information relating to housing supply and the Three Rivers housing target.

The Spatial Strategy states that new development will be directed towards previously developed land and appropriate infilling opportunities within the urban areas of the Principal Town (Rickmansworth) and Key Centres (South Oxhey, Croxley Green, Abbots Langley, Chorleywood, Leavesden and Garston and Mill End) which have been identified as the most sustainable locations within the District. More limited levels of new development will be directed towards Secondary Centres (Kings Langley, Carpenders Park, Eastbury, Oxhey Hall, Maple Cross and Moor Park). Place shaping policies PSP1, PSP2 and PSP3 set out the proportion of development that will be directed towards each.

The proposed site is not within or close to the boundaries of the Key Town, Key Centres or Secondary Centres identified by the Core Strategy. Essentially, Tolpits Lane Employment Area is an ‘out of town’ employment area and is in a relatively isolated location. The nearest station (Croxley Green) is just over 1km away as the crow flies, but in reality a walk to the station will be over 3km and involve walking along Tolpits Lane (which has no footpaths and no street lighting along stretches), a cycle path and then across a main carriageway. An alternative route of just over 1.1km (which was suggested by the applicant in a previous application), would involve walking through the employment area and across Croxley Common Moor (which is a Site of Special Scientific Interest and a Local Nature Reserve) where there is a public right of way but with no formal footpath and across two foot bridges over the canal.

Moor Park Station is approximately 1.8km away as the crow flies. Access to Moor Park Station as suggested by the applicants is via Tolpits Lane (no pavements or dedicated cycle route) and Sandy Lodge Road (a private road and often barrier controlled) on the private Moor Park Estate and would be 2.9km in distance. Realistically car access would be through the southern side of Moor Park via Astons Road off Batchworth Lane resulting in a car journey of 6.9km.

The Design & Access Statement states on p.87 that the proposed Vicarage Road Station (due to be functional in 2020) will be situated at the junction of Vicarage Road and Occupation Road some 1.8 miles (2.9 km) away and have provided indicative travel times by car, bicycle and walking. However, the location of a local car MOT service station has mistakenly been used for their calculations.

The proposed Vicarage Road Station will actually be situated closer to the proposed development site just off Hagden Lane approximately 1.4km away as the crow flies. The journey by car will be approximately 1.8 km, the walking distance would be the same. However walking to this station will involve walking along part of Tolpits Lane which has no footpaths. For cyclists, the same direct route could be used but there are no cycle paths. There is a suggestion that cyclists could use the Ebury Way (cycle route), however this is unlikely to be used for access to Vicarage Road Station as it would mean a travelling a distance of approximately 2.6km.

The proposed Cassiobridge Station (due to be functional in 2020) will be situated off Ascot Road and the most direct pedestrian and cyclist route (1.5km) would be along Tolpits Lane (where there are no footpaths) and then through the use of a cycle path.  The journey by car however would be substantially longer at a distance of 2.7km.

The site is not serviced by public bus routes, Tolpits Lane itself lacks footpaths and there is no specific provision for cyclists on the highway. The site is isolated from key services including schools and health care facilities and does not have easy access to existing community facilities and retail options. The nearest bus stop is nearly a mile away.

The proposed location for this residential development is considered unsustainable and contrary to the objectives set out in the Place Shaping Policies (PSP1, PSP2 and PSP3) and Policy CP1 (Overarching Policy on Sustainable Development) of the Core Strategy. Three Rivers District currently has identified a 9.8 year supply of housing land that meets the definition of deliverability set out in the NPPF and therefore the proposed residential development of 416 apartments is not required to meet the District Housing Targets.

It is therefore not considered that the proposal can be justified on the basis of the criteria contained in Policy CP2 in relation to windfall sites.

Policy CP10 (Transport and Travel), states that all development proposals should be designed and located to minimise the impacts of travel by motor vehicle. In particular, major development will be expected to be located in areas highly accessible by the most sustainable modes of transport, and to all people of abilities in a socially inclusive and safe manner in accordance with the following user hierarchy:

- Pedestrians, particularly people with restricted mobility

- Cyclists and where appropriate horse riders

- Public transport

- All forms of motor vehicle

The location of the proposal, the lack of footpaths and access to public transport (Bus and rail) result in the proposal being contrary to Policy CP10 of the Core Strategy.

The Transport Assessment on p.7 sets out that they (R Square Properties) and Arriva ‘is currently working closely with Arriva’ to provide a bus service and have sought views from HCC on their preferred route from the following two options:

- Wolsey Business Park to Watford Junction

- Wolsey Business Park to Rickmansworth Station

The Transport Assessment contains indicative timetables with bus services generally every 30 minutes. It is also stated (third paragraph) that RSquare Properties undertakes to underwrite the portion of the costs that is not covered by revenue from the first day of occupation of The Moors for a period of 5 years after The Moors has reached 85% occupation on one of the services.

From the email trail provided in the Transport Assessment, HCC have provided comments to say that the Wolsey Business Park to Watford Junction Service would be the most sustainable option but there is doubt that there would be sufficient demand to justify more than an hourly off peak and Saturday service.

Consideration may have to be given as to whether the ‘one’ bus service (either from Wolsey Business Park to Watford Junction or Wolsey Business Park to Rickmansworth Station), would address the accessibility issues to the site as set out above.  However, in the absence of a completed Section 106 Agreement and written confirmation from Arriva, confirming a preferred bus route and timetable, I am unable to consider this at this time.

Policy DM4 of the Development Management Policies LDD requires applicants to demonstrate that development proposals will produce 5% less carbon dioxide emissions than Building Regulations Part L requirements (2013). The information provided shows that the development proposals would meet these requirements and therefore meets Policy DM4. However, the development’s proposed sustainability credentials in relation to energy use and carbon emissions does not mean that the proposal meets the definition of sustainable development as set out in the NPPF. Paragraph 7 of the NPPF sets out that there are three dimensions to sustainable development: economic, social and environmental.

Housing Mix

Policy CP3 of the Core Strategy states that ‘The Council will require housing proposals to take into account the range of housing needs, in terms of size and type of dwellings as identified by the Strategic Housing Market Assessment and subsequent updates.’ The most recent SHMA was published in February 2016 and has identified the indicative targets for market and affordable sectors’ dwelling size within the Three Rivers District as follows:

1 bedroom 7.7% of dwellings

2 bedrooms 27.8% of dwellings

3 bedrooms 41.5% of dwellings

4+ bedrooms 23.0% of dwellings

The development proposes a total of 416 apartments which according to the application form would provide:

1-bed units 58% (241)

2-bed units 40% (167)

3-bed units 2% (8)

4-bed units 0% (0)

The proposed mix is not in accordance with Policy CP3 which seeks a much higher percentage of 3-bed and 4-bed units and a lower percentage of 1-bed and 2-bed units. Whilst it is possible to consider a variation to the percentages set out in Policy CP3 on a case by case basis, no evidence/justification has been provided to support the proposed housing mix.

Affordable Housing

Policy CP4 of the adopted Core Strategy (2011) requires 45% of all new housing to be provided as Affordable Housing, unless it can be clearly demonstrated with financial evidence that this is not viable. As a guide the tenure split should be 70% social rented and 30% intermediate.

According to the Affordable Housing Statement, the development proposes a total of 66 affordable units (16%) with a tenure split of (41 x 1 bed and 25 x 2 bed) to be provided on the basis of 100% socially rented.

Where non-viability is cited as the reason for a development proposal not complying with affordable housing requirements, applicants must submit financial evidence (viability appraisal) to justify this position. Subject to the independent review of the submitted Viability Report, Local Plans reserves comment as to whether the proposals meet Policy CP4 of the Core Strategy.

The development will be replacing two 3 storey buildings in an ‘out of town’ employment area. The other buildings in the employment area are also a similar size to the buildings being replaced by the development. The new development is larger in scale with the residential buildings being seven storeys with a height of approximately nearly 22 metres. Policy CP12 states that development proposals are expected to have regard to the local context and make efficient use of land whilst respecting the distinctiveness of the surrounding area in terms of scale, height and massing. It is considered that the development proposals do not take account of the local context in terms of scale and massing.

The proposals fail to meet policies CP1, CP2, CP3, possibly CP4, CP10 and CP12 of the Core Strategy and Policy SA2 of the Site Allocations LDD. Therefore there is a policy objection to the application.

NHS England: No comments received.

Primary Care Trust: No comments received.

Health & Community Services: No comments received.

Thames Water: [Advisory comments]

APPLICATION A

No comments received.

APPLICATION B

Waste Comments

Thames Water requests that the Applicant should incorporate within their proposal, protection to the property by installing for example, a non-return valve or other suitable device to avoid the risk of backflow at a later date, on the assumption that the sewerage network may surcharge to ground level during storm conditions.

Thames Water recommends the installation of a properly maintained fat trap on all catering establishments. We further recommend, in line with best practice for the disposal of Fats, Oils and Grease, the collection of waste oil by a contractor, particularly to recycle for the production of bio diesel. Failure to implement these recommendations may result in this and other properties suffering blocked drains, sewage flooding and pollution to local watercourses.

A Trade Effluent Consent will be required for any Effluent discharge other than a 'Domestic Discharge'. Any discharge without this consent is illegal and may result in prosecution. (Domestic usage for example includes - toilets, showers, washbasins, baths, private swimming pools and canteens). Typical Trade Effluent processes include: - Laundrette/Laundry, PCB manufacture, commercial swimming pools, photographic/printing, food preparation, abattoir, farm wastes, vehicle washing, metal plating/finishing, cattle market wash down, chemical manufacture, treated cooling water and any other process which produces contaminated water. Pre-treatment, separate metering, sampling access etc, may be required before the Company can give its consent. Applications should be made at http://www.thameswater.co.uk/business/9993.htm or alternatively to Waste Water Quality, Crossness STW, Belvedere Road, Abbeywood, London. SE2 9AQ. Telephone: 020 3577 9200.

Thames Water would recommend that petrol / oil interceptors be fitted in all car parking/washing/repair facilities. Failure to enforce the effective use of petrol / oil interceptors could result in oil-polluted discharges entering local watercourses.

Swimming Pools - Where the proposal includes a swimming pool Thames Water requests that the following conditions are adhered to with regard to the emptying of swimming pools into a public sewer to prevent the risk of flooding or surcharging: - 1.The pool to be emptied overnight and in dry periods. 2. The discharge rate is controlled such that it does not exceed a flow rate of 5 litres/ second into the public sewer network.

No piling shall take place until a piling method statement (detailing the depth and type of piling to be undertaken and the methodology by which such piling will be carried out, including measures to prevent and minimise the potential for damage to subsurface sewerage infrastructure, and the programme for the works) has been submitted to and approved in writing by the local planning authority in consultation with Thames Water.  Any piling must be undertaken in accordance with the terms of the approved piling method statement. Reason: The proposed works will be in close proximity to underground sewerage utility infrastructure.  Piling has the potential to impact on local underground sewerage utility infrastructure. The applicant is advised to contact Thames Water Developer Services on 0800 009 3921 to discuss the details of the piling method statement.

'We would expect the developer to demonstrate what measures he will undertake to minimise groundwater discharges into the public sewer.  Groundwater discharges typically result from construction site dewatering, deep excavations, basement infiltration, borehole installation, testing and site remediation. Any discharge made without a permit is deemed illegal and may result in prosecution under the provisions of the Water Industry Act 1991.  Should the Local Planning Authority be minded to approve the planning application, Thames Water would like  the following informative attached to the planning permission: "A Groundwater Risk Management Permit from Thames Water will be required for discharging groundwater into a public sewer. Any discharge made without a permit is deemed illegal and may result in prosecution under the provisions of the Water Industry Act 1991. We would expect the developer to demonstrate what measures he will undertake to minimise groundwater discharges into the public sewer.  Permit enquiries should be directed to Thames Water's Risk Management Team by telephoning 02035779483 or by emailing wwqriskmanagement@thameswater.co.uk. Application forms should be completed on line via www.thameswater.co.uk/wastewaterquality."

Water Comments

With regard to water supply, this comes within the area covered by the Affinity Water Company. For your information the address to write to is - Affinity Water Company The Hub, Tamblin Way, Hatfield, Herts, AL10 9EZ - Tel - 0845 782 3333.

Supplementary Comments

Based on information contained in The Moors Development Drainage Strategy dated 20/04/2016 we have no objections to the proposals. If any changes, regarding drainage, have been made we expect documents will be provided and a new assessment will be made.

APPLICATIONS C & D

Waste Comments

Thames Water requests that the Applicant should incorporate within their proposal, protection to the property by installing for example, a non-return valve or other suitable device to avoid the risk of backflow at a later date, on the assumption that the sewerage network may surcharge to ground level during storm conditions.

A Trade Effluent Consent will be required for any Effluent discharge other than a 'Domestic Discharge'. Any discharge without this consent is illegal and may result in prosecution. (Domestic usage for example includes - toilets, showers, washbasins, baths, private swimming pools and canteens). Typical Trade Effluent processes include: - Laundrette/Laundry, PCB manufacture, commercial swimming pools, photographic/printing, food preparation, abattoir, farm wastes, vehicle washing, metal plating/finishing, cattle market wash down, chemical manufacture, treated cooling water and any other process which produces contaminated water. Pre-treatment, separate metering, sampling access etc, may be required before the Company can give its consent. Applications should be made at http://www.thameswater.co.uk/business/9993.htm or alternatively to Waste Water Quality, Crossness STW, Belvedere Road, Abbeywood, London. SE2 9AQ. Telephone: 020 3577 9200.

No piling shall take place until a piling method statement (detailing the depth and type of piling to be undertaken and the methodology by which such piling will be carried out, including measures to prevent and minimise the potential for damage to subsurface sewerage infrastructure, and the programme for the works) has been submitted to and approved in writing by the local planning authority in consultation with Thames Water.  Any piling must be undertaken in accordance with the terms of the approved piling method statement. Reason: The proposed works will be in close proximity to underground sewerage utility infrastructure.  Piling has the potential to impact on local underground sewerage utility infrastructure. The applicant is advised to contact Thames Water Developer Services on 0800 009 3921 to discuss the details of the piling method statement.

Water Comments

With regard to water supply, this comes within the area covered by the Affinity Water Company. For your information the address to write to is - Affinity Water Company The Hub, Tamblin Way, Hatfield, Herts, AL10 9EZ - Tel - 0845 782 3333.

Supplementary Comments

Based on information contained in The Moors Development Drainage Strategy dated 20/04/2016 we have no objections to the proposals.

Affinity Water: [Advisory comments]

APPLICATIONS A & B & C

You should be aware that the proposed development site is located close to or within an Environment Agency defined groundwater Source Protection Zone (GPZ) corresponding to Tolpits Lane Pumping Station. This is a public water supply, comprising a number of Chalk abstraction boreholes, operated by Affinity Water Ltd.

The construction works and operation of the proposed development site should be done in accordance with the relevant British Standards and Best Management Practices, thereby significantly reducing the groundwater pollution risk. It should be noted that the construction works may exacerbate any existing pollution. If any pollution is found at the sites then the appropriate monitoring and remediation methods will need to be undertaken.

For further information we refer you to CIRIA Publication C532 "Control of water pollution from construction - guidance for consultants and contractors.

National Grid: [Advisory comments for all applications]

National Grid has identified that it has apparatus in the vicinity which may be affected by the activities specified.  Due to the presence of National Grid apparatus in proximity to the specified area, the contractor should contact National Grid before any works are carried out to ensure National Grid apparatus is not affected by any of the proposed works.

Moor Park 1958 Ltd: [Objection, all applications]

The Directors of Moor Park (1958) Limited would wish to raise a number of concerns, comments and objections in regard to all four of the current applications listed in the heading of this letter as set out below.

Although we acknowledge that each of the four outline applications incorporates a degree of variation in the scale of development, we are nonetheless satisfied that our planning objections and concerns, as listed below, are relevant and material in equal measure in the assessment and determination of all four outline schemes.

In light of this we would ask that our representations are presented on all four cases, as follows:-

1. Impact on the setting of the Moor Park Conservation Area.  Although we acknowledge that there is some distance between the application site and the nearest boundary of the designated Moor Park Conservation Area, there can be no doubt that the sheer scale of the proposed development, especially its height and mass/form of the buildings, is very substantial.

The legal framework (and various planning law cases) associated with the determination of planning applications in relation to heritage assets (viz Conservation Areas), is that decision makers are required to have regard to the potential impact of all developments, upon the setting of the Conservation Area/designated heritage assets. Paragraph 132 of the National Planning Policy Framework states that:-

"when considering the impact of a proposed development on the significance of a designated heritage asset, great weight should be given to the asset's conservation.  The more important the asset, the greater the weight should be ".

In light of the above, we would request that full regard is had, in the assessment and determination of the application, to the potential impact of the proposed development upon the setting of the Moor Park Conservation Area, especially taking into account the degree to which the setting of the Conservation Area, on its northern and eastern sides, is so heavily characterised by the relationship between the Conservation Area and the open outlook and open character of the Metropolitan Green Belt.

2. Traffic generation - along Tolpits Lane.  We wish to express our strong objections to any increase in traffic flows along Tolpits Lane arising from the proposed development that will prejudice the free and safe flow of vehicular traffic into and out of the Moor Park estate (at the junction of Tolpits Lane and Sandy Lodge Road).  This junction already presents a number of extremely difficult manoeuvres; due to the alignment of the sharp bend in Tolpits Lane at the corresponding point with the point of access into and out of the estate, and also the very sharp turn from the entrance point into Sandy Lodge Road itself. We would request that very close attention is paid to this concern having regard to the need to ensure, and to secure, maximum highway safety at this point.

3. Traffic generation - through the Moor Park Estate itself. We wish to express our strong objections and concerns that the scale of the proposed development and especially the likely extent of traffic generation associated with the development, presents a very strong possibility that traffic approaching the application site directly from the south, will be drawn through the roads that serve the Moor Park Conservation Area.  Alternatives, either via Watford (using roads to the east of Moor Park) or via Rickmansworth (using roads to the west), will present a far less attractive proposition than a convenient shortcut through the Conservation Area estate.  It is our firm view that any marked increase of such traffic flows will have a highly detrimental and damaging impact on the local infrastructure of the estate and, in planning terms, will fundamentally fail to preserve and enhance the character and appearance of the designated Conservation Area, contrary to both national and local planning policy.