9.	15/2610/FUL - First floor side and rear extension, replacement of tile hanging with render, addition of flank rooflight and alterations to fenestration at 6 MILL WAY, MILL END, WD3 8QP for Mr and Mrs Allan Iveson

(DCES)

| Parish: Non-parished | Ward: Penn and Mill End |
| --- | --- |
| Expiry Statutory Period: 24.02.2016 | Officer: Amy Atkins |
| Recommendation: That Planning Permission be granted subject to conditions. |  |
| This application is brought before the Committee as a neighbour is an employee of Three Rivers District Council. |  |

1.	Relevant Planning History

1.1	8/594/76 - Kitchen extension. Permitted 28.09.1976

1.2	W/1443/72 - Dining room extension. Permitted 30.05.1972

1.3	W/1966/69 – Porch. Permitted 05.08.1969

2.	Site Description

2.1	The dwelling is a two storey semi detached property located on the north-east side of Mill Way.  The dwelling has a single storey flat roofed garage to the side and additional off-street parking for two cars to the frontage.

2.2	To the rear of the dwelling is a soft landscaped garden with 1.8m high fencing as the boundary treatment. There is a patio area adjacent to the dwelling. The site is adjacent to a playing field to the rear.

2.3	The dwelling has previously been extended to the rear at ground floor level. Behind the existing garage there is a temporary structure which extends along the boundary with the neighbouring dwelling no. 4. The adjoining neighbour, no. 8 has a single storey rear extension.

3.	Description of Proposed Development

3.1	This application seeks full planning permission for a first floor side and rear extension, replacement of tile hanging with render, addition of a flank rooflight and alterations to fenestration.

3.2 	The first floor side and rear extension would be 1.4m wide and 7.4m deep which would wrap around to the rear. The rear extension would be 4.7m wide and 2.15m deep.

3.3	A hipped roof is proposed to the first floor side extension. It would have an eaves height of 4.7m and a maximum height of 8.7m which would extend the existing main ridge by 1.4m. A hipped roof is proposed to the first floor rear extension which would be set down by 1.2m from the main ridge. An eaves height of 4.7m is proposed and a maximum height of 7.5m

3.4	To the rear elevation, at first floor level one large window is proposed and one existing window would be relocated. Two windows are proposed on the flank elevation facing no. 4 Mill Way. One rooflight is proposed to serve the first floor in the flank roofslope facing no. 4 Mill Way.

3.5	As part of the application, the existing flat roof to the garage would be replaced with a hipped roof. An eaves height of 2.5m and maximum height 3.6m is proposed.

3.6	The existing tile hanging at first floor level would be replaced with render.

3.7	Amended plans were received during the application process to show an existing single storey temporary outbuilding and to reduce the depth of the first floor rear extension.

4.	Consultation

4.1	Statutory Consultation

    - National Grid: No response received.

4.2	Public Consultation

4.2.1	Number consulted: 5

4.2.2	Site Notice posted from 05/01/2016- 26/01/2016.

Press notice not required.

    - Number of responses received: 1

    - Summary of responses:

- Impact on the light onto the neighbours patio and garden.

- Privacy of the neighbouring garden.

- Questions regarding the Party Wall Act and footings.

5.		Reason for Delay

5.1		Not applicable.

6.	Relevant Planning Policy, Guidance and Legislation

6.1	The Three Rivers Local Plan

The Core Strategy was adopted on the 17 October 2011 having been through a full public participation process and Examination in Public. Relevant policies include Policies CP1, CP9, CP10 and CP12.

The Development Management Policies Local Development Document (LDD) was adopted on 26 July 2013 after the Inspector concluded that it was sound following Examination in Public which took place in March 2013. Relevant policies include DM1, DM6, DM13 and Appendices 2 and 5.

6.2	National Planning Policy Framework (NPPF)

On 27 March 2012, the framework of government guidance in the form of Planning Policy Statements and Planning Policy Guidance Notes was replaced by the National Planning Policy Framework (NPPF). The application has been considered against the policies of the Core Strategy (adopted October 2011) and the Development Management Policies Local Development Document (adopted July 2013) and the Site Allocations Local Development Document (adopted November 2014) as well as government guidance. The policies of Three Rivers District Council reflect the content of the NPPF.

6.3	Other

The Localism Act received Royal Assent on 15 November 2011. The growth and Infrastructure Act achieved Royal Assent on 25 April 2013.

The Wildlife and Countryside Act 1981 (as amended), the Conservation of Habitats and Species Regulations 2010, the Natural Environment and Rural Communities Act 2006 and the Habitat Regulations 1994 may also be relevant.

7.	Planning Analysis

7.1	Design and Impact on Street Scene

7.1.1	Policy CP1 of the Core Strategy seeks to promote buildings of a high enduring design quality that respect local distinctiveness and Policy CP12 of the Core Strategy relates to design and states that in seeking a high standard of design, the Council will expect development proposals to ‘have regard to the local context and conserve or enhance the character, amenities and quality of an area’. Policy DM1 and Appendix 2 of the Development Management Policies document set out that development should not have a significant impact on the visual amenities of an area and that extensions should respect the existing character of the dwelling, particularly with regard to the roof form, positioning and style of windows and doors, and materials.

7.1.2	The proposed first floor side extension on the south east flank would be set in 1.2m from the flank boundary, which would comply with the Design Guidelines set out in Appendix 2. A 1.2m distance would be sufficient to avoid a terracing effect and to maintain spacing appropriate to the area. There are other side extensions to properties in the vicinity of the application site. As a result it is not considered that the development would appear out of character with the surrounding street scene and would not appear unduly prominent so as to cause harm to the character or appearance of the area or pair.

7.1.3	The first floor side extension would continue the main dwelling ridge height and would maintain the existing roof design. The alterations to the roof would reflect the style of the existing roof and it is not considered that the proposed roof form would adversely affect the character or appearance of the area.

7.1.4	The first floor rear extension would be 4.7m wide and 2.15m deep. The proposed first floor rear extension would be built 1.9m off of the boundary with the neighbouring property no. 8 and 1.2m off of the boundary with no. 4. A hipped roof is proposed which would be set down from the main ridge height by 1.2m. An eaves height of 4.7m is proposed and a maximum roof height of 7.5m. The proposed first floor rear extension would not be visible from Mill Way and would therefore not appear out of character in the street scene or appear unduly prominent.

7.1.5	Although the adjoining dwelling has tile hanging at first floor level, a number of neighbouring dwellings along Mill Way are white rendered. The neighbouring pair of semi detached dwellings to the south east are finished in different materials. No. 4 Mill Way has a similar tile hanging and render finish as existing on the application site and no. 2 Mill Way is finished in brown pebble dash. Therefore the proposed replacement with render would not be out of character in the streetscene.

7.1.6	The first floor side and rear extension and replacement tile hanging with render would not have an adverse impact on the character or appearance of the host dwelling, street scene or area and would be acceptable in accordance with Policies CP1 and CP12 of the Core Strategy and Policy DM1 and Appendix 2 of the Development Management Policies LDD.

7.2	Impact on Neighbours

7.2.1	Policy CP12 of the Core Strategy states that development should ‘protect residential amenities by taking into account the need for adequate levels and disposition of privacy, prospect, amenity and garden space’. Policy DM1 and Appendix 2 of the Development Management Policies document set out that development should not result in loss of light to the windows of neighbouring properties nor allow overlooking, and should not be excessively prominent in relation to adjacent properties. Appendix 2 goes on to state that; "Extensions should not result in loss of light to the windows of neighbouring properties nor allow overlooking". Two-storey rear extensions should not intrude a 45 degree line taken from a point on the joint boundary level with the rear wall of the neighbours.

7.2.2		The first floor side extension would be located closest to the boundary with No. 4 Mill Way. The first floor side extension proposed would not project beyond the existing front building line and would be set 1.2 metres off the boundary. The first floor flank of No. 4 is set approximately 2.5m off the boundary, which combined with the 1.2 spacing at the application site is considered sufficient to ensure that significant overshadowing or loss of light would not occur.

7.2.3		Two windows are proposed in the flank elevation of the first floor side extension. Plans show that one would be fitted with obscure glazing and one would be fitted with clear glazing. A condition would be placed on any permission, requiring both windows to be fitted with obscure glazing and top level opening only. As a result of the additional windows would not allow for additional significant overlooking. One rooflight is proposed in the flank roofslope facing No. 4 Mill Way, this would serve the first floor level and due to the separation and it’s location in the roofslope, it would not allow for significant overlooking towards the neighbouring dwelling no. 4.

7.2.4	The first floor rear extension would project 2.15m deeper than the existing rear elevation and would be set 1.9m off of the boundary with the neighbouring dwelling, No. 8 and 1.2m off of the boundary with No. 4. As mentioned above, Appendix 2 of the DMP LDD outlines that two storey rear and side extensions should not generally intrude a 45 degree splay line drawn across the rear from the point on the boundary level with the rear wall of the adjacent property. This principle is dependant on the spacing and relative positions of properties and consideration will be given to the juxtaposition of properties, land levels and positions of windows and development on neighbouring properties. The 45 degree angle taken from the boundary line adjacent to the original rear building line of No. 8 Mill Way would lead to no intrusion and the neighbouring property to the north west, No. 8 has been extended to the rear at ground floor. The 45 degree angle taken from the boundary line adjacent to the original rear building line of No. 4 Mill Way would lead to a slight intrusion of 0.7m.

7.2.5	Notwithstanding this, taken from the point level with the neighbouring property’s (4 Mill Lane) side/rear extension, the proposed extension would lead to no intrusion. As such the proposed rear extension would not appear overbearing or cause significant loss of light to the neighbouring dwellings.

7.2.6	As part of the proposal, the garage roof would be altered to include a hipped roof. The roof would be at ground floor level and would be hipped away from the neighbouring dwelling, no. 4 Mill Way. Therefore the proposed hipped roof would not lead to any significant overshadowing or appear overbearing.

7.2.7	Given the siting, scale and design of first floor side and rear extension, replacement of tile hanging with render, addition of flank rooflight and alterations to fenestration, the proposed would not result in any significant adverse impact on any neighbouring dwelling and the development would therefore be acceptable in accordance with Policies CP1 and CP12 of the Core Strategy and Policy DM1 and Appendix 2 of the Development Management Policies LDD.

7.3	Parking and Amenity Space

7.3.1		Policy CP12 of the Core Strategy states that development should take into account the need for adequate levels and disposition of privacy, prospect, amenity and garden space. Core Strategy Policy CP10 sets out that development should make adequate provision for car and other vehicle parking and Policy DM13 and Appendix 5 of the Development Management Policies document set out parking standards.

7.3.2	The proposed plans show no increase in the number of bedrooms. Policy DM13 and Appendix 5 of the Development Management Policies document advises that a 3 bedroom dwelling should provide 2 assigned spaces. The hardstanding to the front of the dwelling and the garage would continue to accommodate parking for two cars and therefore the proposal would comply with Policy DM13.

7.3.3	A rear garden of approximately 140sqm would be retained. Adequate amenity space provision would therefore be maintained for current and future occupiers.

7.4	Trees and Landscaping

7.4.1	No significant trees would be affected by the proposed development.

7.5	Biodiversity

7.5.1	Section 40 of the Natural Environment and Rural Communities Act 2006 requires Local Planning Authorities to have regard to the purpose of conserving biodiversity. This is further emphasised by regulation 3(4) of the Habitat Regulations 1994 which state that Councils must have regard to the strict protection for certain species required by the EC Habitats Directive. The Habitats Directive places a legal duty on all public bodies to have regard to the habitats directive when carrying out their functions.

7.5.2	The protection of biodiversity and protected species is a material planning consideration in the assessment of this application in accordance with Policy CP9 of the Core Strategy and Policy DM6 of the Development Management Policies document. National Planning Policy requires Local Authorities to ensure that a protected species survey is undertaken for applications where biodiversity may be affected prior to the determination of a planning application. A Biodiversity Checklist has been submitted with the application and states that no protected species or biodiversity interests will be affected as a result of the application. The site is not in or located adjacent to a designated wildlife site and the Local Planning Authority is not aware of any records of protected species within the immediate area that would necessitate further surveying work being undertaken.  However, given that the works would affect the roofspace, an informative on any consent would advise the applicant of what to do should bats be discovered during the course of development.

8.	Recommendation

8.1	That PLANNING PERMISSION BE GRANTED subject to the following conditions:-

C1	The development hereby permitted shall be begun before the expiration of three years from the date of this permission.

Reason: In pursuance of Section 91(1) of the Town and Country Planning Act 1990 and as amended by the Planning and Compulsory Purchase Act 2004.

C2	The development hereby permitted shall be carried out in accordance with the following approved plans: MW001 Rev A, MW002 Rev A, MW003 Rev A, MW004.

Reason: For the avoidance of doubt and in the proper interests of planning and in the interests of the visual amenities of the locality and residential amenity of neighbouring occupiers, in accordance with Policies CP1, CP9, CP10 and CP12 of the Core Strategy (adopted October 2011) and Policies DM1, DM6, DM13 and Appendices 2 and 5 of the Development Management Policies LDD (adopted July 2013).

C3	Before the building operations hereby permitted are commenced, samples and details of the proposed external materials shall be submitted to and approved in writing by the Local Planning Authority and no external materials shall be used other than those approved.

Reason: This is a pre commencement condition to ensure that the external appearance of the building is satisfactory in accordance with Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policies DM1 and Appendix 2 of the Development Management Policies LDD (adopted July 2013).

C4	Notwithstanding the provisions of the Town and Country (General Permitted Development) Order 2015 (or any other revoking and re-enacting that order with or without modification), no windows/dormer windows or similar openings [other than those expressly authorised by this permission] shall be constructed in the elevations or roof slopes of the extension/development hereby approved.

Reason: To safeguard the residential amenities of neighbouring properties in accordance with Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM1 and Appendix 2 of the Development Management Policies LDD (adopted July 2013).

C5	Before the first occupation of the building/extension hereby permitted the two windows in the first floor flank elevation facing no. 4 Mill Way; shall be fitted with purpose made obscured glazing and shall be top level opening only at 1.7m above the floor level of the room in which the window is installed.  The windows shall be permanently retained in that condition thereafter.

Reason: To safeguard the amenities of the occupiers of neighbouring residential properties in accordance with Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM1 and Appendix 2 of the Development Management Policies LDD (adopted July 2013).

Informatives:

I1	With regard to implementing this permission, the applicant is advised as follows:

All relevant planning conditions must be discharged prior to the commencement of work. Requests to discharge conditions must be made by application form; the relevant form is available on the Council's website (www.threerivers.gov.uk). Fees are £97 per request (or £28 where the related permission is for extending or altering a dwellinghouse or other development in the curtilage of a dwellinghouse). Please note that requests made without the appropriate fee will be returned unanswered.

There may be a requirement for the approved development to comply with the Building Regulations. The Council's Building Control section can be contacted on telephone number 01923 727132 or at the website above for more information and application forms.

Where possible, energy saving and water harvesting measures should be incorporated. Information on this is also available from the Council’s Building Control section. Any external changes to the building which may be subsequently required should be discussed with the Council’s Development Management Section prior to the commencement of work.

I2	The applicant is reminded that the Control of Pollution Act 1974 stipulates that construction activity (where work is audible at the site boundary) should be restricted to 0800 to 1800 Monday to Friday, 0900 to 1300 on Saturdays and not at all on Sundays and Bank Holidays.

I3	The Local Planning Authority has been positive and proactive in its consideration of this planning application, in line with the requirements of the National Planning Policy Framework and in accordance with the Town and Country Planning (Development Management Procedure) (England) Order 2015. The Local Planning Authority suggested modifications to the development during the course of the application and the applicant submitted amendments which result in a form of development that maintains/improves the economic, social and environmental conditions of the District.

I4	Bats are protected under domestic and European legislation where, in summary, it is an offence to deliberately capture, injure or kill a bat, intentionally or recklessly disturb a bat in a roost or deliberately disturb a bat in a way that would impair its ability to survive, breed or rear young, hibernate or migrate, or significantly affect its local distribution or abundance; damage or destroy a bat roost; possess or advertise/sell/exchange a bat; and intentionally or recklessly obstruct access to a bat roost.

If bats are found all works must stop immediately and advice sought as to how to proceed from either of the following organisations:

The UK Bat Helpline: 0845 1300 228

Natural England: 0845 6014523

Herts & Middlesex Bat Group: www.hmbg.org.uk

(As an alternative to proceeding with caution, the applicant may wish to commission an ecological consultant before works start to determine whether or not bats are present. A list of bat consultants can be obtained from Hertfordshire Ecology on 01992 555220).