14.	16/0038/FUL – Single storey rear extension; loft conversion including hip-to-gable alteration, rear dormer and front rooflights; and additional hardstanding to frontage at 41 LUDLOW WAY, CROXLEY GREEN, RICKMANSWORTH, WD3 3SJ for Mr R Lewington

(DCES)

| Parish: Croxley Green Parish Council | Ward: Durrants |
| --- | --- |
| Expiry Statutory Period: 4 March 2016 | Officer: Denis Toomey |
| Recommendation: That planning permission be approved |  |
| Reason for consideration by the Committee: The applicant is related to an employee of the Council |  |

1	Relevant Planning History

1.1	No relevant planning history.

2.	Detailed Description of Application Site

2.1	The application dwelling is a semi-detached bungalow on the north side of Ludlow Way, Croxley Green. Ludlow Way is a residential street characterised predominantly by bungalows, although there are some two storey dwellings. Many of the bungalows have been previously altered and extended and many have loft conversions including hip-to-gable alterations and rear dormer windows.

2.2	The application dwelling comprises pebbledash rendering with aspects of red brick to the front. The rear of the property is rendered. It has a hipped tiled roof with a gable-ended projection to the front elevation and a hipped projection to the rear elevation. The application dwelling is set back from the highway by approximately 5m. Hardstanding is located to the front of the application site and can accommodate 1 car. The rear of the property contains a lean-to conservatory.

2.3	The adjoining property east of the application dwelling (No.43 Ludlow Way) is built on a similar land level. The boundary between both properties consists of a hedge that measures approximately 1m in height. The neighbouring property west of the application dwelling (No.39 Ludlow Way) is built on a slightly higher land level. The rear of this neighbouring property contains a rear dormer and single storey rear extension. An approximately 1.5m high wooden fence forms the boundary between both properties.

3.	Detailed Description of Proposed Development

3.1	The application seeks full planning permission for a single storey rear extension; loft conversion including hip-to-gable alteration, rear dormer and front rooflights; and additional hardstanding to the frontage.

3.2	The single storey extension would in part replace the existing conservatory and would match the stepped rear building line of the original dwelling, measuring 3.6m in depth to result in an L-shaped footprint.  The extension would hold a maximum height of 3.8m and a width of 6.8m. The roof would include a hipped feature on the western portion of the extension along with a crown roofed design on the eastern portion. The rear elevation of the extension would include a patio door and three-casement window and alterations would be made to the fenestration to the west flank of the dwelling resulting in two windows and a door at ground floor level. A rooflight would be included within the western roof slope of the extension. A roof lantern would be inserted into the crown roof element of the extension.

3.3	The proposed development would include a loft conversion with hip to gable extension with a rear dormer and two rooflights would be inserted into the front roof slope of the property. The dormer would hold a depth of 3m, a width of 5.9m and a height of 2.3m. A bathroom and bedroom window would be included to the rear elevation of the dormer.

3.4	The existing hardstanding would be increased to provide parking for two vehicles.

4.	Consultation

4.1	Statutory Consultation

4.1.1	Croxley Green Parish Council - made the following comments:

No grounds for objection.

4.1.2	National Grid – no comments received.

4.2	Public Consultation

4.2.1	Number consulted:	8

Number of responses:		0

4.2.2	Site Notice not required.

4.2.3	Press Notice not required.

4.3	Summary of Representations

4.3.1	None received.

5.	Reason for Delay

5.1	Not applicable.

6.	Relevant Local Planning Policies:

6.1	National Planning Policy Framework (NPPF)

6.1.1	On 27 March 2012, the framework of government guidance in the form of Planning Policy Statements and Planning Policy Guidance Notes was replaced by the National Planning Policy Framework (NPPF). The adopted policies of Three Rivers District Council reflect the content of the NPPF.

6.2	The Three Rivers Local Plan Core Strategy:

6.2.1	The Core Strategy was adopted by the Council on 17 October 2011.  Relevant Policies include: CP1, CP9, CP10 and CP12.

6.3	Development Management Policies LDD:

6.3.1	The Development Management Policies LDD was adopted on 26 July 2013 having been through a full public participation process and Examination in Public.  Relevant policies include: DM1, DM6 and DM13 and Appendices 2 and 5.

6.4	Other

6.4.1	The Localism Act received Royal Assent on 15 November 2011.  The growth and Infrastructure Act achieved Royal Assent on 25 April 2013.

6.4.2	The Wildlife and Countryside Act 1981 (as amended), the Conservation of Habitats and Species Regulations 2010, the Natural Environment and Rural Communities Act 2006 and the Habitat Regulations 1994 may also be relevant.

7.	Analysis

7.1	Impact on Character & Appearance

7.1.1	Policy CP12 of the Core Strategy (adopted October 2011) relates to the ‘Design of Development’ and states that the Council will expect all development proposals to have regard to the local context and conserve or enhance the character, amenities and quality of an area.  Appendix 2 of the Development Management Policies LDD (adopted July 2013) sets out design criteria for residential development that aim to ensure that alterations and extensions do not lead to a gradual deterioration in the quality of the built environment.

7.1.2	Policy DM1 and Appendix 2 of the Development Management Policies LDD (adopted July 2013) set out that development should not have a significant impact on the visual amenities of the area.  The design guidelines within Appendix 2 state that generally, the maximum depth of single storey rear extensions to semi-detached dwellings should be 3.6m. The rear building line of the application dwelling contains a stepped rear building line. The extension would have a depth of 3.6m from the original rear building line of the dwelling replicating the original stepped design. The extension would not extend beyond the western flank elevation of the property and would also be set down from the existing ridge height. As the extension is to the rear of the property it would not be prominent from the street scene. The roof design of the extension would also not be out of character within the area as No. 39 contains a rear extension of a similar design. Due to the design and scale of the extension, it would not impede on the overall character of the application dwelling or wider area.

7.1.3	Appendix 2 of the Development Management Policies LDD advises that in order to prevent a terracing effect and maintain an appropriate spacing between properties in character with the locality; two storey side extensions may be positioned on the flank boundary provided that the first floor element is set in by a minimum of 1.2m.  The proposed alteration to the roof would result in additional built form to the upper portion of the property. As a result of this roof alteration, the roof would be set in from the west flank boundary by 1.3m. The hip to gable extension would comply with the 1.2m guideline and is considered not to impede on the overall character of the application property or to appear prominent within the street scene.

7.1.4	Appendix 2 also advises that hip to gable extensions are discouraged in case of semi-detached properties where they may unbalance the pair and result in loss of symmetry. In some cases, roof forms in a street may be uniform and therefore this type of alteration may erode the group value of the street.

7.1.5	The proposed hip to gable alteration would not have a detrimental impact on the street scene. There have been numerous similar alterations, including at nos. 23, 25, 33 and 39 Ludlow Way which result in unbalanced semi-detached pairs or semi-detached pairs with gable ended roof forms. Therefore whilst the pair would be unbalanced, given the existing varied nature of roof forms in the immediate vicinity it is not considered that the proposal would appear out of character or result in demonstrable harm to the character or appearance of the street scene.

7.1.6	In relation to dormer windows, Appendix 2 sets out that these should always be subordinate to the main roof. They should be set below the existing ridge level, set in from either end of the roof and set back from the plane of the front or rear wall and the roof form should respect the character of the house if possible. The rear dormer would be relatively large but would be set down by 0.3m from the main ridge and would be set in from the western side elevation of the new hip to gable extension. It would also reflect the dimensions of the rear dormer at the neighbouring dwelling. Therefore it is not considered that the dormer would have a significant adverse impact on the character or appearance of the host dwelling or wider area.

7.1.7	The rooflights proposed in the front roof slope of the dwelling would be subordinate to the main roof slope and would not significantly project above the roof slope. They reflect the design of the front rooflights in the neighbouring dwellings at nos. 37 and 39 Ludlow Way and would not result in any adverse impact on the character or appearance of the host dwelling or street scene.

7.1.8	The additional hardstanding would be block paved and would reduce soft landscaping to the front of the property, however, large driveways are a common feature and it would not therefore appear out of character.

7.1.9	Subject to the extensions and alterations being finished in materials to match the existing dwelling, the proposal would not result in demonstrable harm to the character or appearance of the application dwelling, streetscene or wider area and the proposal is considered to be acceptable in accordance with Policies CP1 and CP12 of the Core Strategy and Policy DM1 and Appendix 2 of the Development Management Policies LDD.

7.2	Impact on Residential Amenity

7.2.1	Policy CP12 of the Core Strategy advises that development will be expected to protect residential amenity.  Appendix 2 of the Development Management Policies LDD comments that all developments are expected to maintain acceptable standards of privacy for both new and existing residential buildings and extensions should not result in loss of light to the windows of neighbouring properties nor allow overlooking.

7.2.2	The single storey rear extension would comply with the 3.6m guidance depth set out at Appendix 2 of the Development Management Policies LDD. The extension would be set in from the boundary between the application dwelling and No. 39 by 1.3m and would not be deeper than the existing extension at No.39. The extension would be built in close proximity to the boundary with No. 43. However the extension would comply with the 3.6m guideline and the roof would be hipped away from this neighbour with a maximum height of 3m which is not considered to result in significant harm to the neighbouring property.

7.2.3	The proposed window and patio doors within the rear of the extension would be directed towards the rear garden of the application dwelling and therefore unacceptable overlooking would not occur.  The alterations to windows in the west elevation would result in three windows, serving a kitchen and bathroom, replaced with two windows serving a kitchen and bathroom and in a similar position to the existing windows.  The window alterations would therefore not result in additional overlooking compared to the existing site circumstances.

7.2.4	The proposed hip to gable extension would be set in by 1.3m from the west boundary. There would be no increase in the ridge height of the dwelling and the dwelling would not be extended any further to the west than the existing flank wall of the dwelling. As such, there would not be any significant loss of light to habitable rooms of neighbouring properties.  A first floor window would be included in the western elevation of the proposed gable-end; no. 39 also contains a similar window. The plans indicate that this window would be obscured glazed and would only open 1.7m above the floor in which it would be installed.  Subject to a condition requiring the window to be installed in such a manner, the window would not result in unacceptable loss of privacy to the neighbour.

7.2.5	The proposed fenestration that would be included on the rear elevation of the dormer would be directed towards the rear garden of the application dwellinghouse and therefore no unacceptable overlooking would occur.

7.2.6	The two rooflights proposed in the front roof slope would be flush with the roof slope. As a result of the separation provided by the highway, it is not considered that these rooflights would result in any adverse impacts on dwellings opposite the application site.

7.2.7	Overall, it is not considered that the proposed extensions would result in a significant adverse impact on neighbouring dwellings and the development would be acceptable in accordance with Policy CP12 of the Core Strategy and Policy DM1 and Appendix 2 of the Development Management Policies LDD.

7.3	Amenity Space

7.3.1	Policy CP12 of the Core Strategy states that development should take into account the need for adequate levels and disposition of privacy, prospect, amenity and garden space.  Specific standards for amenity space are set out in Appendix 2 of the Development Management Policies LDD.

7.3.2	The proposal would result in a three bedroom property. Appendix 2 of the Development Management Policies LDD advises that a three bedroom property should retain minimum amenity space levels of 84sqm. The application dwelling would retain a rear amenity of space of approximately 138sqm and would therefore comply.

7.4	Highways, Parking & Access

7.4.1	Policy DM13 of the Development Management Policies LDD requires development to make provision for parking in accordance with the parking standards set out at Appendix 5 of the Development Management Policies LDD.  The parking standards state that a dwelling of four bedrooms or more should have a total of three parking spaces.

7.4.2	The proposed development would result in a three bedroom property. Appendix 5 of the Development Management Policies LDD advises that a three bedroom property should contain on site parking provision for two vehicles. The proposed hardstanding to the front of the property would accommodate two vehicles and therefore the proposal complies.  A condition would be attached to ensure that the hardstanding is provided.

7.5	Trees

7.5.1	Policy DM6 of the Development Management Policies LDD sets out that development proposals should seek to retain trees and other landscape and nature conservation features, and that proposals should demonstrate that trees will be safeguarded and managed during and after development in accordance with the relevant British Standards.

7.5.2	There are no trees within the application site or neighbouring properties that would be harmed by the proposed development.

7.6	Wildlife & Biodiversity

7.6.1	Section 40 of the Natural Environment and Rural Communities Act 2006 requires Local Planning Authorities to have regard to the purpose of conserving biodiversity.  This is further emphasised by regulation 3(4) of the Habitat Regulations 1994 which state that Councils must have regard to the strict protection for certain species required by the EC Habitats Directive.  The Habitats Directive places a legal duty on all public bodies to have regard to the habitats directive when carrying out their functions.

7.6.2	The protection of biodiversity and protected species is a material planning consideration in the assessment of this application in accordance with Policy CP9 of the Core Strategy and Policy DM6 of the Development Management Policies LDD.  National Planning Policy requires Local Authorities to ensure that a protected species survey is undertaken for applications where biodiversity may be affected prior to the determination of a planning application.  A Biodiversity Checklist has been submitted with the planning application and states that a Biodiversity Survey and Assessment is not required.  It is therefore unlikely that there would be any harm to any protected species. However, given that the works would affect the roofspace, and informative would advise the applicant of what to do should bats be discovered during the course of development.

8.	Recommendation

- 8.1		That PLANNING PERMISISON BE GRANTED subject to the following conditions:

C1	The development hereby permitted shall be begun before the expiration of three years from the date of this permission.

Reason:  In pursuance of Section 91(1) of the Town and Country Planning Act 1990 and as amended by the Planning and Compulsory Purchase Act 2004.

C2	The development hereby permitted shall be carried out in accordance with the following approved plans: Drawing Numbers: TRDC 001 (Location Plan), LUD – 41/1 and LUD-41/2A.

Reason: For the avoidance of doubt and in the proper interests of planning in accordance with Policies CP1, CP9, CP10 and CP12 of the Core Strategy (adopted October 2011) and Policies DM1, DM6, DM13 and Appendices 2 and 5 of the Development Management Policies LDD (adopted July 2013).

C3	Unless specified on the approved plans, all new works or making good to the retained fabric shall be finished to match in size, colour, texture and profile those of the existing building.

Reason: To ensure that the external appearance of the building is satisfactory in accordance with Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM1 and Appendix 2 of the Development Management Policies LDD (adopted July 2013).

C4	Before the first occupation of the extension hereby permitted the first floor window on the western flank elevation of the dwelling shall be fitted with purpose made obscured glazing and shall be top level opening only at 1.7m above the floor level of the room in which the window is installed. The window shall be permanently retained in that condition thereafter.

Reason: To safeguard the amenities of the occupiers of neighbouring residential properties in accordance with Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM1 and Appendix 2 of the Development Management Policies LDD (adopted July 2013).

C5	The development shall be constructed in accordance with the approved details with the parking spaces provided prior to the occupation of the extensions hereby permitted.  The parking spaces shall thereafter be kept permanently available for the use of residents and visitors to the site.

Reason: To minimise danger, obstruction and inconvenience to users of the highway and to ensure adequate off-street parking in accordance with Policies CP1 and CP10 of the Core Strategy (adopted October 2011) and Policy DM13 and Appendix 5 of the Development Management Policies LDD (adopted July 2013).

8.2	Informatives

I1	With regard to implementing this permission, the applicant is advised as follows:

All relevant planning conditions must be discharged prior to the commencement of work. Requests to discharge conditions must be made by application form; the relevant form is available on the Council's website (www.threerivers.gov.uk). Fees are £97 per request (or £28 where the related permission is for extending or altering a dwellinghouse or other development in the curtilage of a dwellinghouse). Please note that requests made without the appropriate fee will be returned unanswered.

There may be a requirement for the approved development to comply with the Building Regulations. The Council's Building Control section can be contacted on telephone number 01923 727132 or at the website above for more information and application forms.

Where possible, energy saving and water harvesting measures should be incorporated. Information on this is also available from the Council’s Building Control section. Any external changes to the building which may be subsequently required should be discussed with the Council’s Development Management Section prior to the commencement of work.

I2 	The applicant is reminded that the Control of Pollution Act 1974 stipulates that construction activity (where work is audible at the site boundary) should be restricted to 0800 to 1800 Monday to Friday, 0900 to 1300 on Saturdays and not at all on Sundays and Bank Holidays.

I3	The Local Planning Authority has been positive and proactive in its consideration of this planning application, in line with the requirements of the National Planning Policy Framework and in accordance with the Town and Country Planning (Development Management Procedure) (England) Order 2015. The development maintains/improves the economic, social and environmental conditions of the District.

I4	Bats are protected under domestic and European legislation where, in summary, it is an offence to deliberately capture, injure or kill a bat, intentionally or recklessly disturb a bat in a roost or deliberately disturb a bat in a way that would impair its ability to survive, breed or rear young, hibernate or migrate, or significantly affect its local distribution or abundance; damage or destroy a bat roost; possess or advertise/sell/exchange a bat; and intentionally or recklessly obstruct access to a bat roost.

If bats are found all works must stop immediately and advice sought as to how to proceed from either of the following organisations:

The UK Bat Helpline: 0845 1300 228

Natural England: 0845 6014523

Herts & Middlesex Bat Group: www.hmbg.org.uk

(As an alternative to proceeding with caution, the applicant may wish to commission an ecological consultant before works start to determine whether or not bats are present. A list of bat consultants can be obtained from Hertfordshire Ecology on 01992 555220).