6.	16/2040/AOD – Approval of Details: Details pursuant to Condition 18 of hybrid planning permission 16/0005/FUL comprising layout, scale, appearance and landscaping for Phase 1B (Station Approach), at STATION APPROACH, SOUTH OXHEY, HERTFORDSHIRE for Countryside Properties PLC

(DCES)

| Parish: Watford Rural | Ward: Carpenders Park |
| --- | --- |
| Expiry Statutory Period: 27 January 2017 (Agreed extension) | Officer: Claire Westwood |
| Recommendation: APPROVAL subject to conditions |  |
| Reason for consideration by the Committee: Council interest in land. |  |

JANUARY 2017 COMMITTEE UPDATE

The application was deferred at the Planning Committee meeting on 15 December 2016 due to concerns with the proposed parking arrangements including pick up and drop off.

To clarify, the proposal includes:

- Loading Bay & Station Drop-Off/Pick-Up: The 2 areas originally shown as loading bays have been re-designated as dual use loading bay and drop-off/pick-up areas.  The plans have been updated to reflect this and both areas will be clearly signed.  A condition on any grant of consent will require details of the signage and management of the dual use bays to be submitted for approval (Condition C12).

The dual use bays have a total length of approximately 34 metres and could accommodate either 7 cars or 3/4 delivery vehicles (or a combination of both) at any one time.  It is considered that this would achieve an appropriate balance to support both users of the station and the commercial premises.

It is also noted that there is a separate access to the station from Carpenders Park.

- Taxi Rank: The taxi rank as amended will provide space for 4 vehicles.  The Licensing Officer has noted the amended plans and welcomes the additional taxi rank space which they consider should provide sufficient capacity.

- Disabled Spaces: 2 disabled spaces of appropriate size are proposed within Station Approach in addition to the short stay and dual use loading/drop-off and pick-up spaces.

- Short Stay Parking: 3 short stay parking bays are proposed. These will be time limited and are intended for those using the commercial premises.

- Station Approach Car Park: The 34 existing spaces are to be re-provided and consideration is being given to the appropriate time limiting in this car park.  At the committee in December Members raised concerns regarding possible conflict on the forecourt loop road due to vehicles entering it to find that the car park was full.  As such the developer will be required to provide signage (Condition C12) to indicate the availability of spaces.  Such signage should be suitably located so that users do not enter the forecourt loop road unnecessarily.

- Wider South Oxhey Re-development: It is relevant to note that Station Approach (Phase 1B) forms part of the wider South Oxhey re-development.  With regards to commercial parking, the HPP set out that a total of 140 car parking spaces are proposed across South Oxhey Central.  This is in addition to parking proposed to serve the residential elements of the scheme.  It should also be noted that the taxi stand, 3 short stay parking spaces, 2 disabled parking spaces and dual use bays for loading/drop-off and pick-up are in addition to the 140 spaces specified in the HPP.

- Summary: The overall parking provision for the commercial elements of the South Oxhey re-development exceeds adopted standards.  The current RM application proposes taxi stand, 3 short stay parking spaces, 2 disabled parking spaces and dual use bays for loading/drop-off and pick-up to serve Station Approach area.  Subject to conditions to secure appropriate signage and management, the provision at Station Approach is considered to strike an appropriate balance to meet the needs of both those accessing the Station and adjacent shops whilst ensuring that the vision to improve the public realm across South Oxhey Central, particularly at Station Approach in order to provide a pedestrian friendly environment and welcoming setting for the new retail provision and Carpenders Park Station is supported.

DECEMBER 2016 COMMITTEE UPDATE (As per December Agenda)

A preliminary report was considered by Members at the Planning Committee meeting on 17 November 2016.

The main points raised at that meeting related to:

- Public Realm/Management and maintenance of landscaping.

- Pick-up / Drop-off facilities.

- Taxi provision.

- Refuse provision.

- Cycle parking.

- Need to ensure appropriate lighting.

The report below has been updated where appropriate, however, for ease of reference, a summary response to these points from the applicant (in italics) and officers is provided below:

- Public Realm/Management and maintenance of landscaping

Members will appreciate that the existing road loop on Station Approach provides a range of transport functions, including: a taxi stand for approximately 6 taxis; 20 short stay parking spaces; two disabled parking spaces; a bus stop/stand for the W19 and R17 bus services; a drop off facility for up to three vehicles; and kerbside servicing for the existing retail units. It is as a result of these functions that the existing public space (or lack thereof) is geared towards vehicular access.

The hybrid planning application was underpinned by the vision to improve the public realm across the South Oxhey estate, particularly Station Approach in order to provide a pedestrian friendly environment and welcoming setting for the new retail provision and Carpenders Park Station. The illustrative proposals (which formed part of the hybrid application) for Station Approach envisaged the shortening of the forecourt loop road to facilitate the creation of a public space (Station Square) in front of the Station.

As a result, the illustrative layout proposed a taxi stand for approximately three taxis; two disabled parking spaces; a dedicated drop-off facility for up to three vehicles; retention of the existing bus stop; and loading bays on the outside of the shortened vehicular loop. The short stay parking previously within the loop was shown as relocated to a dedicated short-stay public car park within the north-west corner of the site, providing convenient and easy access to the station and centre.

Following the grant of the hybrid planning consent, further detailed design work has been undertaken in preparation of the reserved matters application to develop and refine the proposals for Station Approach in order to successfully deliver the vision. The broad principles of the illustrative scheme remain unchanged and this was recognised by the Committee. The submitted reserved matters application proposes to further shorten the loop road to provide an enlarged and improved public space in front of the station. The proposed layout presented to the Committee on 17th November provided a stand for three taxis; three short stay parking spaces; two disabled parking spaces; retention of existing bus stop; and dedicated loading bays for the proposed retail units. This has since been reviewed by the applicant following the comments raised, in balance of delivering the vision to improve the public realm.

Officer comment regarding management and maintenance of landscaping: Condition 35 of the HPP requires the submission of details of landscaping (including species and size, specification, locating, timing of planting and future management) to be submitted for approval by the LPA in consultation with the Landscape Officer.  Application 16/2067/DIS is pending consideration.

- Pick-up / Drop-off facilities

It is not proposed to further revise the scheme to cater for additional pick-up drop off as the proposed provision is considered adequate to serve the centre. The reduction in road space associated with the revised layout means that kerb space is at a premium and priority has been allocated to disabled access.

Pick-up/drop-off will take place in the short-stay parking bays on Station Approach or in the public car park to the north-west corner of the site.

It is noted that whilst outside the application site Gibbs Couch (north side of the Station) also provides the ability for pick-up/drop-off at any time.

Officer comment: The proposed plans do include short stay parking to provide 3 drop-off/pick-up spaces and 2 disabled parking bays in addition to loading bays to the front of Blocks X and Y.

- Taxi provision

It is recognised that the taxi rank at Station approach currently provides waiting space for up to six taxis. It is noted that demand is unlikely to reach the level whereby there is constant turnover of six taxis. Therefore, the six spaces are primarily for waiting taxis. The taxi service is radio control managed which allows for taxis to be called to the location to collect passengers as required.

The illustrative plans and proposed reserved matters submission proposed to reduce this to three spaces for waiting taxis. This level of provision is considered sufficient to meet the demand for waiting taxis and to allow the taxi rank to adequately function.

Notwithstanding this, the applicant appreciates the proposals will result in a reduction. Therefore, further consideration has been given following the comments raised by TRDC licensing and members’ discussion at 17th November Committee. The applicant has since revised the proposed layout to extend the taxi bay to 19.25m in order to accommodate up to four taxis which is appropriate given the radio control management. This has been achieved without undermining the vision for the new public realm.

- Refuse provision

The provision and required capacity of refuse storage has been confirmed with the Environmental Enforcement Manager at Three Rivers District Council. Each unit will be provided with space so that the Occupier can accommodate one large bin (1100L) and a recycling bin (240L) which meets TRDC requirements for the proposed development. The bin stores have been sensitively designed within the scheme in order to ensure ease of access for occupiers/collection, whilst sited in a secure area protecting the amenity of the public realm and avoiding conflict with its users.

- Cycle parking

Station Approach currently benefits from 10 covered cycle parking spaces located on the central island (surrounded by the road network in the middle of the public realm).

Three Rivers parking standards require the proposed development to provide a minimum of 24 cycle parking spaces, 11 short-term and 13 long-term.

The proposals include a total of 32 cycle parking spaces in order to encourage sustainable modes of transport. A total of 12 short-term spaces will be located either side of the access road to the car park, and 20 long-term covered spaces will be located adjacent to the café. This level of provision exceeds Three Rivers’ minimum standards.

In addition to this it is noted that there are a further eight covered spaces located on the ramp down to the station, outside the planning application boundary, which we understand will remain in situ.

- Need to ensure appropriate lighting

Officer comment: Details of lighting are controlled by condition on the HPP.  The Design Guidelines (A Primary Control Document) state that lighting will be to Secure By Design Standards to ensure sufficient levels are provided for all external areas of the site.  The comments raised by Members at the November Planning Committee are noted and will be taken into consideration by officers when determining any future Discharge of Condition application for lighting.

1.	Relevant Planning History

1.1	12/1594/PREAPP - Re-development of the site to create new retail, residential and community uses. Closed October 2012.

1.2	15/1996/PREAPP - Phased comprehensive redevelopment of the land at South Oxhey (South Oxhey Central, Maylands Road, Hayling Road and Hallowes Crescent) to include the demolition of existing buildings and provision of residential-led mixed use development comprising Use Classes C3, A1, A1/A3, A4, D1 and D2, with associated site preparation/enabling works, transport infrastructure works, landscaping works and provision of car parking. Closed November 2015.

1.3	16/0005/FUL - Hybrid planning application for the phased comprehensive redevelopment of the land at South Oxhey (South Oxhey Central, Maylands Road, Hayling Road and Hallowes Crescent) to include the demolition of existing buildings and provision of residential led mixed use development comprising Use Classes C3, A1/A2/A3/A4/A5 and D1/D2, with associated site preparation/enabling works, transport infrastructure works, landscaping works and provision of car parking.  Permitted 31.05.16.  Implemented with works commenced in relation to Phase 1A.

1.4	Various Discharge of Condition (DIS) applications pursuant to planning application 16/0005/FUL.

1.5	16/2053/NMA - Non Material Amendment to planning permission 16/0005/FUL: Amendment  to the Development Schedule to include up to 550sqm gross internal floorspace for Class B1 (office) use within the defined Town Centre Uses; and amendment to the suite of parameter plans to provide an additional development zone for a single storey building within Phase 1B. Permitted October 2016.

1.6	16/2067/DIS - Discharge of Conditions 29 (Delivery and servicing plan), 35 (Landscaping), 36 (C-Plan energy and sustainability measures), 43 (Flood mitigation and drainage) and 47 (Bicycle parking) and part discharge of conditions 33 (Public open space) and 46 (Highways access and layout) pursuant to planning permission 16/0005/FUL (Phase 1B). Pending.

1.7	16/2264/NMA - Non material amendment to planning permission 16/0005/FUL: Amendment to the Development Schedule to include up to 186sq.m gross internal floorspace for use as a Bookmakers (Sui Generis) within the defined Town Centre Uses. Permitted October 2016.

2.	Site Description

2.1	South Oxhey lies to the north west of London and to the south of Watford. It is segregated from Carpenders Park to the east by the railway line.  Carpenders Park Station is sited between the two, in close proximity to the centre of South Oxhey.

2.2	South Oxhey is essentially a large urban estate almost exclusively made up of Council built properties, although many of these houses are now privately owned.  The estate was built after the Second World War to help alleviate housing pressures in London.

2.3	The current Reserved Matters application relates to the Station Approach area.  The main part of the site has a depth of approximately 70 metres and a width of approximately 75 metres, with the width increasing to approximately 115 metres to the rear of No’s. 39 – 49 Prestwick Road.  This part of the site accommodates a car park.

2.4	The site slopes down from Prestwick Road towards Carpenders Park Station, with the Station entrance set at a lower level.  To the east and outside of the application site there are steps and a ramp providing access to the station.  There is also an under pass which provides pedestrian access to Carpenders Park to the east of the station.

2.5	The existing buildings are located along the northern and southern boundaries of the site (excluding car park area).  To the north at the junction with Prestwick Road is a two-storey brick building (former Barclays Bank) with parking at the rear.  To the rear of this building is access to the existing car park.  Beyond the access is a single storey flat roofed parade containing 4 units.  To the south of the site there is a similar flat roofed parade (4 units) closest to the station, with the corner at the junction with Prestwick Road currently occupied by a car sales room/garage; this includes a single storey building set back from the road and a large forecourt area.

2.6	There are 2 existing vehicular access points from Prestwick Road with a one-way ‘in’ and ‘out’ loop layout.  Adjacent to the northern arm of the access road there are parking bays (1 hour stay) and a taxi rank.  There is a parking bay to the east.  To the southern arm there are 14 parking spaces (1 hour stay) set at an angle to the access road with a further parking bay to the opposite side to the front of existing units.  The central area is laid to hard standing with various items of street furniture including a bus stop (adjacent to Prestwick Road), cycle shelter, bollards, benches, bins, signs and lighting.

2.7.	To the north are two-storey residential properties on Prestwick Road.  No. 49 Prestwick Road (closest to the site) is set at a lower level with a retaining wall adjacent to the site boundary.  To the north of the car park are allotment gardens.  To the east is the railway and Carpenders Park Station with the settlement of Carpenders Park beyond.  To the south is a petrol station with residential properties beyond.  To the west is the existing South Oxhey shopping precinct which forms part of the wider redevelopment area.

3.	Description of Proposed Development

3.1	Hybrid planning permission (HPP) 16/0005/FUL granted consent for the phased comprehensive redevelopment of the land at South Oxhey (South Oxhey Central, Maylands Road, Hayling Road and Hallowes Crescent) to include the demolition of existing buildings and provision of residential led mixed use development comprising Use Classes C3, A1/A2/A3/A4/A5 and D1/D2, with associated site preparation/enabling works, transport infrastructure works, landscaping works and provision of car parking.

3.2	Details for Phase 1B (Station Approach) were approved in outline with details of access agreed.

3.3	The current application seeks approval for the details or ‘Reserved Matters’ (RM) for Phase 1B comprising details of layout, scale, appearance and landscaping and has been prepared in accordance with the approved Primary Control Documents (parameter plans, outline development schedule and Design Guidelines Document) and commitments set out within the Secondary Control Documents.

  - In summary, consent is sought for:

- Demolition of all existing buildings.

- Provision of 1,972sqm of flexible ‘Town Centre’ floorspace (use classes A1-5, D1, D2, B1 and bookmakers) provided in 18 ground floor units spread across four main blocks (W, X, Y and Z) and a single standalone unit (Unit 19) within Station Square, with flexible accommodation above Block X.

- Public realm works to include:

  - Taxi stand for 4 taxis;

  - 3 short stay parking spaces and 2 disabled parking spaces;

  - Re-provision of existing bus stop on Prestwick Road (Bus Stop B);

  - Re-provision of bus stop/stand on the southern side of the loop road (Bus Stop A);

  - Dual use bays for loading/drop-off and pick-up; and

  - Access to adjacent car park.

3.5	An ‘in’ and ‘out’ loop access road is proposed that would extend into the site for approximately 42 metres.  Blocks Y and W would be located to the north with Blocks X and Z to the south.  To the east of the access loop road a further building (Unit 19) is proposed to the front of the station.

3.6	Block Y would have a width of 32 metres and depth of 12.8 metres and would include 5 ground floor units (Units 10 – 14).  The Units would front Station Approach with the corner Unit (Unit 10) also including an active frontage to Prestwick Road.  Block Y would be set back from Prestwick Road by approximately 6.6 metres and set in from the northern boundary by approximately 3 metres, with this rear space (accessed from the east of the Block) providing a shared space for the Units for access for deliveries/service and refuse storage.  Block Y would be single storey with a flat roof stepped down in height from west to east with a maximum height of 6.4 metres decreasing to 5.4 metres to the east.

3.7	Block W would have a width of 22.4 metres and depth of approximately 16.8 metres and would include 4 ground floor units (Units 15 – 18) fronting Station Approach.  Block W would be single storey with a flat roof stepped down from west to east with a maximum height of approximately 5 metres.  An enclosed refuse store is proposed to the west of the Block with access to the rear for service and deliveries.

3.8	Block X would have a width of 43.5 metres and maximum depth of 14.6 metres (decreasing to 11.2 metres).  Block X would be partly single storey with a 3 storey element closest to Prestwick Road.  The building would have a maximum height of approximately 14.5 metres (3 storey element) decreasing to 5.2 metres (single storey element).  Block X would include 7 units at ground floor level (Units 1 – 7) with flexible accommodation above.  Units would front Station Approach with the corner unit (Unit 1) also including an active frontage to Prestwick Road.  Access to the first and second floor level accommodation would be from the west (Prestwick Road).  An access to the east of the Block would provide access for deliveries/servicing and refuse storage to the rear of the Block.  This access would be shared with Block Z to the east.

3.9	Block Z would have a width of approximately 13 metres and depth of approximately 11.2 metres and would include 2 ground floor units (Units 8 and 9) fronting Station Approach.  Block Z would be single storey with a flat roof and maximum height of 5.5 metres.  An enclosed area at the rear would provide for servicing/deliveries and refuse storage and would be accessed to the west of Block Z (access shared with Block X).  A substation would be located to the east of Block Z.

3.10	Across the 4 blocks the predominant material for the elevations is brick with feature brickwork to prominent locations including the corners of Station Approach and Prestwick Road.  The shop fronts are largely glazed with zones for signage to ensure consistency.  Brown roofs are proposed to assist with SuDS for the site and to create a bio-diverse habitat.

3.11	In addition to the 4 main blocks a further single storey building (Unit 19) is proposed to be located to the front of Blocks W and Z within the space to the front of the Station.  Unit 19 would have an unusual triangular footprint with a length of approximately 19 metres and depth of approximately 12 metres.  Unit 19 would have a slightly sloping roof with a maximum height of 5.5 metres creating a covered canopy to the front of the building.  The front (north) part of the unit would be largely glazed with an area for outdoor seating to the front.  The central part of the unit would comprise a covered cycle store which would be open to the sides (west and east).  To the rear an enclosed refuse store is proposed.

3.12	The access road includes a dual use loading/drop-off and pick-up bay, disabled bay and 3 short stay parking spaces to its northern arm with a further disabled bay to the east.  The southern arm includes a dual use loading/drop-off and pick-up bay, taxi rank and Bus Stop A.  Bus Stop B would be located to the site frontage adjacent to Prestwick Road.

3.13	Various public realm works are proposed.  In the central area these include a new shelter; raised angular grass mounds; semi-circular seating; banners; boxed trees; specimen trees; and large planted pots.  In the area around Unit 19 boxed trees and semi-circular seating is proposed.

3.14	The proposals are designed to create a new pedestrian friendly arrival space from Carpenders Park Station.  A shared surface ensures pedestrians and vehicles can share the highway safely whilst creating a safe route between the Station and South Oxhey Central shopping area to the west.

3.15	The Station Square car park to the rear of Block W is retained (a requirement of the hybrid planning permission) with 34 car parking spaces.  Whilst the layout of the car park has been amended to make more efficient use of the space, there is no change to the number of spaces.

3.16	The application is accompanied by:

- CIL Additional Information Form

- Accommodation Schedule

- Local Biodiversity Checklist

- Design & Access Statement

- Energy & Sustainability Statement

- Mechanical & Electrical Design Report

- Statement of Community Involvement

- Planning Statement

- Daylight & Sunlight Assessment

- Transport Assessment Compliance Statement

- Drainage Strategy Statement

3.17	Amended plans have been received during the course of the application to increase the capacity of the proposed taxi rank.  In addition, the 2 areas originally shown as loading bays have been re-designated as dual use loading bay and drop-off/pick-up areas.

4.	Consultation

4.1.	Statutory Consultation

4.1.1	National Grid – [Advisory comments]

Due to the presence of National Grid apparatus in proximity to the specified area, the contractor should contact National Grid before any works are carried out to ensure our apparatus is not affected by any of the proposed works.

4.1.2	Watford Rural Parish Council – No comments received.

4.1.3	Hertfordshire Highways – [No objection]

Notice is given under article 18 of the Town and Country Planning (Development Management Procedure) (England) Order 2015 that the Hertfordshire County Council as Highway Authority does not wish to restrict the grant of permission.

The views of the highway authority have been sought on a Reserved Matters application through the submission of details on the discharge of Condition 18 of hybrid planning permission 16/0005/FUL comprising layout, scale, appearance and landscaping for Phase 1B (Station Approach).

Planning permission 16/0005/FUL was granted by Three Rivers DC on 31st May 2016 and allowed for the phased comprehensive redevelopment of land at South Oxhey (South Oxhey Central, Maylands Road, Hayling Road and Hallowes Crescent) to include demolition of existing buildings and provision of residential-led mixed use development comprising Use  Classes C3, A1/ A2/ A3/ A4/ A5 and D1/ D2 with associated site preparation/ enabling works, transport infrastructure works, landscaping works and provision of car parking.

Condition 18 requires that: No development shall commence within any outline development areas shown as Phases 1B, 23A, 2B, 3A and 3B on approved plan number 1275-O-262 B (Development Area) until details of the appearance, landscaping, layout and scale (Reserved Matters) for each Development Area has been submitted to and approved in writing by the Local Planning Authority. The development shall only be implemented in accordance with the approved details.

The application seeks approval for layout, scale appearance and landscaping to deliver the following elements for Phase 1B:

- demolition of all existing buildings;

- provision of 1,972sqm of flexible town centre floorspace (use classes A1-5, D1, D2 and B1) provided in 19 units spread across four blocks (W, X, Y, and Z);

- public realm works including:

+ taxi stand for 2-3 taxis;

+ 3 short-stay parking spaces and 2 disabled parking spaces;

+ reprovision of existing bus stop on Prestwick Road (Bus Stop B);

+ reprovision of bus stop/ stand on the southern side of the loop road (Bus Stop A);

+ dedicated loading bay for proposed units; and

+ access to the adjacent car park.

Information provided to support the variation of this condition considered by the Highway Authority consisted of:

- completed application form

- covering letter

- Design & Access Statement

-	Planning Statement

-	Transport Statement

-	Proposed site plan drawing

-	Proposed black plan drawing

-	Landscape layout drawing

-	Landscape sections drawing

-	Proposed street scene elevations 1 & 2

-	Proposed street scene elevation 3

The development of the design shown in the documents submitted with this application has been carried out in close cooperation with officers of the Highway Authority with responsibility for development management and the provision of bus infrastructure and services. The impacts of the proposed scheme and its mitigation is analysed in the accompanying Transport Statement document which meets with the approval of the Highway Authority.

Paragraph 32 of the NPPF states that ‘development should only be prevented or refused on transport grounds where the residual cumulative impacts of development are severe’. This Transport Statement demonstrates that the proposals for Station Square are not considered to have a significant impact on local transport networks and as such it is concluded that the development should not be refused on transportation grounds.

4.1.4	Network Rail – No comments received.

4.1.5	London Underground Limited – [No comments to make]

I can confirm that London Underground Infrastructure Protection has no comment to make on this planning application.  However, there are Network Rail assets close to this site.  Please contact the following to query what affect if any your proposals will have on the railway: Asset Protection Anglia Route Network Rail Floor 11 One Stratford Place Stratford London E20 1EJ Telephone number 0203 356 2510 Email: AssetProtectionLNEEM@networkrail.co.uk

4.1.6	TRDC Traffic Engineer – No comments received.

4.1.7	Sustrans – No comments received.

4.1.8	Landscape Officer – [No objection]

I have no objection to the scheme submitted for the above.

4.1.9	Licensing Officer – [Initial verbal comment raising some concerns regarding the reduction in the size of the taxi rank.  The Licensing Officer has noted the amended plans and welcomes the additional taxi rank space which they consider should provide sufficient capacity, although they would also welcome a feeder rank outside of the site and ask that this be considered at future Phases].

4.1.10a	Herts Constabulary – [Advisory comments]

I am generally content with this application and make the following points for consideration, and would encourage some CCTV coverage of the area.

- Low wall at rear of parking spaces:  There is a low wall shown at the rear of parking spaces behind block w.   I would ask that this wall has a brick dragon tooth design on its top to stop it becoming an informal seating position and then a gathering point.

- Unit 19:  I am not sure how high the roof is on the building at the rear, but I would ask that this single story building with long term cycle parking underneath, should have a high enough roof to stop anyone trying to climb onto the roof.

- Public realm seating:  public realm seating is shown.  I would ask that this is single seats or if benches do not have a back and has arms separating individual seats, so as to encourage proper use.

- CCTV:  I would ask for some public realm CCTV generally covering the area, so as to deter Anti-Social Behaviour (ASB) etc in the area.

I hope the above is of use to you in your deliberations and will help the development achieve that aims of the National Planning Policy Framework (NPPF):

- 69 – re safe and accessible environments where crime and disorder, and the fear of crime, do not undermine quality of life or community cohesion.

& the National Planning Practice Guidance (NPPG) under ‘Design’

- 010 – re Sec 17 of the Crime and Disorder Act 1998 – to prevent crime & disorder.

- 011 – re taking proportionate security measures being a central consideration to the planning and delivery of new developments and substantive retrofits.

And CP1 and CP12 of Three Rivers Core Strategy.

However, in the meantime, if you or the applicants have any queries about crime prevention design in relation to the proposals then please feel free to contact me.

4.1.10b	Agent response to Herts Constabulary Comments:

1. Low wall at rear of parking spaces (Block W) – It is intended that rather than a wall, this element will feature a simple curb to delineate the parking area. The exact design of this boundary treatment is envisaged to come forward through detailed landscape/design, which the LPA could secure by condition. On this basis we do not foresee any secured by design concerns with this treatment in comparison with a low level wall in this location.

2. Unit 19 Height – The height from ground level to the underside of eaves of proposed Unit 19’s roof level is 2.9m – with the roof depth in addition to this. This is well out of reach from ground level – prohibiting anyone trying to climb on to the roof.

3. Public Realm Seating – Whilst the detail of the seating proposed does not feature backrests, their shape, being round, does not allow for arm rests. However, this is not a convenient space or seating for rough sleepers.

4. CCTV – There are no standalone CCTV cameras proposed in order to avoid clutter of the public realm. However, the proposed lighting columns are capable of supporting a CCTV connection.

4.1.10c	Herts Constabulary – Further Comments (response to agent comments above):

1. I am content with just a kerb as proposed at this location.

2. As the roof will be 2.9 metres high as a minimum I am content.

3. I am content with the agents comments regarding the round seating with no back rests.

4. Noted. If funds are available I would urge that CCTV is factored in to cover the Station Approach and the entrance to the pedestrian underpass.

4.1.11a	Fire Protection Department – [Advisory comments, response from agent at 4.1.11b]

ACCESS AND FACILITIES

1. Access for fire fighting vehicles should be in accordance with The Building Regulations 2010 Approved Document B (ADB), section B5, sub-section 16.

2. Access routes for Hertfordshire Fire and Rescue Service vehicles should achieve a minimum carrying capacity of 18 tonnes.

3. Turning facilities should be provided in any dead-end route that is more than 20m long. This can be achieved by a hammer head or a turning circle designed on the basis of Table 20 in section B5.

WATER SUPPLIES

4. Water supplies should be provided in accordance with BS 9999.

5. This authority would consider the following hydrant provision adequate:

- Not more than 60m from an entry to any building on the site.

- Not more than 120m apart for residential developments or 90m apart for commercial developments.

- Preferably immediately adjacent to roadways or hard-standing facilities provided for fire service appliances.

- Not less than 6m from the building or risk so that they remain usable during a fire.

- Hydrants should be provided in accordance with BS 750 and be capable of providing an appropriate flow in accordance with National Guidance documents.

- Where no piped water is available, or there is insufficient pressure and flow in the water main, or an alternative arrangement is proposed, the alternative source of supply should be provided in accordance with ADB Vol 2, Section B5, Sub section 15.8.

6. In addition, buildings fitted with fire mains must have a suitable hydrant sited within 18m of the hard standing facility provided for the fire service pumping appliance.

The comments made by this Fire Authority do not prejudice any further requirements that may be necessary to comply with the Building Regulations.

4.1.11b	Agent response to Fire Protection Department Comments:

Access – Fire access is covered in paras 4.3.18-20 in the submitted Transport Assessment, with swept path analysis for a fire tender provided in Appendix D of this document. These arrangements are in accordance with the requirements in Building Regs Part B5 Section 16, resulting in no conflicts.

Water Supplies - Water hydrant requirements will be checked with the fire officer prior to the work commencement and if necessary any additional fire hydrant will be provided in accordance to BS9999.

4.1.12	Flood Risk Management Team – [No comments to make]

We will not going to make any comments to the Planning Application 16/2040/AOD - Street Record, Station Approach, South Oxhey, because it is not related with flood risk and drainage management.

However, we will provide our comments to the discharge of Condition 43 of the Planning application 162067DIS - Land at South Oxhey Central (Discharge of Condition 43).

4.1.13	Local Plans – No comments received.

4.1.14	Hertfordshire Ecology – No comments received.

4.1.15	HCC Waste & Minerals Team – [No comments to make]

The county council as Minerals and Waste Planning Authority does not have any comments to make in relation to the discharge of condition 18 of hybrid planning permission 16/0005/FUL.

4.1.16	Environmental Health – No comments received.

4.1.17	Environmental Protection – No comments received.

4.2	Public Consultation

4.2.1	Number consulted: 579

4.2.2	Number of responses received: 1

4.2.3	Site notices (x4): Expired 1 November 2016

4.2.4	Press notice: Expired 4 November 2016

4.2.5	The applicant provided Oxhey Library and Watford Parish Council Offices with hard copies of the application submission.

4.2.6	Summary of Responses

Concern regarding overlooking from Block Y to residential properties on Prestwick Road; Appropriate boundary screening required to ensure privacy of neighbours; Request that no units (adjacent to existing residents) are food and/or alcohol outlets; Concerns regarding noise, smell, waste and social disturbance; Such units should be on opposite side of Station Approach.

5.	Reason for Delay

5.1	Deferred for further consideration of issues.  Agreed extension of time.

6.	Relevant Planning Policy, Guidance and Legislation

6.1	The Three Rivers Local Plan

The Core Strategy was adopted on 17 October 2011 having been through a full public participation process and Examination in Public. Relevant policies include Policies PSP2, CP1, CP6, CP7, CP8, CP9, CP10, CP12 and CP13.

The Development Management Policies Local Development Document (LDD) was adopted on 26 July 2013 having been through a full public participation process and Examination in Public.  Relevant policies include DM3, DM4, DM6, DM7, DM8, DM9, DM10, DM12 and DM13 and Appendix 5.

The Community Infrastructure Levy (CIL) Charging Schedule and Regulation 123 List were adopted in February 2015.

The Site Allocations LDD was adopted in November 2014. Site R(B) is relevant.  Policies SA4 and SA6 are also applicable.

6.2	National Planning Policy Framework (NPPF)

On 27 March 2012, the framework of government guidance in the form of Planning Policy Statements and Planning Policy Guidance Notes was replaced by the National Planning Policy Framework (NPPF). The application has been considered against the policies of the Core Strategy (adopted October 2011), the Development Management Policies Local Development Document (adopted July 2013) and the Site Allocations Local Development Document (adopted November 2014) as well as the Government‘s online guidance known as the National Planning Practice Guidance (NPPG). The policies of Three Rivers District Council reflect the content of the NPPF.

6.3	Other

Three Rivers Annual Monitoring Report (2013/14).

Three Rivers Retail and Leisure Study (2012)

South Oxhey Initiative Invitation to Participate in Dialogue – Stage 2 (Deloitte, January 2015)

Hertfordshire County Council Waste Core Strategy and Development Management Policies Development Plan Document (2012) and Hertfordshire Minerals Local Plan Review 2002-2016 (2007).

National Planning Policy for Waste (2014).

Roads in Hertfordshire: Highway Design Guide (2011).

Hertfordshire County Council Local Transport Plan 3 (2011).

HCC Active Travel Strategy (2013).

HCC Rail Strategy (2011)

South West Hertfordshire Transport Plan – Review and Action Plan (2008).

Manual for Streets (Department for Transport, 2007).

Site Layout Planning for Daylight and Sunlight: A Guide To Good Practice (Second Edition, 2011) by Paul Littlefair of BRE.

The Localism Act received Royal Assent on 15 November 2011. The Growth and Infrastructure Act achieved Royal Assent on 25 April 2013.

The Planning (Listed Buildings and Conservation Areas) Act 1990, the Wildlife and Countryside Act 1981 (as amended), the Conservation of Habitats and Species Regulations 2010, the Natural Environment and Rural Communities Act 2006, the Habitat Regulations 1994 and The Flood and Water Management Act 2010 are also relevant.

7.	Planning Analysis

7.1	Background/Principle of Development

7.1.1	Planning application 16/0005/FUL was granted in May 2016 for the redevelopment of South Oxhey Central and Satellite Sites.  The planning permission was granted in hybrid form (HPP), with part in full (Satellite Sites and Phase 1A) and part in outline (the remainder).  With regard to the outline element, with the exception of access, all matters were reserved.  The parameters for the Reserved Matters (RM) applications are set out within Primary Control Documents (approved as part of application 16/0005/FUL) and supporting Secondary Control Documents.

7.1.2	The principle of the redevelopment of the site has therefore been established by the HPP (16/0005/FUL).

7.2	Uses

7.2.1	Policy CP7 of the Core Strategy (adopted October 2011) sets out that where there is an identified need for new town centre development, Town and District Centres will be the focus for this development.  South Oxhey is identified as a District / Key Centre.

7.2.2	Policy SA4 of the Site Allocations LDD (adopted November 2014) states that retail development will be acceptable in principle within the identified shopping hierarchy of centres including the District Centre at South Oxhey.

7.2.3	Policy SA6 of the Site Allocations LDD (adopted November 2014) states that the Council will promote the regeneration of South Oxhey to deliver improved access to services, improved shopping facilities, better quality leisure and community facilities and improved access to employment, education, skills and training.

7.2.4	Policy CP6 of the Core Strategy (adopted October 2011) relates to employment and economic development and supports sustainable growth.

7.2.5	The NPPF states in paragraph 23 that Local Planning Authorities should, amongst other things:

- ‘recognise town centres as the heart of their communities’,

- ‘promote competitive town centres that provide customer choice and a diverse retail offer and which reflect the individuality of town centres’, and

- ‘where town centres are in decline, local planning authorities should plan positively for their future to encourage economic activity’.

7.2.6	Phase 1B comprises ‘Town Centre Uses’ only.  The approved Development Schedule sets out the type and maximum quantity of development that can be provided across the South Oxhey redevelopment as a whole.  This permits up to 5,137 sqm of flexible town centre uses (Classes A1-A5, D1, D2, B1 and bookmakers) including a foodstore of up to 1,714 sqm.  The foodstore is proposed to be delivered within Phase 3A and would therefore form part of a subsequent RM application.

7.2.7	The proposal for Phase 1B comprises of 1,972 sqm of A1-5, D1, D2, B1 and sui generis (bookmakers) use across 19 units.  Ground floor units would range from 62 sqm to 83 sqm.  All units would remain flexible so as to not overly restrict the ability for future tenants to occupy the units, however, B1 use would be restricted to the upper floors of Block X only.  It is considered appropriate that B1 use is restricted to upper floors as this would not typically have an active frontage or attract passing trade.  A mix of A1-A5, D1 and D2 uses at ground floor will ensure an appropriate mix and would support the vitality and viability of the District Centre.

7.2.8	Concerns regarding a Public House being incorporated within the development at Station Approach are noted, however, Class A4 use which includes public houses, falls within the defined Town Centre Uses which have been previously approved under 16/0005/FUL.  Impact on residential amenity, noise, air quality etc. is discussed below in more detail, however, it should be noted that planning conditions including restrictions on the hours of operation and requiring details of any external extraction/ventilation equipment to be submitted for approval, are suggested in the interests of safeguarding neighbouring amenity.

7.2.9	The proposal is in accordance with the Development Schedule in terms of the quantum, mix and number of units.  It is considered that the proposed development would enhance the vitality and viability of this District Centre and is therefore acceptable in this regard.

7.3	Layout, Scale and Design

7.3.1	Policy CP1 of the Core Strategy (adopted October 2011) seeks to ensure that all development will contribute to the sustainability of the District.  This includes taking into account the need to (n) ‘promote buildings and public spaces of a high enduring design quality that respects local distinctiveness, is accessible to all and reduces opportunities for crime and anti-social behaviour’.

7.3.2	Policy CP12 of the Core Strategy (adopted October 2011) seeks to ensure that all development has a high standard of design. For example, development proposals should:

- have regard to the local context and conserve or enhance the character, amenities and quality of an area

- make efficient use of land whilst respecting the distinctiveness of the surrounding area in terms of density, character, layout and spacing, amenity, scale, height, massing and use of materials

- ensure buildings and spaces are, wherever possible, orientated to gain benefit from sunlight and passive solar energy

- design out opportunities for crime and anti-social behaviour

- incorporate visually attractive frontages to adjoining streets and public spaces

- ensure all appropriate frontages contain windows and doors that assist informal surveillance of the public realm

- use high standards of building materials and finishes

- make a clear distinction between public and private spaces and enhance the public realm

- provide convenient, safe and visually attractive areas for the parking of vehicles and cycles without dominating the development or its surroundings.

7.3.3	The NPPF states in paragraphs 56 and 57 that:

‘The Government attaches great importance to the design of the built environment. Good design is a key aspect of sustainable development, is indivisible from good planning, and should contribute positively to making places better for people.

It is important to plan positively for the achievement of high quality and inclusive design for all development, including individual buildings, public and private spaces and wider area development schemes.’

7.3.4	The forecourt loop road has been shortened to facilitate the creation of a public space (Station Square) to the front of the Station.  Drop-off, taxi and loading bays are retained (access and servicing is discussed below), however, a reduction in their number enables a larger area of Station Approach to become traffic free (with the exception of limited access for maintenance).  Units for Town Centre uses front the northern and southern sides of the square, with the existing car park retained.  It is considered that the layout of Phase 1B is in accordance with the outline components of the HPP.

7.3.5	Parameter Plans (Development Zones and Building Heights) approved with the hybrid planning permission define the maximum envelope within which a building may be built.  The submitted details accord with the requirements of the Parameter Plans in this regard.

7.3.6	Blocks W, Y and Z would be single storey with flat roofs.  The roofs would be stepped down in height towards the Station to reflect the sloping nature of the site.  Block Z would be set forward of adjacent properties in Prestwick Road and it is also noted that the immediate neighbour No. 49 Prestwick Road is at a slightly lower level.  Whilst there would be views of Block Z when approaching the site from the north, given its single storey nature it is not considered that it would appear excessively prominent.  Feature brickwork is proposed and an active frontage is proposed to both the side and front elevation of the corner unit to add interest.

7.3.7	Block X would be part single storey and part 3-storey, with the 3-storey element adjacent to Prestwick Road.  The building line (Prestwick Road frontage) would be comparable with the existing building line to the south of the site.  Whilst higher than other buildings on this side of Prestwick Road, the variation in height provides interest and will assist in integrating Phase 1B with the wider South Oxhey Central redevelopment to the opposite side of Prestwick Road which will be up to 7 storeys in height.  As with the corner unit on Block Z, feature brickwork is proposed and an active frontage to both Station Approach and Prestwick Road to break up the built form and add interest.

7.3.8	Due to the fall in land levels the Station is largely hidden from view from Prestwick Road with generous spacing between the northern and southern units fronting Station Approach.  The inclusion of Unit 19 will add interest in this central area, creating a feature building within the pedestrianized part of Station Square.

7.3.9	Brick is proposed as the predominant material and as previously noted, feature brickwork is proposed to the corner locations to articulate the built form and provide interest.  Shop fronts are proposed to be largely glazed, both to increase light within the units and create an inviting frontage.  A zone for signage is also proposed to each unit to ensure consistency within Station Approach and enhance the visual appearance of the development.

7.3.10	Station Square would provide a usable area of public open space and would result in uplift in the quality of the public realm and public open space.

7.3.11	It is considered that the scale has been carefully considered to optimise and make best use of this brownfield site in accordance with the National Planning Policy Framework, whilst respecting the surrounding context.  The scale of the majority of the development would be single storey and therefore comparable with that existing, with the higher 3 storey building providing variation and linking the development to the wider redevelopment site to the west of Prestwick Road.  There is no objection to the layout, scale and design which it is considered would be appropriate within this context and would support the regeneration of this District Centre.

7.4	Residential Amenity

7.4.1	Policy CP12 of the Core Strategy states that in seeking a high standard of design development proposals should protect residential amenities by taking into account the need for adequate levels and disposition of privacy, prospect, amenity and garden space.

7.4.2	The proposed development would be single storey closest to the Station and would be separated from the settlement of Carpenders Park by the railway line and associated buildings.  Whilst land to the immediate east of the station is at a lower level, given the separation distance it is not considered that there would be an adverse effect.

7.4.3	Block X (part single, part 3 storey) would be sited adjacent to the boundary with the petrol station forecourt and would therefore be separated from dwellings to the south by over 40 metres.

7.4.4	The 3 storey element of Block X would be sited closest to Prestwick Road with a distance of approximately 35 metres between the proposed building and existing 3/4 storey buildings opposite.  These buildings are proposed to be replaced as part of a later phase of the South Oxhey redevelopment.  The proposed separation distance is considered sufficient to ensure the amenities of existing and future occupiers are protected.

7.4.5	Block Y would be located opposite existing 2-storey dwellings at the junction of Prestwick Road and Fairfield Avenue.  It would be single storey and set over 25 metres from these existing neighbours at the closest point.  The single storey nature of the building and intervening highway are sufficient to ensure that the amenities of existing and occupiers are protected.

7.4.6	The closest residential dwelling to the proposed development is No. 49 Prestwick Road to the immediate north of Block Y.  This 2-storey end of terrace dwelling is set off the boundary and at a lower land level, with a detached garage set back from the dwelling adjacent to the boundary.  The proposed single storey Block would be set off the boundary with No. 49 Prestwick Road by approximately 3 metres but would be set forward by approximately 7.6 metres.

7.4.7	In order to fully consider the impact of the proposed development on the residential amenities of occupiers of No. 49 Prestwick Road, a Daylight and Sunlight Assessment has been prepared.  This report expands on the overarching Daylight and Sunlight Assessment submitted in support of the original HPP which found no adverse daylight or sunlight issues for the proposed development within Phase 1B.

7.4.8	The assessment uses methodology from ‘Site Layout Planning for Daylight and Sunlight: A Guide To Good Practice’ (Second Edition, 2011) by Paul Littlefair of the BRE (Building Research Establishment), which is a guide commonly used by architects. The numeric methods used to assess impacts on the surrounding buildings are as follows:

Vertical Sky Component (VSC): This is the ratio of diffuse light (the light available on a cloudy day) on a vertical surface to the ratio of light on an unobstructed horizontal surface (an unobstructed vertical surface will receive a VSC of 40%). As a guide, a target of >27% (or 80% of previous value) is used. Section 2.2.7 of the BRE Guide explains that:

‘If this VSC is greater than 27% then enough skylight should still be reaching the window of the existing building. Any reduction below this level should be kept to a minimum. If the VSC, with the new development in place is both less than 27% and less than 0.8 time its former value, occupants of the existing building will notice the reduction in amount of skylight. The area lit by the window is likely to appear more gloomy, and electric lighting will be needed more of the time.’

Annual Probable Sunlight Hours (APSH): This is the fraction of sunlit hours received by a surface. As a guide, for rooms within 90° due south the following targets are used:

- 25% (or 80% of previous value)

- (winter) > 5% (or 80% of previous value)

- reduced by less than 4%

7.4.9	The proposed Block would be set forward relative to the existing building on the site (former Barclays Bank), however, a greater separation to the northern boundary is proposed and the proposed Block would be single storey only (the existing building extending to 2-storeys in part).  The report submitted in support of the current RM application shows that based on the current detailed proposals, the VSC, APSH and APSH winter criteria will be achieved, therefore demonstrating that all windows (3 front windows to No. 49 Prestwick Road) will achieve acceptable levels of sunlight and daylight.  One of the three windows assessed (lower left) will experience a reduction in the APSH of 5%, which is above the recommended target of a change of no more than 4%.  This means that, although the residents will be receiving an acceptable level of sunlight through this window, they may notice the difference in the level of sunlight from that experienced before the development.  On balance, given that the change in APSH is exceeded by only 1%, that it is only affecting one of the three windows assessed and that the absolute targets for minimum APSH are still achieved, the overall impact on No. 49 Prestwick Road from the proposed development is considered to be acceptable.

7.4.10	As noted above, the existing building (former Barclays Bank) is part single, part 2-storey and extends close to the boundary with No. 49.  Whilst Block Y would project beyond the rear of No. 49 Prestwick Road, the projection would be comparable with that existing and it is noted that there would be increased spacing and the proposed Block would also be of reduced height relative to that existing.  As such it is not considered to result in demonstrable harm to the residential amenities of occupiers of No. 49 Prestwick Road.

7.4.11	The space to the rear (north) of Block Y would provide a servicing area for the Units.  Comings and goings would be limited to those associated with the 5 Units forming Block Y with access restricted such that it is not considered that the level of activity or comings and goings would result in demonstrable harm to neighbouring amenity.  The submitted elevations indicate that air-conditioning units/plant would be located at a low level so that it is largely screened from view from Prestwick Road and from the neighbouring property.

7.4.12	As previously noted, the neighbouring dwellings on Prestwick Road are located at a lower land level and comments received regarding the need to ensure an appropriate level of screening between the development and neighbouring residential dwellings are noted and considered valid.  A condition requiring the submission of boundary treatment details for approval is therefore suggested.

7.4.13	The impact of the proposed uses on neighbouring amenity e.g. in terms of noise, smell etc. are discussed in the appropriate sections below.

7.5	Highways, Access & Servicing

7.5.1	The Core Strategy recognises that Key Centres generally have good access to public transport.  Policy PSP2 of the Core Strategy (adopted October 2011) states that development in Key Centres will contribute to an integrated approach to improve transport, including public transport, and movement into and around the Key Centres and connectivity with all other centres in the District and adjacent counties.

7.5.2	Policy CP10 of the Core Strategy (adopted October 2011) relates to transport and travel and states that major development should be located in highly accessible areas, have a safe and adequate means of access and make provision for all users including people with mobility difficulties.  It states that all development should be designed and located to minimise the impacts of travel by motor vehicle on the District; appropriate in scale to existing transport infrastructure including public transport; and be integrated with the wider network of transport routes.

7.5.3	Policy DM10 of the Development Management Policies LDD (adopted July 2013) requires that adequate provision for the storage and recycling of waste should be fully integrated into design proposals.

7.5.4	South Oxhey Central is well served by public transport. It is located adjacent to Carpenders Park Station which forms part of the London Overground system, with peak time service frequencies of 3 trains per hour to Watford Junction / London Euston.  It is also relatively well served by buses, with bus stops located at Station Approach and along Fairfield Avenue and Prestwick Road.

7.5.5	The Station Approach area currently accommodates a taxi stand for approximately 6 taxis, a bus stop stand for the W19 bus service, a drop off area for 2 -3 vehicles, approximately 20 short stay car parking spaces and 2 disabled spaces, servicing for the existing retail units and access to the Station Approach (longer term) car park.

7.5.6	The proposed layout as amended will provide a taxi stand for 4 taxis; 3 short stay parking spaces to provide drop-off/pick-up; 2 disabled parking spaces; retention of existing bus stop on Prestwick Road (Stop B); retention of existing bus stop/stand on southern side of loop road (Stop A); dual use loading/drop-off and pick-up bays; and access to the adjacent car park which provides 34 parking spaces.

7.5.7	The proposals seek to promote the use of sustainable modes of transport and create a larger traffic free area at Station Approach, with enhancements to the public realm space around the Station.  (Cycle provision is discussed at 7.6 below).

7.5.8	The Licensing Officer raised some initial concerns regarding the loss of taxi rank spaces and welcomes the amended layout which provides an additional space.  Whilst there would be a reduction from 6 to 4 spaces in the taxi rank, the proposed rank is considered of sufficient size to serve the Station/area and no objection is raised.

7.5.9	Access for servicing/deliveries/refuse storage is included to the rear of each Block.    A condition on the HPP requires a Delivery and Servicing Plan to be submitted for each Phase.  This should include:

- details of the delivery and servicing requirements (including refuse collection) for the proposed uses;

- a scheme for coordinating deliveries and servicing for the proposed development;

- areas within the development site that will be used for loading and manoeuvring of delivery and servicing vehicles;

- access to / from the site for delivery and servicing vehicles and details of routing;

- restrictions on the timing of deliveries so that they principally occur outside peak periods of activity;

- details of marshalling and a vehicle booking system for the foodstore such that reversing from Oxhey Drive into the site is safely managed; (not applicable for Phase 1B)

- restrictions on the timing of servicing for the units in Prestwick Road / Market Square to ensure they are outside opening hours;

- the requirement for service vehicles to enter and exit the highway in forward gear, with manoeuvring limited to the parking forecourt areas and activity coordinated through a vehicle booking system;

- servicing of the units in Station Square only taking place in designated zones, with activity coordinated through a vehicle booking system.

7.5.10	A Delivery and Servicing Plan has been submitted pursuant to this condition and is pending consideration.  In addition, a further condition on the RM application will require details of the management of the Station Approach parking to include details of the management of the dual use bays and details of their signage.

7.5.11	Refuse and recycling storage is proposed to the rear of Blocks X, Y and Z and to the flank of Block W.  A separate store is also proposed for Unit 19.  The applicant has discussed the provision and required capacity with TRDC.  Each unit will be provided with space to accommodate one large bin (1100L) and a recycling bin (240L) which meets TRDC requirements for the proposed development.  The applicant has confirmed that; “The bin stores have been sensitively designed within the scheme in order to ensure ease of access for occupiers/collection, whilst sited in a secure area protecting the amenity of the public realm and avoiding conflict with its users”.  Exact details would be secured via condition.

7.6	Parking

7.6.1	Policy CP10 of the Core Strategy (adopted October 2011) states that development proposals must make provision for all users, including car and other vehicle parking, with priority given to people with mobility difficulties, pedestrians, cyclists and equestrians.

7.6.2	With regards to commercial parking, the HPP set out that a total of 140 car parking spaces are proposed (across South Oxhey Central) as follows:

| Retail parking location | No. of spaces |
| --- | --- |
| Foodstore car park, Oxhey Drive | 62 |
| Prestwick Road, to front of Blocks O, P & Q | 44 |
| Station Approach car park | 34 |
| Total: | 140 |

7.6.3	It should be noted that the above parking numbers are in addition to the parking proposed to serve the residential elements of the scheme.

7.6.4	The location of the additional commercial parking and its proximity to Station Approach (Phase 1B) is shown on plan 1275-O-162 C submitted pursuant to the HPP.  The wider redevelopment scheme will provide a total of 140 car parking spaces, in addition to the short stay, disabled spaces and dual use loading/drop-off and pick-up bays proposed as part of the current RM application at Station Approach.  These 140 spaces were considered acceptable to serve the development as approved under the HPP.

7.6.5	In terms of the 5,137sqm of commercial floorspace this would represent a ratio of 1 car parking space per 37sqm.  The Transport Assessment included a survey of existing parking ratios and demand and the proposal was found to result in a relative increase in supply when compared to the ratio of existing spaces to floor area.  A Management Plan (required by condition on the HPP) would also place appropriate restrictions on parking in retail designated spaces during the day to prevent misuse and commuter parking.

7.6.6	The 34 parking spaces within Station Approach car park will be re-provided following the completion of works.  There would be no parking available at the Station during the period of works and it is noted that this would result in loss of income for the Council during this period.  The car park will continue to be managed by Three Rivers and consideration is being given to the appropriate time limiting measures.  At the committee in December Members raised concerns regarding possible conflict on the forecourt loop road, and as such the developer will be required to provide signage (details to be secured via condition) to indicate the availability of spaces.  Such signage should be suitably located so that users do not enter the forecourt loop road unnecessarily.

7.6.7	As noted above, the proposed layout will also provide a taxi stand for 4 taxis; 3 short stay parking spaces; 2 disabled parking spaces; and dual use bays for loading/drop-off and pick-up with capacity for either 7 cars, 3/4 delivery vehicles or a combination of both.  These would be in addition to the 140 spaces specified in the HPP.

7.6.8	In summary, whilst there would be a reduction in parking provision within the Station forecourt as set out at 7.5.5 above, the proposals for Phase 1b should be viewed in the context of the wider South Oxhey re-development and the overall parking provision for the commercial elements of the South Oxhey re-development exceeds adopted standards.  Subject to conditions to secure appropriate signage and management, the provision at Station Approach is considered to strike an appropriate balance to meet the needs of both those accessing the Station and adjacent shops whilst ensuring that the vision to improve the public realm across South Oxhey Central, particularly at Station Approach is supported.

7.6.9	There are currently 5 stands providing 10 cycle spaces at Station Approach.  The standards set out in Appendix 5 of the Development Management Policies LDD (adopted July 2013) require 24 spaces (11 short term and 13 long term).  The proposal would provide a total of 32 cycle spaces, 12 short term (either side of the access to the car park) and 20 long term covered spaces adjacent to Unit 19.  The provision would exceed standards and significantly exceed the existing provision.  It is also noted that 8 covered spaces are provided adjacent to the station (outside of the application site) and it is understood that these would remain in situ.

7.7	Public Open Space

7.7.1	Policy CP9 of the Core Strategy (adopted October 2011) states that the Council will seek a net gain in the quality and quantity of green infrastructure, through the protection and enhancement of assets and provision of new green spaces.

7.7.2	Station Square would provide a usable area of public open space and it is considered that the proposal would result in a significant uplift in the quality of the public realm and public open space.

7.8	Trees and Landscaping

7.8.1	Policy DM6 of the Development Management Policies LDD states that development proposals should:

- include landscaping proposals with new trees and other planting to enhance the landscape of the site and its surroundings

- be designed in such a way as to allow trees and hedgerows to grow to maturity without causing undue problems of visibility, shading or damage

- include suitable replacement planting where the felling of a tree or removal of a hedgerow is permitted.

7.8.2	Policy DM7 refers to landscape character, stating that the Council will require all development proposals to make a positive contribution to the surrounding landscape.

7.8.3	Policy CP12 of the Core Strategy identifies that developments should be adequately landscaped and designed to retain, enhance or improve important natural features. It requires landscaping proposals to reflect the surrounding landscape of the area.

7.8.4	The site is currently dominated by vehicles and hard surfaced areas with limited planting or soft landscaping.  There are some existing trees, however, these Category B and C trees do not make a significant contribution to the visual amenity of the area and their loss (subject to appropriate replacement planting) was agreed at the time of the HPP.

7.8.5	The proposal would include the provision of 19 semi-mature trees (increase of 10) across the site and the proposed public realm works would also include raised angular grass mounds to enhance the development.

    - Details including species and size, specification, locating, timing of planting and future management have been submitted for approval by the LPA in consultation with the Landscape Officer (application 16/2067/DIS is pending consideration).

7.9	Ecology

7.9.1	Section 40 of the Natural Environment and Rural Communities Act 2006 requires Local Planning Authorities to have regard to the purpose of conserving biodiversity. This is further emphasised by regulation 3(4) of the Habitat Regulations 1994 which state that Councils must have regard to the strict protection for certain species required by the EC Habitats Directive.

7.9.2	The protection of biodiversity and protected species is a material planning consideration in the assessment of applications in accordance with Policy CP9 of the Core Strategy (adopted October 2011) and Policy DM6 of the Development Management Policies LDD (adopted July 2013).  National Planning Policy requires Local Authorities to ensure that a protected species survey is undertaken for applications where protected species may be affected, prior to determination of a planning application.

7.9.3	The HPP was accompanied by a Biodiversity Checklist and Phase 1 Habitat Survey and Bat Survey which made up a Preliminary Ecology Appraisal (PEA) consisting of a desk study and a field study.  No evidence was found of any protected species.

7.9.4	The biodiversity strategy set out in the PEA identified reasonable efforts to embed biodiversity into the overall design, including the incorporation of brown roofs and bird and bat boxes.  It was considered that these measures would help enable the proposed development to make a contribution towards achieving biodiversity gain as encouraged by the NPPF.

7.9.5	A condition on the HPP therefore requires that the development is carried out in accordance with the recommendations in the PEA with a five year ecological management plan to be submitted prior to the commencement of development in each phase.

7.10	Sustainability

7.10.1	Policy CP1 of the Core Strategy (adopted October 2011) requires applications to demonstrate the extent to which sustainability principles have been incorporated into the location, design, construction and future use of proposals and the expected carbon emissions.

7.10.2	Policy DM4 of the Development Management Policies LDD (adopted July 2013) requires that development must produce 5% less carbon dioxide emissions than Building Regulations Part L (2013) requirements having regard to feasibility and viability.

7.10.3	Policy CP12 of the Core Strategy (adopted October 2011 relates to the design of development and states that development proposals should use innovative design to reduce energy and waste and optimise the potential of the site and should ensure that buildings and spaces are, wherever possible, orientated to gain benefit from sunlight and passive solar energy.

7.10.4	An Energy and Sustainability Statement was submitted in support of the HPP and a further assessment has been undertaken at this RM stage to confirm that the proposals for Phase 1B in regards to energy and sustainability are in compliance with the HPP.

7.10.5	The submitted Energy and Sustainability Statement demonstrates that the development will achieve a 10% saving and would therefore meet the target of a 5% improvement in CO2 emissions compared to Building Regulations Part L (2013), in line with Policy DM4 of the Three Rivers District Council Development Management Policies LDD (adopted July 2013).

7.11	Waste and Minerals

7.11.1	Policy DM10 of the Development Management Policies LDD (adopted July 2013) states that new development should reduce the amount of waste generated, including reusing materials wherever possible as part of construction.  Regard should also be had to the adopted Hertfordshire County Council Waste Core Strategy and Development Management Policies Development Plan Document 2012 as well as the National Planning Policy for Waste (2014).

7.11.2	An Outline Site Waste Management Plan (SWMP) was submitted with the HPP and provided an estimate of the amount of site waste that would be produced.  A more detailed SWMP is required by condition on the HPP.

7.12	Flooding and Drainage

7.12.1	Policy CP1 of the Core Strategy (adopted October 2011) states that there is a need to avoid development in areas at risk from flooding and to minimise flood risk through the use of Sustainable Drainage Systems (SuDS). Since April 2015, SuDS are a compulsory requirement for all major development.  Policy CP12 of the Core Strategy (adopted October 2011) states that resilience should be built into a site’s design taking into account climate change.

7.12.2	Policy DM8 of the Development Management Policies LDD (adopted July 2013) states that development will only be permitted where it would not be subject to unacceptable risk of flooding, and would not unacceptably exacerbate risk of flooding elsewhere. It states that there must be sufficient surface water drainage

7.12.3	The site lies within Flood Zone 1, which is the zone with least risk (1 in 1000 year probability of fluvial flooding).  However, as the wider South Oxhey Central site has an area greater than 1 ha a Flood Risk Assessment (FRA) was submitted with the HPP.  A Drainage Strategy was also submitted.  These concluded that the site has a low overall risk of flooding and that surface water drainage is currently managed through the Thames Water drainage network, with separate foul and surface water systems.  It was stated that the new development would be piped into the existing Thames Water system, with SuDS attenuation to accommodate the remaining storage of a 1:100 year rainfall event.

7.12.4	An updated Drainage Strategy Statement has been submitted which confirms that the piped drainage system will be designed so that it does not flood for a 1:30 year event.  The SuDS attenuation systems for the site shall be sized to accommodate the remaining storage of the 1:100 (+30% climate change) rainfall event.  The SuDS measures proposed include brown roofs, permeable paving systems, cellular storage systems and flow restriction devices.

7.12.5	Details of how SuDS measures shall be managed and maintained for the lifetime of the development have been submitted pursuant to condition 43 (Flood mitigation and drainage) of the HPP and are being considered under discharge of conditions application 16/2067/DIS.

7.13	Contamination

7.13.1	Policy CP1 of the Core Strategy (adopted October 2011) states that there is a need to manage and reduce risk of and from pollution in relation to quality of land, air and water and dealing with land contamination. Policy DM8 of the Development Management Policies LDD (adopted July 2013) states that development must protect the quantity and quality of surface and groundwater resources from aquatic pollution. Policy DM9 refers to contamination and pollution control.

7.13.2	The HPP was accompanied by a Land Contamination Assessment and a Preliminary Ground Water Risk Assessment.  The site is located within an area of a Minor Aquifer but no groundwater was encountered during the soils investigation exploratory works. The Environment Agency advised that the reports did not suggest any signs of major contamination.

7.13.2	It was recommended that further site investigation work is carried out at South Oxhey Central to ascertain the deeper ground conditions below the site to allow piling design to take place.  Following demolition of the existing structures, a post demolition contamination survey should also be undertaken, comprising a series of excavated trial pits along with further contamination testing and analysis. This is controlled by condition on the HPP.

7.14	Air Quality

7.14.1	Policy DM9 of the Development Management Policies LDD (adopted July 2013) refers to contamination and pollution control. It states that development will not be permitted where it would have an adverse impact on air pollution levels or would be subject to unacceptable levels of air pollutants or disturbance from existing pollutant sources.

7.14.2	An Air Quality Screening Assessment was undertaken and submitted with the HPP.  The Environmental Health Officer raised no objections subject to a condition requiring mitigation measures to control dust emissions during construction.  A condition on the HPP requires the submission of an Environmental Health and Safety Plan and a Dust Management Plan prior the commencement of any site works.

7.14.3	Details of extraction and ventilation for individual units (including any specific requirements associated with food establishments) are not provided at this stage, however, in the interests of safeguarding neighbouring amenity it would be appropriate to condition any grant of consent to require the submission of further details if such equipment is required.

7.15	Noise Pollution

7.15.1	Policy DM9 of the Development Management Policies LDD (adopted July 2013) states that development will not be permitted where it would have an adverse impact on the acoustic environment of existing or planned development, would have an unacceptable impact on countryside areas of tranquillity, or would be subject to unacceptable noise levels or disturbance from existing noise sources whether irregular or not. Reference is made to Appendix 4 of the Development Management Policies LDD (adopted July 2013) which sets out noise exposure categories for residential development.

7.15.2	The HPP was accompanied by a Noise Impact Assessment and no objection was raised to the findings/recommendations.  Now that further details of the design of Phase 1B are known, this RM application has been accompanied by a Mechanical and Electrical Design Report.  This sets out that from the air handling units (AHUs), supply and extract ductwork will be extended and run within the ceiling void to serve the commercial units.  The fresh air intake and exhaust will be taken/ extracted from/to the

atmosphere via suitably sized louvers and silencers to ensure the noise generated from the plant has been mitigated to the minimum and the ventilation system will meet the recommended maximum noise ratings as defined in the HPP.

7.15.3	As noted above, details of any extraction of ventilation for individual units would be required to be secured by condition.

7.15.4	The proposed hours of operation for the A1- A5 units are 6am to midnight Monday to Sundays (including National Holidays).  This is to retain flexibility for future operators to ensure their viability.  These are not considered uncharacteristic or inappropriate in a mixed use District Centre, however, the proximity of Block Y to existing residential units is noted and it is considered that 7am to 11pm would be appropriate for this Block.  These hours are considered to strike an appropriate balance in terms of enhancing the vitality and viability of the District Centre whilst ensuring that the residential amenities of neighbouring occupiers are safeguarded.

7.16	Lighting

7.16.1	Policy CP12 of the Core Strategy (adopted October 2011) relates to the design of development and states that development should make a clear distinction between public and private spaces and enhance the public realm. Policy DM9 of the Development Management Policies LDD (adopted July 2013) states that lighting proposals should ensure that:

- Proposed lighting schemes are the minimum required for public safety and security

- There is no unacceptable adverse impact on neighbouring or nearby properties

- There is no unacceptable adverse impact on the surrounding countryside

- There is no dazzling or distraction to road users including cyclists, equestrians and pedestrians

- Road and footway lighting meets the Country Council’s adopted standard

- There is no unacceptable adverse impact on wildlife

- Proposals in the vicinity of habitats and habitat features important for wildlife ensure that the lighting scheme is sensitively designed.

7.16.2	Design Guidelines were approved as part of the HPP and form part of the Primary Control Documents governing the overarching design of the wider South Oxhey redevelopment.  Design Guideline 9 includes reference to lighting and states;

Lighting provision is to be coordinated with Three Rivers District Council and Secure By Design to ensure sufficient light levels are provided to all external areas of site…

7.16.3	Further details of the overarching approach to lighting design are referred to in the Design and Access Statement which accompanied the HPP.  It stated that;

‘The lighting of the central area will provide a safe and coherent landscape in the evenings and at night. The lighting will provide a direction through the development and a visually attractive space for users.

Lighting will be sympathetic to the surrounding environment and neighbours. The locations and style will be focussed to accentuate the strong geometric grid, combined with coherent wayfinding elements. A detailed lighting strategy including all lighting locations and specification is subject to lux levels analysis, and Secure by Design requirements’.

7.16.4	In consideration of the application at the November Planning Committee meeting Members noted the importance of ensuring that an appropriate lighting scheme is agreed, including the need to ensure that this provides lighting for those accessing/exiting the Station in line with first and last train times.

7.16.5	Details of lighting are controlled by condition on the HPP and are required to be submitted for approval.  The comments raised by Members are noted and will be taken into consideration by officers when determining any future Discharge of Condition application for lighting.

7.16.6	The shop fronts include zones for signage to ensure consistency, however, no further details have been provided at this stage as it is anticipated that this level of detail would not be known until future occupies had been established.  Advertisement Consent would likely be required for signage and an advisory informative is included.

7.17	Historic Environment

7.17.1	Policy DM3 of the Development Management Policies LDD (adopted July 2013) states that development outside but near to a Conservation Area should not adversely affect the setting, character, appearance or views into or out of the Conservation Area.  It also states that development should not adversely affect the character of Listed Buildings.  Where an application site has the potential to include heritage assets with archaeological interest, development proposals must be accompanied by a desk-based assessment and potentially a field evaluation.

7.17.2	Policy CP12 of the Core Strategy (adopted October 2011) relates to design of development and states that, in seeking a high standard of design, the Council will expect all development proposals to conserve and enhance natural and heritage assets.

7.17.3	The site is not located within a Conservation Area.  Oxhey Hall Conservation Area is located approximately 700 metres to the north west with Oxhey Hall, a Scheduled Monument and a Grade II* Listed Building beyond this.  Watford Heath Conservation Area is also located some 1.2km to the north of the site.  The nearest Listed Building is Oxhey Chapel (All Saints Church, Grade II*) located on Gosforth Lane approximately 350 metres north west of the site. An archaeological area lies to the west of the site.

7.17.4	It is not considered that the proposal would result in any significant impact on the character and appearance of the Conservation Areas or the setting of the Listed Buildings in the vicinity.

7.17.5	The Historic Environment Desk-Based Assessment submitted with the HPP identified that there were no previously recorded heritage assets at the site.

7.18	Impact to Railway

7.18.1	Due to the proximity of the railway, London Underground and Network Rail were consulted as part of the application. London Underground Infrastructure Protection have confirmed that they have no comment to make.

7.18.2	Network Rail have not commented in relation to the RM application, however, at the time of the HPP they stated that safe access to the railway needs to be maintained at all times and that there must be no physical encroachment onto Network Rail land.

7.19	CIL

7.19.1	The Three Rivers Community Infrastructure Levy (CIL) came into force on 1 April 2015. This is a charge on new developments for use towards infrastructure projects within the District. As set out in the adopted CIL Charging Schedule (February 2015), it has been judged on the basis of viability that there is a £nil charge for development in the area within which the application site is located (Zone C).

7.19.2	Furthermore the adopted CIL Regulation 123 List (February 2015) notes that the ‘South Oxhey Regeneration Scheme’ is not intended as a target for the use of CIL in relation to education, publicly accessible leisure and open space, healthcare facilities, other social and community facilities, and emergency services, on the basis that these will be ‘secured through s.106 or other alternative measures’. Provision of, and contributions towards, such facilities and services for the South Oxhey regeneration scheme must therefore be secured via a Section 106 Agreement or conditions as appropriate.

7.20	Planning Obligations

7.20.1	Policy CP8 of the Core Strategy (adopted October 2011) states that development should provide, or make adequate contribution towards, infrastructure and services to make a positive contribution to safeguarding or creating sustainable and linked communities, to offset the loss of any infrastructure through compensatory provision and to meet ongoing maintenance costs where appropriate.

7.20.2	The contributions in the table below were secured as part of the S106 Agreement completed pursuant to the HPP.  It was considered that these contributions were necessary to mitigate the direct impacts of the development.

| Obligation | Amount |
| --- | --- |
| Nursery and childcare services | £69,067.02 |
| Primary education | £481,708.53 |
| Secondary education | £150,758.87 |
| Youth facilities | £4,055.13 |
| Library provision | £41,247.72 |
| NHS England | £218,691.53 |
| Total: | £965,528.80 |

7.21	Crime

7.21.1	Policy CP12 of the Core Strategy (adopted October 2011) seeks to ensure that all development has a high standard of design and should design out opportunities for crime and anti-social behaviour.

7.21.2	The Crime Prevention and Design Advisor raised no objections having reviewed the submitted details and provided advisory comments for consideration, including in relation to the design of seating and inclusion of CCTV.  The applicant has responded to these comments (4.1.10b) and the Crime Prevention and Design Advisor is satisfied with the responses provided regarding provision of kerb (rear of Block W), height of central unit and seating (as set out at 4.1.10c).  However, they do request that consideration is given to the provision of CCTV to cover Station Approach, particularly the access to the pedestrian underpass.  An advisory informative is suggested to highlight this request to the applicant.

7.22	Conclusion

7.22.1	Hybrid planning permission 16/0005/FUL granted consent for the phased comprehensive redevelopment of the land at South Oxhey (South Oxhey Central, Maylands Road, Hayling Road and Hallowes Crescent) to include the demolition of existing buildings and provision of residential led mixed use development with associated site preparation/enabling works, transport infrastructure works, landscaping works and provision of car parking.  The proposals for Phase 1B (Station Approach) are in accordance with the parameters previously agreed and will assist in the regeneration and uplift of the South Oxhey area in accordance with the requirements of Policies PSP2 and CP1 of the Core Strategy (adopted October 2011) and Policy SA6 of the Site Allocations Local Development Document (adopted November 2014) and is therefore recommended for approval.

8.	Recommendation

8.1	That PLANNING PERMISSION BE GRANTED subject to the following conditions:

C1	Plans

The development shall be carried out in accordance with the following approved plans:

1281-D-1100 O

1281-D-1500 E

1281-D-1804 D

1281-D-1501 E

1281-D-1803 C

1281-D-1700 A

1281-D-1800 F

1281-D-1805 E

1281-D-1102 D

1281-D-1000

1281-D-1802 J

1281-D-1801 F

437-SQ-03

11363/5002 P3 (Drainage Strategy)

1281-D-1101 E

437-SQ-02 E

Reason: For the avoidance of doubt, in the proper interests of planning and in accordance with Policies PSP2, CP1, CP6, CP7, CP8, CP9, CP10, CP12 and CP13 of the Core Strategy (adopted October 2011), Policies DM3, DM4, DM6, DM7, DM8, DM9, DM10, DM12 and DM13 and Appendix 5 of the Development Management Policies LDD (adopted July 2013) and Policies SA4 and SA5 of the Site Allocations Local Development Document (adopted November 2014).

C2	Materials

Before any building operations above ground level hereby permitted are commenced, samples and details of the proposed external materials shall be submitted to and approved in writing by the Local Planning Authority and no external materials shall be used other than those approved.

Reason: To prevent the development being constructed in inappropriate materials in accordance with Policies CP1 and CP12 of the Core Strategy (adopted October 2011).

C3	Use Classes

The ground floor of Blocks W, X, Y and Z and Unit 19 as shown on approved site plan 1281-D-1100 shall not be used other than for uses falling within Classes A1, A2, A3, A4, A5, D1, D2 and Bookmakers (Sui Generis) of the Town and Country Planning (Use Classes) Order 1987 (as amended).

The first and second floors of Block X shall not be used other than for uses falling within Classes A1, A2, A3, A4, A5, B1, D1, D2 and Bookmakers (Sui Generis) of the Town and Country Planning (Use Classes) Order 1987 (as amended).

Reason: To ensure the vitality of the District Centre and an appropriate mix of uses in accordance with Policies PSP2, CP1, CP6 and CP7 of the Core Strategy (adopted October 2011) and Policy SA4 of the Site Allocations Local Development Document (adopted November 2014).

C4	Hours of Operation 1

The Class A1/A2/A3/A4/A5/D1/D2/Sui Generis uses hereby permitted within Blocks W, X and Z and Unit 19 shall not operate other than between the hours of 06:00 to Midnight (inclusive) Mondays to Sundays and including National Holidays.

Reason: To safeguard the residential amenities of the occupiers of neighbouring properties in accordance with Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM9 of the Development Management Policies LDD (adopted July 2013).

C5	Hours of Operation 2 – Block Y

The Class A1/A2/A3/A4/A5/D1/D2/Sui Generis uses hereby permitted within Block Y shall not operate other than between the hours of 07:00 to 23:00 (inclusive) Mondays to Sundays and including National Holidays.

Reason: To safeguard the residential amenities of the occupiers of neighbouring properties in accordance with Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM9 of the Development Management Policies LDD (adopted July 2013).

C6	Ventilation & Extraction Equipment

No ventilation or extraction equipment shall be installed unless details of its method of construction, odour control measures, noise levels, its appearance and finish have first been submitted to and been approved in writing by the Local Planning Authority.  The equipment shall be implemented and maintained as approved.

Reason: To safeguard the residential amenities of the occupiers of neighbouring properties in accordance with Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM9 of the Development Management Policies LDD (adopted July 2013).

C7	Boundary Details

Prior to occupation of any part of the development hereby permitted, a plan indicating the positions, design, materials and type of boundary treatment to be erected on the site shall be submitted to and approved in writing by the Local Planning Authority. The boundary treatment shall be erected prior to occupation on a phased basis in accordance with the approved details and shall be permanently maintained as such thereafter.

Reason: To ensure that appropriate boundary treatments are proposed to safeguard the amenities of neighbouring properties and the character of the locality in accordance with Policies CP1 and CP12 of the Core Strategy (adopted October 2011).

C8	Drainage

The development shall be carried out in accordance with the updated Drainage Strategy Statement (September 2016) and approved plan 11363/5002 P3.

Reason: To prevent flooding by ensuring the satisfactory attenuation and disposal of surface water from the site in accordance with Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM8 of the Development Management Policies LDD (adopted July 2013).

C9	Energy Statement

The development shall not be occupied until the energy saving and renewable energy measures detailed within the Energy and Sustainability Statement submitted as part of the application are incorporated into the approved development.

Reason: To ensure that the development meets the requirements of Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM4 of the Development Management Policies LDD (adopted July 2013) and to ensure that the development makes as full a contribution to sustainable development as possible.

C10	Mechanical & Electrical Design

The development shall be carried out and maintained in accordance with the Mechanical and Electrical Design Report (September 2016) prior to the commencement of the approved use.

Reason: To protect adjoining occupiers and the area in accordance with Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM9 of the Development Management Policies LDD (adopted July 2013).

C11		Refuse & Recycling

The development shall not be occupied until a scheme for the separate storage and collection of waste has been submitted to and approved in writing by the Local Planning Authority.  Details shall include siting, size and appearance of refuse and recycling facilities on the premises. The scheme has been implemented on a phased basis in accordance with the approved details prior to occupation and these facilities should be retained permanently thereafter.

Reason: To ensure that satisfactory provision is made, in the interests of amenity and to ensure that the visual appearance of such provision is satisfactory in compliance with Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM10 of the Development Management Policies document (adopted July 2013).

C12		Parking

The dual use loading/drop-off and pick-up bays; disabled parking bays; taxi rank; short stay parking bays; and cycle racks (thereinafter the ‘spaces’) as shown on approved site plan 1281-D-1100 O shall be provided in accordance with a Management Plan which shall first be submitted to and approved in writing by the Local Planning Authority.

The Management Plan shall include details of:

- Allocation and management of all ‘spaces’.

- Details of signage. To include signage for ‘spaces’ and car park at Station Approach (to avoid vehicles entering loop road if no spaces).

The ‘spaces’ shall be made available prior to the first occupation of any building hereby permitted and shall thereafter be kept permanently available for their allocated use and shall be managed in accordance with the approved Management Plan.

Reason: To ensure adequate provision and in the interests of highway safety in accordance with Policies CP1, CP10 and CP12 of the Core Strategy (adopted October 2011) and Policy DM13 and Appendix 5 of the Development Management Policies LDD (adopted July 2013).

8.2	Informatives

I1	General Advice:

With regard to implementing this permission, the applicant is advised as follows:

All relevant planning conditions must be discharged prior to the commencement of work. Requests to discharge conditions must be made by formal application. Fees are £97 per request (or £28 where the related permission is for extending or altering a dwellinghouse or other development in the curtilage of a dwellinghouse). Please note that requests made without the appropriate fee will be returned unanswered.

There may be a requirement for the approved development to comply with the Building Regulations. The Council's Building Control section can be contacted on telephone number 01923 727132 or at the website above for more information and application forms.

Community Infrastructure Levy (CIL) - If your development is liable for CIL payments, it is a requirement under Regulation 67 (1) of The Community Infrastructure Levy Regulations 2010 (As Amended) that a Commencement Notice (Form 6) is submitted to Three Rivers District Council as the Collecting Authority no later than the day before the day on which the chargeable development is to be commenced. DO NOT start your development until the Council has acknowledged receipt of the Commencement Notice. Failure to do so will mean you will lose the right to payment by instalments (where applicable), lose any exemptions already granted, and a surcharge will be imposed.

Where possible, energy saving and water harvesting measures should be incorporated. Information on this is also available from the Council's Building Control section. Any external changes to the building which may be subsequently required should be discussed with the Council's Development Management Section prior to the commencement of work.

I2	Construction Hours:

The applicant is reminded that the Control of Pollution Act 1974 stipulates that construction activity (where work is audible at the site boundary) should be restricted to 0800 to 1800 Monday to Friday, 0900 to 1300 on Saturdays and not at all on Sundays and Bank Holidays.

I3	Positive & Proactive:

The Local Planning Authority has been positive and proactive in its consideration of this planning application, in line with the requirements of the National Planning Policy Framework and in accordance with the Town and Country Planning (Development Management Procedure) (England) Order 2015. The development maintains/improves the economic, social and environmental conditions of the District.

I4	National Grid:

Due to the presence of National Grid apparatus in proximity to the application site, the Applicant should contact National Grid before any works are carried out to ensure National Grid apparatus is not affected by any of the proposed works. Further 'Essential Guidance' can be found on the National Grid website at www.nationalgrid.com or by contacting National Grid on 0800688588.

I5	Network Rail:

Due to the proximity of Network Rail assets to the site you are advised to contact Network Rail.  Telephone no. 0203 356 2510 or Email: AssetProtectionLNEEM@networkrail.co.uk

I6	Signage/Illumination:

The applicant is reminded that the addition of signage/illumination to units would likely require consent.

I7	CCTV:

The applicant is referred to the comments of the Crime Prevention Design Advisor who has requested that consideration is given to the provision of CCTV to cover Station Approach and the access to the pedestrian underpass.