15.	16/0867/FUL - Single storey rear extension at 268 BALDWINS LANE, CROXLEY GREEN, HERTFORDSHIRE, WD3 3LG for Mr and Mrs Evans

(DCES)

| Parish: Croxley Green | Ward: Dickinsons |
| --- | --- |
| Expiry Statutory Period: 16 June 2016 | Officer: Joanna Bowyer |
| Recommendation: That Planning Permission be granted |  |
| This application is brought before the Committee as the agent is related to a member of staff. |  |

1.	Relevant Planning History

  - No relevant planning history.

2.	Site Description

  - The application site is to the north of Baldwins Lane, Croxley Green and includes a two storey semi-detached dwelling. The dwelling has a pitched tiled roof and is finished in red brick and tile hanging. There is a single storey outbuilding to the rear of the dwelling which is set on the east boundary of the site.

  - The property is set back approximately 8m from the highway and to the frontage of the site is an area of soft landscaping and parking for two vehicles. To the rear of the dwelling is a garden of approximately 200sqm which widens at the rear to wrap around the site of 268a Baldwins Lane. At the rear of the dwelling, the boundary treatment to the east is vegetation, and there is fencing and vegetation to the west boundary with the adjoining neighbour 270 Baldwins Lane, with a 1.2m deep parapet wall closest to the rear elevation.

  - 268a to the east of the site is in use as a Doctors’ surgery and has a single storey side and rear projection closest to the application site. The adjoining neighbour at 270 Baldwins Lane has a small lean-to canopy to the rear elevation.

3.	Description of Proposed Development

  - This application seeks full planning permission for a single storey rear extension. The outbuilding to the rear of the dwelling would be demolished and an extension constructed across the full width of the dwelling. The extension would have a depth of 3.6m and would be 6.6m wide where it would adjoin the rear of the existing dwelling although the west flank of the extension would be splayed reflecting the site boundary and the extension would reduce to a width of 6.1m at the deepest point.

  - It would have a monopitched roof with a maximum height of 3.6m and an eaves height of 2.5m, although there would be a parapet wall 3.7m high for a depth of 1.2m at the west flank. There would be a set of three patio doors and a two-casement window in the rear elevation, and three rooflights.

  - Amended plans submitted during the course of the application have reduced the height of the extension roof and parapet wall.

4.	Consultation

4.1.	Statutory Consultation

    - Croxley Green Parish Council: No grounds for objection.

    - National Grid (Gas): No response received.

4.2	Public Consultation

4.2.1	Number consulted:	6

4.2.2	Site Notice: Not required.

Press notice:  Not required.

4.2.3	Number of responses received: none at time of report, the Committee will be updated verbally as to any responses received.

5.	Reason for Delay

5.1	Not applicable.

6.	Relevant Planning Policy, Guidance and Legislation

  - On 27 March 2012, the framework of government guidance in the form of Planning Policy Statements and Planning Policy Guidance Notes was replaced by the National Planning Policy Framework (NPPF). The adopted policies of Three Rivers District Council reflect the content of the NPPF. The Three Rivers emerging Local Plan is currently being drawn up. The Core Strategy was adopted on 17 October 2011 after the Inspector concluded that it was sound following Examination in Public which took place in June 2011. Relevant policies of the adopted Core Strategy include CP1, CP9, CP10 and CP12.

  - The Development Management Policies Local Development Document (LDD) was adopted on 26 July 2013 after the Inspector concluded that it was sound following Examination in Public which took place in March 2013. Relevant policies of the adopted Development Management Policies LDD include DM1, DM6, DM13 and Appendices 2 and 5.

  - The Community Infrastructure Levy (CIL) Charging Schedule (adopted February 2015).

  - The Localism Act received Royal Assent on 17 November 2011. The Growth and Infrastructure Act received Royal Assent on 25 April 2013.

  - The Wildlife and Countryside Act 1981 (as amended), the Conservation of Habitats and Species Regulations 2010, the Natural Environment and Rural Communities Act 2006 and the Habitat Regulations 1994 may also be relevant.

7.	Planning Analysis

  - Design and Impact on Street Scene

    - Policy CP1 of the Core Strategy seeks to promote buildings of a high enduring design quality that respect local distinctiveness and Policy CP12 of the Core Strategy sets out that development should ‘have regard to the local context and conserve or enhance the character, amenities and quality of an area’.

    - Policy DM1 and Appendix 2 of the Development Management Policies document set out that extensions should not be excessively prominent and should respect the character of the street scene, particularly with regard to the spacing of properties, roof form, positioning and style of windows and doors and materials. The Design Criteria at Appendix 2 of the Development Management Policies document state that generally the maximum depth of single storey rear extensions to semi-detached dwellings may be 3.6m although this distance should be reduced if the extension would adversely affect adjoining properties or be unduly prominent.

    - The extension would be to the rear of the dwelling and would not be readily apparent from the street scene. It would have a depth of 3.6m which would comply with the Design Criteria at Appendix 2 of the Development Management Policies document and the depth proposed would not appear disproportionate to the host dwelling or site. The monopitched roof form would reflect the character of the host dwelling, and while the extension would have a splayed flank boundary, this would reflect the site boundary. There are varied extensions in the area such that it is not considered that this would result in harm to the character or appearance of the dwelling or area so as to justify refusal of permission.

    - Subject to the use of appropriate materials which would be secured by condition on any consent, it is not therefore considered that the proposal would adversely affect the character or appearance of the host dwelling or area and the development would be acceptable in accordance with Policies CP1 and CP12 of the Core Strategy and Policy DM1 and Appendix 2 of the Development Management Policies LDD.

  - Impact on Neighbours

    - Policy CP12 of the Core Strategy states that development should ‘protect residential amenities by taking into account the need for adequate levels and disposition of privacy, prospect, amenity and garden space’. Policy DM1 and Appendix 2 of the Development Management Policies document set out that extensions should not result in loss of light to the windows of neighbouring properties nor allow overlooking, and should not be excessively prominent in relation to adjacent properties.

    - The extension would have a depth of 3.6m which would comply with the Design Criteria at Appendix 2 of the Development Management Policies document. It would be set on the boundary with the adjoining neighbour at 270 Baldwins Lane which has a small lean-to canopy to the rear. The extension would be deeper than this lean-to and it is noted that there would be a 3.7m high parapet wall. However, this parapet wall would only project 1.2m in depth and the monopitched roof of the extension would fall to an eaves height of 2.5m at the deepest point. Given this and the 3.6m depth proposed, it is not therefore considered that the extensions would be overbearing or cause significant loss of light to this neighbour so as to result in demonstrable harm.

    - The extension would also be set on the boundary with 268a Baldwins Lane, however it would not project deeper than the rear of this property which is in use as a Doctors’ surgery and it would not therefore be overbearing or cause loss of light to this neighbour.

    - There would be no glazing to the flanks of the extension and the rear glazing proposed would look onto the application site garden and as a consequence of this and the boundary treatment to the site would not cause unacceptable overlooking to any neighbours. The height and profile of the rooflights to the rear extension would not allow overlooking to any neighbours.

    - In summary, the proposed development would not result in any significant adverse impact on the residential amenity of any neighbouring dwelling so as to justify refusal of the application and would be acceptable in accordance with Policy CP12 of the Core Strategy and Policy DM1 and Appendix 2 of the Development Management Policies LDD.

  - Amenity Space

    - Policy CP12 of the Core Strategy states that development should take into account the need for adequate levels and disposition of privacy, prospect, amenity and garden space and Appendix 2 of the Development Management Policies document set out indicative standards. The extension would not result in any additional bedrooms and a rear garden of over 175sqm would be retained. There would therefore be sufficient amenity space provision for current and future occupiers.

  - Trees and Landscaping

    - No significant trees would be affected by the development.

  - Highways, Parking and Access

    - Core Strategy Policy CP10 requires development to demonstrate that it will provide a safe and adequate means of access and to make adequate provision for all users including car and other vehicle parking. Policy DM13 and Appendix 5 of the Development Management Policies document set out parking standards. The extension would not affect the requirement for, or the provision of, parking which would remain sufficient to serve the dwelling.

  - Biodiversity

    - Section 40 of the Natural Environment and Rural Communities Act 2006 requires Local Planning Authorities to have regard to the purpose of conserving biodiversity. This is further emphasised by regulation 3(4) of the Habitat Regulations 1994 which state that Councils must have regard to the strict protection for certain species required by the EC Habitats Directive. The Habitats Directive places a legal duty on all public bodies to have regard to the habitats directive when carrying out their functions.

    - The protection of biodiversity and protected species is a material planning consideration in the assessment of this application in accordance with Policy CP9 of the Core Strategy, and Policy DM6 of the Development Management Policies document. National Planning Policy requires Local Authorities to ensure that a protected species survey is undertaken for applications where biodiversity may be affected prior to the determination of a planning application. A Biodiversity Checklist was submitted with the application and states that no protected species or biodiversity interests will be affected as a result of the application. The site is not in or located adjacent to a designated wildlife site and the Local Planning Authority is not aware of any records of protected species within the immediate area that would necessitate further surveying work being undertaken.

8.	Recommendation

8.1	That subject to no new material considerations being raised, PLANNING PERMISSION BE GRANTED subject to the following conditions:–

C1	The development hereby permitted shall be begun before the expiration of three years from the date of this permission.

Reason: In pursuance of Section 91(1) of the Town and Country Planning Act 1990 and as amended by the Planning and Compulsory Purchase Act 2004.

C2	The development hereby permitted shall be carried out in accordance with the following approved plans: 1 (Existing Plans and Elevations) and 2 Rev A (Rear Extension)

Reason: For the avoidance of doubt and in the proper interests of planning and in the interests of the visual amenities of the locality and the residential amenity of neighbouring occupiers, in accordance with Policies CP1, CP9, CP10 and CP12 of the Core Strategy (adopted October 2011) and Policies DM1, DM6, DM13 and Appendices 2 and 5 of the Development Management Policies LDD (adopted July 2013).

C3	Unless specified on the approved plans, all new works or making good to the retained fabric shall be finished to match in size, colour, texture and profile those of the existing building.

Reason: To ensure that the external appearance of the building is satisfactory in accordance with Policies CP1 and CP12 of the Core Strategy (adopted October 2011) and Policy DM1 and Appendix 2 of the Development Management Policies LDD (adopted July 2013).

8.2	Informatives

I1	With regard to implementing this permission, the applicant is advised as follows:

All relevant planning conditions must be discharged prior to the commencement of work. Requests to discharge conditions must be made by application form; the relevant form is available on the Council's website (www.threerivers.gov.uk). Fees are £97 per request (or £28 where the related permission is for extending or altering a dwellinghouse or other development in the curtilage of a dwellinghouse). Please note that requests made without the appropriate fee will be returned unanswered.

There may be a requirement for the approved development to comply with the Building Regulations. The Council's Building Control section can be contacted on telephone number 01923 727132 or at the website above for more information and application forms.

Where possible, energy saving and water harvesting measures should be incorporated. Information on this is also available from the Council’s Building Control section. Any external changes to the building which may be subsequently required should be discussed with the Council’s Development Management Section prior to the commencement of work.

I2	The applicant is reminded that the Control of Pollution Act 1974 stipulates that construction activity (where work is audible at the site boundary) should be restricted to 0800 to 1800 Monday to Friday, 0900 to 1300 on Saturdays and not at all on Sundays and Bank Holidays.

I3	The applicant is advised that the requirements of the Party Wall Act 1996 may need to be satisfied before development commences.

I4	The Local Planning Authority has been positive and proactive in its consideration of this planning application, in line with the requirements of the National Planning Policy Framework and in accordance with the Town and Country Planning (Development Management Procedure) (England) Order 2015. The Local Planning Authority suggested modifications to the development during the course of the application and the applicant submitted amendments which result in a form of development that maintains/improves the economic, social and environmental conditions of the District.